# Assessment Requirements for FWPWPP3203 Produce fibre from chips

Release: 1

Assessment Requirements for FWPWPP3203 Produce fibre from chips

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPIWPP3203B Produce fibre from chips, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. Assessment requirements now specify the performance and knowledge evidence, as well as assessment conditions for this unit of competency. These sections simplify and replace components used in the previous unit format, including Required Skills and Knowledge, Evidence Guide and Range Statement |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

complete fibre production jobs and safely operate defibrator in line with individual work orders prescribing different fibre size and quantity for each job

complete assessments of the source wood chips for:

species

size

moisture content

contamination issues

monitor and adjust the feed system according to:

white water

stock level

fibre consistency

feed level

steam flow

raw water

noise

power consumption

after each of the above jobs, complete operator maintenance on the defibrator.

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

environmental protection practices for producing fibre from wood chips:

reducing water and energy use

cleaning plant, tools and equipment

disposing of, recycling and reusing fibre, pulp and other waste

typical flow of material supply in and out of the fibre production area

methods used to assess wood chips for:

species

size

moisture content

contamination issues

typical work flow processes and techniques used to produce fibre from wood chips

purpose, features and operation of equipment used to produce fibre from chips:

defibrator

feed system

methods for assessing defibrator blade condition and completing basic non-specialist repair and maintenance

organisational procedures specific to producing fibre from chips:

workplace health and safety with particular emphasis on equipment lock out and use of personal protective equipment

communication reporting lines

recording and reporting production outcomes, equipment faults and maintenance requirements.

# Assessment Conditions

The following resources must be made available:

defibrator

feed system

maintenance tools and equipment

consumable defribrator blades

personal protective equipment suitable for producing fibre from chips

wood chips from various timber species

manufacturers’ instructions for use and maintenance of equipment

work order with specific instructions for assessing wood chips and producing fibre to a specified grade

template documents for recording production outcomes, equipment faults and maintenance requirements

relevant personnel for the purposes of communicating information

organisational procedures for producing fibre from chips.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

### Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47