# Assessment Requirements for FWPWPP3208 Punch peg holes in panels

Release: 1

Assessment Requirements for FWPWPP3208 Punch peg holes in panels

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPIWPP3208B Punch peg holes in panels, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. Assessment requirements now specify the performance and knowledge evidence, as well as assessment conditions for this unit of competency. These sections simplify and replace components used in the previous unit format, including Required Skills and Knowledge, Evidence Guide and Range Statement. |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

produce punched panels in line with individual work orders for each panel type prescribing different hole layouts/patterns and number of panels; material type include:

medium density fibreboard

chipboard

fibreboard

for each of the above jobs, stack batches of punched panels in storage location according to material type, dimension and hole layout/pattern

after each of the above jobs, complete operator maintenance on the punching machine used.

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

environmental protection practices for forest products factory settings:

reducing water and energy use

cleaning plant, tools and equipment

disposing of, recycling and reusing timber and other waste

storage layout and categories for:

raw materials

stacking finished panels

typical panel defects and how they affect ability to produce quality punched panels:

warp

wane

cupping

common uses for punched panels and typical hole layout/patterns

typical work flow processes and veneer punching techniques for optimising use of panels

appropriate range of feed rates to produce specified hole layout/pattern and quality finish

features of defects in finished panels which would trigger rejection and disposal:

incorrect margins

incorrect patterns

doglegs

purpose, features and operation of equipment used to punch peg holes in panels:

measuring equipment

punching machines

methods for assessing punch condition and completing basic non-specialist repair and maintenance for punching machines

organisational procedures specific to punching peg holes in panels:

workplace health and safety with particular emphasis on equipment lock out and use of personal protective equipment

communication reporting lines

recording and reporting production outcomes, equipment faults and maintenance requirements.

# Assessment Conditions

The following resources must be made available:

measuring equipment

punching machines

maintenance tools and equipment

personal protective equipment suitable for punching peg holes in panels

commercial timber panels which must include one or more of the following:

medium density fibreboard

chipboard

fibreboard

manufacturers’ instructions for use and maintenance of equipment

work order with specific instructions for the punching and storing timber panel products

template documents for recording production outcomes, equipment faults and maintenance requirements

relevant personnel for the purposes of communicating information

organisational procedures for punching peg holes in panels.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

### Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47