# FWPWPP3211 Maintain caul plates and screens

Release: 1

FWPWPP3211 Maintain caul plates and screens

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPIWPP3211B Maintain caul plates and screens, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. |

# Application

This unit of competency describes the outcomes required to clean, inspect and replace caul plates and screens in laminating and pressing equipment.

The unit applies to production operators and technicians who maintain caul plates and screens in a forest and wood products factory setting.

No licensing, legislative, regulatory, or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Wood Panel Products

# Elements and Performance Criteria

| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold italicised text is used, further information is detailed in the range of conditions. |
| --- | --- |
| 1. Prepare for maintenance | 1.1 Review work order and where required check with appropriate personnel. 1.2 Select appropriate repair, maintenance and personal protective equipment and check for operational effectiveness. 1.3 Remove caul plates and screens for maintenance from designated machinery. 1.4 Plan maintenance process in line with work order. |
| 2. Change, clean and polish caul plates | 2.1 Follow organisational safety procedures and manufacturer’s recommendations throughout all maintenance and repair processes. 2.2 Identify plates requiring cleaning and move from circuit to plate rack. 2.3 Process plates through full cleaning cycle; inspect for defects and separate for repair where required. 2.4 Dry plates, prepare for use and return to circuit. 2.5 Polish plates using polishing table and head. 2.6 Store polished plates for use in line with work order. 2.7 Store or dispose of unused chemicals in line with environmental protection practices. |
| 3. Prepare backing screens | 3.1 Roll screens from plate and secure. 3.2 Process screens through full cleaning cycle; stack and inspect for faults. 3.3 Identify defective screens, remove and dispose of these in line with environmental protection practices. |
| 4. Remove and replace transport plates | 4.1 Coordinate load shifting and transport requirements to remove and replace transport plates with minimal downtime. 4.2 Identify defective plates, remove from circuit and dispose of these in line with environmental protection practices. 4.3 Prepare and fit replacement plates. 4.4 Record and report process outcomes and equipment faults to appropriate personnel. |

# Foundation Skills

| This section describes those core and employment skills that are essential to performance and are not explicit in the performance criteria. |  |
| --- | --- |
| Numeracy skills to: | identify and set cycle time for chemical and heat treatment baths read basic numerical machine settings. |
| Oral communication skills to: | ask questions and actively listen to clarify contents of work orders. |
| Reading skills to: | interpret: workplace health and safety and other organisational procedures work orders safety data sheets for chemicals manufacturers’ instructions and maintenance recommendations. |
| Writing skills to: | complete accurate basic records for process outcomes and equipment faults. |
| Planning and organising skills to: | efficiently and logically sequence the stages of repair and maintenance activities using work order to guide activities. |
| Problem-solving skills to: | identify and dispose of defective materials recognise faulty equipment and resolve by reporting or rectifying. |
| Technology skills to: | select and use appropriate hand-held tools and personal protective equipment to complete maintenance and repairs use chemical and heat treatment baths to maintain caul plates and screens. |

# Range of Conditions

Not Applicable

# Unit Mapping Information

FPIWPP3211B Maintain caul plates and screens

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47