# Assessment Requirements for FWPWPP3216 Press material using the daylight process

Release: 1

Assessment Requirements for FWPWPP3216 Press material using the daylight process

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPIWPP3216B Press material using the daylight process, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. Assessment requirements now specify the performance and knowledge evidence, as well as assessment conditions for this unit of competency. These sections simplify and replace components used in the previous unit format, including Required Skills and Knowledge, Evidence Guide and Range Statement. |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

compress material using the daylight process in line with individual work orders that prescribe different quantities; types of materials for compression include:

chips

fibre

flakes

assess formed material for moisture content, weight, thickness, density and defects and adjust forming cycle times, pressures and temperatures to produce quality outputs

after each of the above jobs, complete operator maintenance to clean equipment.

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

environmental protection practices for forest products factory settings:

reducing water and energy use

cleaning plant, tools and equipment

disposing of, recycling and reusing timber and other waste

typical flow of material supply in and out of the pressing area

typical work flow processes and processing techniques used to compress material using formers and a heat driven press system

purpose, features and operation of equipment used to press material using the daylight process:

daylight process presses driven by steam pressure, including pneumatic or hydraulic loading and unloading equipment

pre-presses

formers

forming equipment:

mats

screens

trays

plates

press bars or stops

cooling receivers

blending equipment

trimming equipment

appropriate cycle times, pressures and temperatures for the materials that can be compressed:

chips

fibre

flakes

methods used to assess formed material for moisture content, weight, thickness, density and defects

features of typical defects in formed materials:

surface blemishes

discolouration

natural timber defects

forming deformities

methods used to strip and clean equipment

organisational procedures specific to pressing material using the daylight process:

workplace health and safety with particular emphasis on equipment lock out and use of personal protective equipment

communication reporting lines

recording and reporting production outcomes and equipment faults.

# Assessment Conditions

The following resources must be made available:

tools and equipment for pressing material using the daylight process which must include at least one of the following:

daylight process presses driven by steam pressure

pre-presses

formers

forming equipment which must include at least one of the following:

mats

screens

trays

plates

press bars or stops

cooling receivers

blending equipment

trimming equipment

personal protective equipment suitable for pressing material using the daylight process

commercial material to be compressed which must include at least one of the following:

chips

fibre

flakes

manufacturers’ instructions for use and cleaning of equipment

work order with specific instructions for pressing material using the daylight process

template documents for recording production outcomes and equipment faults

relevant personnel for the purposes of communicating information

organisational procedures for pressing material using the daylight process

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

### Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47