# FWPWPP3222 Press laminated ply

Release: 1

FWPWPP3222 Press laminated ply

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPIWPP3222B Press laminated ply, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. |

# Application

This unit of competency describes the outcomes required to set up, operate and shut down hot and cold presses as well as their combined operation used to press laminated ply.

The unit applies to production technicians and operators who press laminated ply in a forest and wood products factory setting.

Licensing, legislative, regulatory, or certification requirements apply to this unit in some states and territories at the time of publication, and may differ according to jurisdiction.

# Pre-requisite Unit

Nil

# Unit Sector

Wood Panel Products

# Elements and Performance Criteria

| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold italicised text is used, further information is detailed in the range of conditions. |
| --- | --- |
| 1. Prepare for pressing. | 1.1 Review and check work order with appropriate personnel. 1.2 Identify hazards and control risk in the work area. 1.3 Acquire type and quantity of ply to be pressed from the assembly process. 1.4 Conduct pre-start checks on equipment in line with manufacturer’s recommendations and site requirements. 1.5 Select pressing process in line with site procedures. 1.6 Communicate with relevant personnel as required, to ensure safety and production requirements are met. |
| 2. Operate, monitor and report pressing process. | 2.1 Set and adjust press cycle times, pressures, temperatures and product parameters to meet production requirements. 2.2 Assess assembled ply and check tack level before loading with defective assemblies returned for reworking. 2.3 Operate and monitor hot or cold press to press boards in line with site requirements. 2.4 Perform unload and reload cycles. 2.5 Regularly assess pressed and finished ply and action necessary changes to assembly and pressing processes. 2.6 Dispose of sub-standard pressed ply in line with site procedures and environmental requirements. 2.7 Record and report pressing process and equipment faults to appropriate personnel. |
| 3. Unload, maintain and load press. | 3.1 Open press in line with operating procedures. 3.2 Separate and remove ply and plates. 3.3 Load ply into transport or storage racks for cooling. 3.4 Monitor plate condition during unloading, clean and spray with release agent and remove defective plates for repair. 3.5 Stack assembled ply, plates and overlay panels in press to specified height or number and squareness. 3.6 Transfer finished ply for further processing in line with site requirements. |

# Foundation Skills

| This section describes those core and employment skills that are essential to performance and are not explicit in the performance criteria. |  |
| --- | --- |
| Numeracy skills to: | interpret instruments, gauges and data recording equipment. |
| Reading skills to: | interpret safety data sheets (SDS) monitor process information displayed on screens |
| Writing skills to: | maintain documentation (log activity) record quality outcomes. |

# Range of Conditions

Not Applicable

# Unit Mapping Information

FPIWPP3222B Press laminated ply.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47