# FWPWPP3233 Operate a laminating press for engineered wood panels

Release: 1

FWPWPP3233 Operate a laminating press for engineered wood panels

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 4.0. |

# Application

This unit of competency describes the skills and knowledge required to operate laminating presses and ancillary equipment to laminate engineered wood panels in a manufacturing facility.

The unit applies to individuals who work as production operators and technicians. Operators generally work under broad or limited direction to complete routine activities related to their own work, and take responsibility for their work.

All work must be carried out to comply with workplace procedures, according to state/territory health and safety regulations, legislation and standards that apply to the workplace.

No licensing, legislative, regulatory or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Wood Panel Products

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for laminating engineered wood panels | 1.1 Review work order, workplace health and safety requirements, environmental protection practices and emergency procedures for lamination operation and, where required, check with appropriate personnel 1.2 Identify and report hazards and use personal protective equipment according to workplace requirements and health and safety legislation to maintain safe work practices 1.3 Obtain the type and quantity of engineered wood panel and laminating material from the production process 1.4 Select production process according to work order |
| 2. Operate a laminating press to laminate engineered wood panels | 2.1 Use laminating press and ancillary equipment according to workplace health and safety procedures, manufacturer instructions and environmental protection practices 2.2 Complete pre start-up checks on the laminating press and ancillary equipment to ensure operational effectiveness 2.3 Load laminating material and engineered wood panel into the laminating press and feed in the correct sequence to suit planned assembly 2.4 Check pressing process regularly to prevent creases, bubbles or other surface defects 2.5 Monitor engineered wood panel and laid up laminating material for position, adhesion, contamination and finish 2.6 Transfer finished engineered wood panels for further processing according to workplace health and safety procedures 2.7 Dispose of sub-standard laminated material according to environmental protection practices |
| 3. Shut down and clean equipment | 3.1 Coordinate machine shutdown with other line operators according to workplace health and safety procedures 3.2 Strip and clean laminating press according to workplace procedures 3.3 Dispose of or store excess bonding agent according to environmental protection practices 3.4 Keep machine clear of dust and debris according to workplace health and safety procedures 3.5 Record and report production outcomes and laminating press and ancillary equipment faults to appropriate personnel |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret safety data sheets for bonding agents |
| Oral communication | Ask questions and actively listen to clarify contents of work orders |
| Numeracy | Identify quantities of required material within work orders, and count numbers of panels Read basic numerical machine settings to set temperatures |
| Navigate the world of work | Understand main tasks, responsibilities and boundaries of own role |
| Interact with others | Use modes of communication suitable for the purpose to confirm and clarify understanding |
| Get the work done | Recognise and respond to routine problems |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPWPP3233 Operate a laminating press for engineered wood panels | FWPWPP3210 Laminate board | Updated industry terminology, workplace health and safety requirements and foundation skills; clarified intent of the unit and assessment requirements; changed unit title and code for compliance and consistency with the updates. | Equivalent unit |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPWPP3233 Operate a laminating press for engineered wood panels

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 4.0. |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit. There must be evidence that, on at least one occasion, the individual has:

followed workplace health and safety requirements and environmental protection procedures for laminating engineered wood panels

laminated one or more of the following types of engineered wood panels according to individual work orders prescribing different laminating thicknesses and quantities of panels:

medium density fibreboard

chipboard

plywood

fibreboard

applied different types of laminating material, which include:

laminate

veneer

treated paper

stripped and cleaned the laminating press.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

health and safety hazards or risks related to laminating presses and ancillary equipment as identified through risk assessments, manufacturer technical information, regulations and industry health and safety guides

workplace procedures specific to laminating engineered wood panel:

health and safety, with particular emphasis on equipment lockout, emergency procedures, use of personal protective equipment (PPE) and handling hazardous substances according to safety data sheets for glue

use of laminating equipment

communication reporting lines

recording and reporting production outcomes and equipment faults

environmental protection practices for engineered wood panel facilities:

reducing water and energy use

cleaning plant, tools and equipment

disposing of, recycling and reusing engineered wood panels

disposing of hazardous substances

typical flow of material supply in and out of the engineered wood panel laminating area

properties, at a basic level of understanding, of laminating materials and their uses for different types of engineered wood panels:

laminate

veneer

treated paper

characteristics of engineered wood panel types and the effects of lamination:

medium density fibreboard

chipboard

plywood

fibreboard

typical workflow processes and techniques used to laminate engineered wood panels:

applying laminating material

spreading bonding agent

compressing laminating material to engineered wood panel

purpose, features and operation of equipment used to laminate engineered wood panels:

laminating press with bond spreading stations

feeding systems

methods used to:

strip and clean laminating presses

safely store bonding agents.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in an environment that accurately represents workplace conditions

resources, equipment and materials:

laminating press with bond spreading stations

feeding systems

PPE suitable for laminating engineered wood panels

engineered wood panels suitable for lamination operations

laminating material

bonding agent

specifications:

manufacturer instructions for use and cleaning of equipment

work order with specific instructions for laminating engineered wood panel

emplate documents for recording production outcomes and equipment faults

workplace procedures for laminating engineered wood panel, including health and safety and emergency procedures and environmental protection procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47