# ICPCBF2180 Produce basic folded and glued cartons

Release: 1

ICPCBF2180 Produce basic folded and glued cartons

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 4.0. |

# Application

This unit describes the skills and knowledge required to produce basic folded and glued cartons and correctly shut down machinery when the job is completed.

It applies to individuals in the printing and graphic arts industry who produce basic folded and glued cartons to meet production requirements. They generally work under direct supervision.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Unit Sector

Converting, Binding and Finishing

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Maintain operation of carton blank system | 1.1 Monitor and adjust feeder to maintain continuous feeding to machine 1.2 Monitor and adjust carton blank pick-up and transport system to maintain continuous carton blank handling and operation 1.3 Review and adjust transfer systems to maintain continuous carton blank handling and operation 1.4 Check and adjust delivery to maintain product delivery according to quality requirements |
| 2. Maintain basic gluing and continuous folding process | 2.1 Monitor and adjust registration and squareness of fold to confirm quality of product meets approved sample standard 2.2 Check and adjust registration of gluing to confirm quality of product meets approved sample standard 2.3 Observe and adjust adhesion to confirm quality meets approved sample standard |
| 3. Identify and rectify minor problems | 3.1 Monitor and verify performance using the process control system according to enterprise procedures 3.2 Identify and report problems in folding operation 3.3 Carry out adjustments and corrections to folding operation within scope of own role 3.4 Check folding to confirm operation meets requirements 3.5 Identify and report problems in gluing unit 3.6 Carry out adjustments and corrections to gluing unit according within scope of own role 3.7 Check gluing unit operation to confirm operation meets requirements |
| 4. Conduct shutdown of production process | 4.1 Follow shutdown sequence according to manufacturer specifications and enterprise procedures 4.2 Conduct shutdown in association with fellow workers and in compliance with work health and safety (WHS) requirements 4.3 Wash glue system and prepare for next run 4.5 Remove waste from operating area and recycle or dispose of, where required, according to regulatory requirements 4.6 Identify machine faults requiring repair and report to designated person 4.7 Verify repairs and adjustments prior to resumption of operations 4.8 Complete production records and other documentation |

# Foundation Skills

This section describes language, literacy, numeracy and employment skills incorporated in the performance criteria that are required for competent performance.

| Skill | Description |
| --- | --- |
| Reading | Identifies and interprets text to confirm work requirements from information contained within relevant specifications and enterprise documents |
| Writing | Enters routine data associated with production processes and enterprise procedures |
| Oral Communication | Participates in routine spoken interactions to report issues or to confirm or clarify requirements |
| Self-management | Identifies and follows all procedures for operation and maintenance of equipment, incorporating regulatory and enterprise requirements |
| Teamwork | Cooperates with others as part of familiar routine activities, and contributes to specific activities requiring joint responsibility and accountability |
| Planning and organising | Sequences and schedules activities required to complete tasks and monitors implementation making adjustments as necessary Uses analytical processes to interpret relevant information and make decisions directly related to tasks |
| Problem solving | Identifies and anticipates an increasing range of familiar problems, actively looking for early warning signs and implementing contingency plans |
| Technology | Follows routine procedures for using digital technology by reading data from electronic monitoring and control systems |

# Unit Mapping Information

Supersedes and is equivalent to ICPCBF218 Produce basic folded and glued cartons.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d

Assessment Requirements for ICPCBF2180 Produce basic folded and glued cartons

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 4.0. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

produce at least four simple straight line folded and glued cartons of different sizes and weights according to manufacturer and job specifications and enterprise procedures on one or more industry standard carton folding and gluing machines.

In the course of the above the candidate must:

operate production process in association with fellow workers and according to enterprise procedures and planned daily schedule

follow work health and safety (WHS) requirements

anticipate production difficulties and take preventive action to prevent occurrence.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

enterprise procedures, manufacturer specifications and work health and safety (WHS) requirements for:

setting up and operating machine delivery systems

operating, adjusting and cleaning folding and gluing machines

different types of folding and gluing machines

checks to be undertaken when substrate is removed from the machine

production areas to continually observe to maintain operation of the machine

causes of out-of-square folding and correction procedures

the segments of quality assurance inspected at completion of the sample run, and the changes required to production

parts of the machine that can be adjusted if carton blanks are creasing

factors that affect glue adhesion on cartons, and the requirements to adjust glue application

machine shutdown procedures

procedures for preparing the finished work for dispatch

areas of the machine that need regular cleaning, and materials that need to be cleaned

requirements to keep the machine clear of surface rust (condensation)

factors that determine a quality product in a completed folded job

regulations and enterprise procedures for waste management

manufacturer manuals, enterprise procedures and work health and safety documentation, including their location and purpose.

# Assessment Conditions

Skills in this unit must be demonstrated in a workplace or simulated environment where the conditions are typical of those in a working environment in this industry.

This includes access to:

special purpose tools, equipment and materials

industry standard folding and gluing machine.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d