# Assessment Requirements for ICPCBF3060 Set up machine for basic carton folding and gluing

Release: 1

Assessment Requirements for ICPCBF3060 Set up machine for basic carton folding and gluing

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 4.0. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

set up a machine, on at least four occasions, for basic straight line folding and gluing of cartons with different weights and sizes of carton board according to manufacturer and job specifications and enterprise procedures.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

work health and safety (WHS) risks and requirements for:

setting up folder transportation and delivery systems

setting up and adjusting the folding unit

adjusting the inkjet printer

working with hot melt adhesives

the job documentation or production control system and the information it contains

areas of the sheet-fed transportation system requiring continuous observation to meet job requirements

areas of the delivery system requiring continuous observation to prevent damage to the finished product

techniques for securing folded sheets for dispatch

the causes of scratching of substrate during transportation

key factors determining speed of the machine and common problems if speed is too fast

checks to confirm roller pressures meet requirements

the adjustments to make if the sheet is out-of-square and reasons why this happens

adjustment procedures for:

confirming sheets leave the folding unit and are unscuffed

positioning units

steps taken to ensure alignment of the inkjet printer

factors affecting the required binding technique for a job

methods of adhesive metering available on the machine

the importance of precautions to ensure a clean adhesive binding job

quality criteria to inspect at the completion of the sample run

communication protocols if an error occurs during production or the job does not meet quality assurance checks

manufacturer manuals, enterprise procedures and work health and safety documentation, including their location and purpose.

# Assessment Conditions

Gather evidence to demonstrate consistent performance in conditions that are safe and replicate the workplace. Noise levels, production flow, interruptions and time variances must be typical of those experienced in the converting, binding and finishing field of work and include access to and use of special purpose tools, materials and equipment including:

special purpose tools, equipment and materials

carton folding and gluing machinery.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d