# ICPCBF3060 Set up machine for basic carton folding and gluing

Release: 1

ICPCBF3060 Set up machine for basic carton folding and gluing

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 4.0. |

# Application

This unit describes the skills and knowledge required to set up and adjust a machine for basic carton folding and gluing to ensure the finished product is in accordance with specifications and requirements.

It applies to individuals who work under direct supervision but have limited responsibility with preparing materials, equipment and machinery to ensure the production process is maintained.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Unit Sector

Converting, Binding and Finishing

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Set up carton systems | 1.1 Set up and adjust feeder according to job specifications 1.2 Set up and adjust carton blank pick-up and transportation system 1.3 Prepare and adjust transfer systems 1.4 Prepare and adjust delivery system 1.5 Remove substrate from process according to job specifications |
| 2. Set up machine for basic folding and gluing | 2.1 Set up and adjust folding units 2.2 Prepare and adjust required folding rollers, belts and rails 2.3 Establish and adjust gluing system |
| 3. Conduct sample run | 3.1 Operate machine according to manufacturer specifications and enterprise procedures to produce specified sample 3.2 Monitor and adjust slitters 3.3 Inspect and test proof as required according to enterprise procedures 3.4 Interpret results and make adjustments according to product and machine specifications as required |

# Foundation Skills

This section describes language, literacy, numeracy and employment skills incorporated in the performance criteria that are required for competent performance.

| Skill | Description |
| --- | --- |
| Self-management | Identifies and follows enterprise and regulatory requirements associated with own role Implements actions according to instructions, making slight adjustments if necessary |
| Planning and organising | Determines priorities and sequences steps involved in clearly defined, familiar tasks Plans, identifies and assembles resources required to complete tasks |

# Unit Mapping Information

Supersedes and is equivalent to ICPCBF306 Set up machine for basic carton folding and gluing.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d

Assessment Requirements for ICPCBF3060 Set up machine for basic carton folding and gluing

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 4.0. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

set up a machine, on at least four occasions, for basic straight line folding and gluing of cartons with different weights and sizes of carton board according to manufacturer and job specifications and enterprise procedures.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

work health and safety (WHS) risks and requirements for:

setting up folder transportation and delivery systems

setting up and adjusting the folding unit

adjusting the inkjet printer

working with hot melt adhesives

the job documentation or production control system and the information it contains

areas of the sheet-fed transportation system requiring continuous observation to meet job requirements

areas of the delivery system requiring continuous observation to prevent damage to the finished product

techniques for securing folded sheets for dispatch

the causes of scratching of substrate during transportation

key factors determining speed of the machine and common problems if speed is too fast

checks to confirm roller pressures meet requirements

the adjustments to make if the sheet is out-of-square and reasons why this happens

adjustment procedures for:

confirming sheets leave the folding unit and are unscuffed

positioning units

steps taken to ensure alignment of the inkjet printer

factors affecting the required binding technique for a job

methods of adhesive metering available on the machine

the importance of precautions to ensure a clean adhesive binding job

quality criteria to inspect at the completion of the sample run

communication protocols if an error occurs during production or the job does not meet quality assurance checks

manufacturer manuals, enterprise procedures and work health and safety documentation, including their location and purpose.

# Assessment Conditions

Gather evidence to demonstrate consistent performance in conditions that are safe and replicate the workplace. Noise levels, production flow, interruptions and time variances must be typical of those experienced in the converting, binding and finishing field of work and include access to and use of special purpose tools, materials and equipment including:

special purpose tools, equipment and materials

carton folding and gluing machinery.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d