# Assessment Requirements for ICPCBF309 Produce complex folded and glued cartons

Release: 1

Assessment Requirements for ICPCBF309 Produce complex folded and glued cartons

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 1.0. |

# Performance Evidence

Evidence of the ability to:

complete THREE crashlock, six-corner and specialty work carton jobs, with different sizes and weights of carton board, using multiple gluing units, according to manufacturer’s and job specifications and enterprise procedures on either a Royal 40, a Bobst Media or Bobst Domino.

Note: If a specific volume or frequency is not stated, then evidence must be provided at least once.

# Knowledge Evidence

To complete the unit requirements safely and effectively, the individual must:

explain work health and safety factors for:

setting and/or operating folder/gluer transport and delivery systems

adjusting/correcting the machine

cleaning the machine

identify areas of the feeder to monitor to ensure trouble-free operation

identify parts of the pick-up system to adjust to ensure accurate and continuous feeding

explain which areas of the delivery system to observe to prevent damage to the finished product

describe areas to observe continually to ensure smooth, trouble-free operation of the machine

explain areas of the gluing unit to monitor continuously

list causes of incorrect folding, and explain how each can be corrected

identify segments of quality assurance inspected at sample run completion

describe action to take if adhesive viscosity in gluing unit is too low/high

explain how application of glue is adjusted

identify areas of the machine to adjust if:

sheets are not neatly entering delivery

carton blanks are creasing

carton blanks are not entering the machine

explain checks to correctly shut down the machine

describe checks to make when preparing finished work for dispatch

identify areas of the machine to clean regularly, and materials to clean from the machine

explain how to keep the machine clear of surface rust (condensation)

identify recommended cleaning agents

identify production records to keep or write-up, and information to include

explain quality aspects to consider in a completed folded carton

list steps to ensure important features of the production control system are followed

explain how production needs to be altered to meet client requirements

list items to check against the client sample

identify machine manuals, safety and other documentation relevant to this task, where they are kept, and the information included in them.

# Assessment Conditions

Gather evidence to demonstrate consistent performance in conditions that are safe and replicate the workplace. Noise levels, production flow, interruptions and time variances must be typical of those experienced in the converting, binding and finishing field of work and include access to and use of special purpose tools, equipment and materials used to produce folded and glued cartons.

Assessors must satisfy NVR/AQTF assessor requirements.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d