# ICPCBF327 Set up machine for complex rotary die cutting or embossing

Release: 1

ICPCBF327 Set up machine for complex rotary die cutting or embossing

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 1.0. |

# Application

This unit describes the skills and knowledge required to set up a machine for complex rotary die cutting or embossing.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Unit Sector

Converting, Binding and Finishing

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for job | 1.1 Job specifications are read and interpreted from job documentation or production control system 1.2 Set-up is planned and carried out correctly in minimum time with minimum wastage 1.3 Availability of all job related components is checked |
| 2. Mount rotary cutting or embossing devices | 2.1 Cutting devices or dies are correctly mounted to die 2.2 Cutting devices or dies are registered and proofed on die cylinder 2.3 Appropriate cutting devices or dies are selected and secured to machine according to job specifications |
| 3. Set up delivery systems | 3.1 Delivery systems are set up and adjusted according to job specifications 3.2 Substrate is removed according to job specifications |
| 4. Set up machine for basic rotary cutting | 4.1 Rotary cutting devices are set up and adjusted according to job specifications 4.2 Cutting pressures are set up and adjusted according to job specifications 4.3 Counter knives/anvils are set in correct position |
| 5. Set up in-line units | 5.1 Minor in-line printing/converting/binding units are set up for basic processes and adjusted according to machine requirements and job specifications 5.2 Assistance is given in set-up of major in-line printing/converting/binding units |
| 6. Conduct sample run | 6.1 Material to be used for sample is organised correctly 6.2 Machine is set up and operated to produce a specified sample according to work health and safety (WHS) requirements, manufacturer’s specifications and enterprise procedures 6.3 Sample is visually inspected and/or tested, or laboratory testing is organised, according to enterprise procedures 6.4 Results are interpreted to determine adjustment requirements 6.5 Adjustments are carried out according to product and machine specifications |

# Foundation Skills

This section describes language, literacy, numeracy and employment skills incorporated in the performance criteria that are required for competent performance.

| Skill | Performance Criteria | Description |
| --- | --- | --- |
| Reading | 1.1, 4.1, 4.2, 6.5 | Recognises and interprets text to establish job requirements from information in relevant procedures and specifications |
| Oral Communication | 5.2 | Participates effectively in spoken interactions to confirm or clarify understanding |
| Navigate the world of work | 6.2, 6.3 | Recognises and follows legislative requirements and organisational policies and procedures relevant to own role |
| Interact with others | 5.2 | Cooperates with others as part of familiar routine activities, and contributes to specific activities requiring joint responsibility and accountability |
| Get the work done | 1.1-1.3, 2.1-2.3, 3.1, 3.2, 4.1-4.3, 5.1, 6.1-6.5 | Determines priorities and sequences steps involved in clearly defined, familiar tasks Implements actions as per instructions, making slight adjustments if necessary Plans, identifies and assembles resources required to complete tasks Follows routine procedures for using digital technology by reading data from electronic monitoring and control systems |

# Range of Conditions

This section specifies different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included.

| Delivery systems MUST include EITHER: | reel systems OR sheet systems. |
| --- | --- |
| Setting up and adjusting MUST include EITHER: | unwind and rewind reels web control system folder and sheeter OR feeder and delivery systems sheet pick-up and transportation systems transfer systems. |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| ICPCBF327 Set up machine for complex rotary die cutting or embossing | ICPCF327C Prepare for cutting forme and stripper making | Updated to meet Standards for Training Packages | Equivalent unit |

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d