# ICPCBF3440 Produce complex collated or inserted product

Release: 1

ICPCBF3440 Produce complex collated or inserted product

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 4.0. |

# Application

This unit describes the skills and knowledge required to produce a complex collated or inserted sheet, section or reel product, to monitor and adjust machinery, maintain transportation of the substrate, identify and rectify faults, and clean and shut down equipment.

It applies to individuals who work under direct supervision with limited responsibility in preparing materials and equipment and resolving issues to produce a finished product.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Unit Sector

Converting, Binding and Finishing

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Maintain delivery systems | 1.1 Monitor and adjust delivery systems according to work health and safety (WHS) requirements 1.2 Add substrate to and remove substrate from the process according to job specifications |
| 2. Maintain production process | 2.1 Review and adjust collating or inserting process according to sample quality requirements 2.2 Monitor and adjust in-line processes 2.3 Use machine controls as required 2.4 Review and verify performance using the process control system 2.5 Report process adjustments to eliminate problems according to enterprise procedures |
| 3. Identify and rectify problems and faults | 3.1 Identify and report problems in machine operation according to enterprise procedures 3.2 Carry out adjustments or corrections according to specified procedures, and within scope of own role 3.3 Verify repairs and adjustments as required prior to resumption of operations |
| 4. Conduct shutdown of production process | 4.1 Follow shutdown sequence according to manufacturer specifications and enterprise procedures 4.2 Remove solid and liquid waste from operating area and recycle and dispose of, as required, according to regulatory requirements and enterprise procedures 4.3 Clean and disengage all required parts of machinery used in printing process according to manufacturer specifications and enterprise procedures 4.4 Complete required production records and enterprise documentation |

# Foundation Skills

This section describes language, literacy, numeracy and employment skills incorporated in the performance criteria that are required for competent performance.

| Skill | Description |
| --- | --- |
| Writing | Enters routine data associated with production processes and enterprise procedures |
| Self-management | Identifies and follows all procedures for operation, cleaning and maintenance of equipment, incorporating regulatory requirements and enterprise procedures |
| Teamwork | Cooperates with others as part of familiar routine activities, and contributes to specific activities requiring joint responsibility and accountability Identifies what to communicate, with whom and how, in routine work situations |
| Planning and organising | Determines priorities and sequences steps involved in clearly defined, familiar tasks |
| Problem solving | Identifies and anticipates an increasing range of familiar problems, their symptoms and causes; actively looking for early warning signs and implementing contingency plans |
| Technology | Follows routine procedures for using digital technology by reading data from electronic monitoring and control systems |

# Range of Conditions

This section specifies different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included.

| Delivery systems must include either: | reel transportation or sheet transportation. |
| --- | --- |
| Monitoring and adjusting must include either: | cleaning collating or inserting (sheet/section/reel) machine web control system or feeder sheet pick-up and transportation system transfer system. |

# Unit Mapping Information

Supersedes and is equivalent to ICPCBF344 Produce complex collated or inserted product.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d

Assessment Requirements for ICPCBF3440 Produce complex collated or inserted product

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 4.0. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

operate machines for complex collating for a minimum of four products, including in-line processes, including:

two sheet jobs, each using different sizes and weights of substrate

two section jobs, with and without lip/lap.

In the course of the above the candidate must:

operate production process in association with fellow workers and according to enterprise procedures and planned daily schedule

follow work health and safety (WHS) requirements, manufacturer specifications and enterprise procedures

anticipate production difficulties and take preventive action to prevent occurrence.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

WHS risks and requirements for:

setting up and operating the machine

adjust the machine

cleaning the machine

machine shutdown

factors affecting the speed the machine operates at

indicators that the machine needs lubrication

circumstances when the machine requires adjustment

areas to check when sections fail to open on the chain

checks to conduct during machine shutdown

areas of the machine that need regular cleaning and required cleaning agents

steps taken to keep the machine clear of surface rust (condensation)

production records to be kept, and the information to include

steps taken to ensure important features of the production control system are followed

quality requirements for a collating result

the cause of sheets creasing in the machine delivery and correction methods

causes of sheets misfeeding during production and methods to prevent double sheet feeds

adjustments required to prevent bruising of no carbon required sheets

manufacturer manuals, enterprise procedures and work health and safety documentation, including their location and purpose.

# Assessment Conditions

Gather evidence to demonstrate consistent performance in conditions that are safe and replicate the workplace. Noise levels, production flow, interruptions and time variances must be typical of those experienced in the converting, binding and finishing field of work and include access to:

special purpose tools, equipment and materials

collating or inserting machinery.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d