# ICPPRN271 Set up for basic coating

Release: 1

ICPPRN271 Set up for basic coating

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 1.0. |

# Application

This unit describes the skills and knowledge required to set up and adjust rollers and the reel or sheet systems for coating a range of aqueous coatings, UV varnishes and machine varnishes.

It applies to individuals who work under direct supervision and prepare material, and monitor and run equipment and machinery.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Unit Sector

Printing

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Confirm job specifications | 1.1 Job requirements are read and interpreted from job documentation or production control system 1.2 Set up is carried out correctly in minimum time with minimum wastage 1.3 Availability of all job related components is checked |
| 2. Set up delivery systems | 2.1 Delivery systems and are set up and adjusted according to job specifications 2.2 Substrate is removed from process according to job instructions |
| 3. Select and prepare coating | 3.1 Coating is selected according to job specifications and end-user requirements 3.2 Quality and suitability of coating is checked and appropriate action is taken 3.3 Coatings and additives are prepared according to work health and safety (WHS) requirements, and manufacturer’s/supplier’s instructions with suitable precautions to minimise waste 3.4 Correct weight/volume of coating is prepared to match requirements of the job specification and coating process 3.5 Check the viscosity of coating is correct for the job 3.6 Coating formulation is appropriately recorded |
| 4. Set up machine for coating | 4.1 Appropriate rollers/cylinders are selected and secured to the machine and set 4.2 Application system is set up and adjusted according to job specifications 4.3 Coating delivery system is set up with correct flow and return flow determined by air pressure or pump speeds, and adjusted according to job specifications 4.4 Coating blanket is cut or a plate installed for non-image areas 4.5 Blanket or plate packing is checked for job suitability 4.6 Coating temperature is checked for job suitability 4.7 Drying system is set up and adjusted according to job specifications |
| 5. Conduct proof run | 5.1 Material to be used for proof is organised correctly 5.2 Machine is set up and operated to produce a specified proof according to work health and safety (WHS) requirements, manufacturer’s specifications and enterprise procedures 5.3 Proof is visually inspected and/or tested or laboratory testing organised according to enterprise procedures 5.4 Production does not commence without client authority, where appropriate 5.5 Results are interpreted and adjustment changes are carried out according to product and machine specifications 5.6 Adjustments are carried out according to product and machine specifications |

# Foundation Skills

This section describes language, literacy, numeracy and employment skills incorporated in the performance criteria that are required for competent performance.

| Skill | Performance Criteria | Description |
| --- | --- | --- |
| Reading | 1.1, 3.1, 3.3, 3.4, 4.2, 4.3, 4.7, 5.2, 5.4-5.6 | Recognises and evaluates text to establish job requirements from information contained within relevant systems |
| Writing | 3.6 | Completes workplace forms accurately |
| Oral Communication | 2.1-2.5, 3.1-3.5, 4.1, 4.3, 4.4, 5.2, 5.3, 5.7, 6.2-6.6 | Participates effectively in spoken interactions to confirm or clarify understandings |
| Numeracy | 4.4 | Interprets and analyses a range of mathematical information used in familiar and routine tasks |
| Navigate the world of work | 3.1-3.5, 4.1-4.3, 4.5-4.7, 5.1-6.6 | Complies with legislative requirements and follows organisational policies and procedures relevant to own role Understands roles and responsibilities for task and makes basic decisions on work completion requirements, seeking assistance when needed |
| Interact with others | 3.1, 3.3, 3.4, 4.2, 4.3, 4.7, 5.2, 5.3, 5.5, 5.6 | Cooperates with others as part of familiar routine activities, and contributes to specific activities requiring joint responsibility and accountability |
| Get the work done. | 1.1-1.3, 3.1-3.5, 4.1-4.7, 5.1-5.3, 5.5, 5.6 | Determines priorities and sequences steps involved in clearly defined tasks Implements actions as per instructions, making slight adjustments if necessary Plans, identifies and assembles resources required to complete tasks Follows routine procedures for using digital technology by reading data from electronic monitoring and control systems |

# Range of Conditions

This section specifies different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included.

| Delivery systems must include EITHER: | reel OR sheet |
| --- | --- |
| Monitoring and adjusting MUST include EITHER: | unwind and rewind reels web control system print web viewing devices set off / marking prevention devices OR feeder and delivery sheet pick up and transportation system transfer and control systems set off / marking prevention devices |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| ICPPRN271 Set up for basic coating | ICPPR271C Set up for basic coating | Updated to meet Standards for Training Packages | Equivalent unit |

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d