# Assessment Requirements for ICPPRN3130 Set up for basic flexographic printing

Release: 1

Assessment Requirements for ICPPRN3130 Set up for basic flexographic printing

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 4.0. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

set up a press on two occasions for basic flexographic printing, according to manufacturer specifications and enterprise procedures.

In the course of the above the candidate must:

demonstrate all safety devices on the machine

demonstrate use of control, monitoring and data entry systems.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

enterprise procedures, manufacturer specifications and work health and safety (WHS) requirements for:

operating the sheet and reel in-feed and delivery systems

handling inks and additives

handling blades

setting up and operating the machine

actions required if vital information is missing from the job ticket

precautions and checks required:

to avoid damaging plates and cylinders

for materials availability prior to set up

to protect in-line processing from damage

for quality control of proof and initial print

common issues arising with reel systems and correction methods, including:

low or excessive web tension

selecting printing side of the material

when UV drying is used to dry the ink film

types of web splices available for the job

common issues arising with sheet systems and correction methods, including:

sheet positioning in the in-feed

selection of front and side lays

machine indications of missing sheets

required applications of spray powder

excessive jogging on the stack

procedures for:

a register check

matching inks to jobs and modifying ink colour

required for print approval

determining machine specifications for particular jobs

minimising and handling waste

recording final results for future reference

reasons for a two-sheet cut in most sheet system feeders

slowdown devices and their uses in delivery

effects on the job of ink viscosity

enterprise personnel who pass colour prior to the job

steps taken to ensure inking system is adjusted as required and the reasons for maintaining ink levels within a required range

optimum make-ready-speed for the job

basic in-line processes and the steps taken to incorporate them into the make-ready

quality control measurements to apply to proof and initial print

machine setting adjustments, the criteria determining their use, and processes to plot their success

tests for scuffing and coefficient of friction and the circumstances requiring them

leaching

major environmental concerns with regard to inks and additives

manufacturer manuals, enterprise procedures  and work health and safety documentation, including their location and purpose.

# Assessment Conditions

Skills in this unit must be demonstrated in a workplace or simulated environment where the conditions are typical of those in a working environment in this industry.

This includes access to:

special purpose tools, equipment and materials

a range of standard inks and substrates

a variety of stack, in-line and central impression flexographic printing machines.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d