# Assessment Requirements for ICPPRN3220 Produce complex gravure printed product

Release: 1

Assessment Requirements for ICPPRN3220 Produce complex gravure printed product

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 4.0. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

monitor production output and make necessary adjustments to maintain print quality on a gravure machine while producing a complex print on two occasions for two different jobs.

In the course of the above the candidate must:

operate production process in association with fellow workers and according to enterprise procedures and planned daily schedule

follow work health and safety (WHS) requirements, manufacturer specifications and enterprise procedures

anticipate production difficulties and take preventive action to prevent occurrence.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

enterprise and manufacturer procedures and WHS requirements for:

rewinding and sheeting

handling ink

handling and cleaning blades

cleaning feed, transportation, delivery and in-line sections of the machine and waste disposal

machine shutdown

enterprise escalation procedures and required personnel

required checks of in-line machine components and their function

common issues arising with ink in the machine and correction methods

causes and correction methods of faults and errors such as:

the reel wandering and the web breaking at the unwind unit

sheet cut-off wander

substrate distortion

effects of:

poorly adjusted nip rollers when rewinding and sheeting

incorrect drying temperatures on the finished product or in chillers

excessive pressure applied on the slitters on the job

inadequate work team communication on a gravure printing machine

relative humidity increases in the press room on the substrate

purpose and function of removing solvents from final ink film

storage procedures for printed jobs and printing cylinders

procedures for handling substrates and labelling printed product prior to press room removal

difference between a “flying paster” and “zero speed” type reel-stand

excess ink and additive labelling and storage procedures

waste sorting and disposal procedures, and benefits of reusable waste

link between driers and set-off and marking

methods of controlling ratio of print to in-line speed

outcome if machine guards are removed and micro switches are disconnected on a machine, and the personnel responsible for this action

steps for addressing production problems and eliminating processing of unacceptable product

industry standards of client communication and client approval procedure for printed product

procedures for reporting gravure machine problems and calling service personnel

further operations required for printed reels after removal from the printing machine

use of completed records in the final analysis of the job and their benefits

product quality requirements and machine adjustment procedures to achieve them

manufacturer manuals, enterprise procedures  and work health and safety documentation, including their location and purpose.

# Assessment Conditions

Gather evidence to demonstrate consistent performance in conditions that are safe and replicate the workplace. Noise levels, production flow, interruptions and time variances must be typical of those experienced in the printing field of work and include access to:

special purpose tools, equipment and materials

gravure printing machines and in-line units.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d