# ICPPRN3220 Produce complex gravure printed product

Release: 1

ICPPRN3220 Produce complex gravure printed product

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 4.0. |

# Application

This unit describes the skills and knowledge required to produce non-routine gravure printed product.

It applies to individuals who operate a gravure press and maintain product quality requirements. Individuals may also provide leadership and guidance to members of their team.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Unit Sector

Printing

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Maintain non-routine operation of reel transportation and delivery systems | 1.1 Monitor and adjust reel stand and web control system according to work health and safety (WHS) requirements 1.2 Add substrate to and remove substrate from process according to job specifications 1.3 Monitor and adjust reel rewind and sheeting sections 1.4 Review and adjust set-off and marking prevention systems according to proof quality requirements |
| 2. Maintain complex gravure printing process | 2.1 Review and adjust gravure cylinder and impression roller condition 2.2 Check and adjust gravure inking system and doctor blade 2.3 Monitor and adjust drying systems |
| 3. Maintain non-routine production process | 3.1 Monitor and adjust in-line processes 3.2 Use machine controls as required 3.3 Monitor and verify performance using the process control system 3.4 Monitor and adjust ink performance, colour, register and position 3.5 Check product quality and adjust performance according to job specifications 3.6 Report process adjustments to eliminate problems according to enterprise procedures |
| 4. Identify and rectify faults | 4.1 Identify and report problems in machine operation according to enterprise procedures 4.2 Carry out adjustments or corrections according to specified procedures, and within scope of own role 4.3 Verify repairs and adjustments as required prior to resumption of operations |
| 5. Conduct shutdown of production process | 5.1 Follow shutdown sequence according to manufacturer specifications and enterprise procedures 5.2 Label and store unused ink and embellishment equipment according to manufacturer specifications and enterprise procedures 5.3 Remove solid and liquid waste from operating area and recycle and dispose of, as required, according to regulatory requirements and enterprise procedures 5.4 Clean and disengage all required parts of machinery used in printing process 5.5 Complete required production records and enterprise documentation |

# Foundation Skills

This section describes language, literacy, numeracy and employment skills incorporated in the performance criteria that are required for competent performance.

| Skill | Description |
| --- | --- |
| Writing | Enters routine data associated with production processes and enterprise procedures |
| Numeracy | Interprets and analyses a range of mathematical information used in familiar and routine tasks |
| Self-management | Identifies and follows explicit and implicit protocols and meets expectations associated with own role Identifies and follows all procedures for cleaning and maintenance of equipment, incorporating regulatory requirements and enterprise procedures |
| Teamwork | Cooperates with others as part of familiar routine activities, and contributes to specific activities requiring joint responsibility and accountability Identifies what to communicate, with whom and how, in routine work situations |
| Planning and organising | Determines priorities and sequences steps involved in clearly defined, familiar tasks |
| Problem solving | Identifies and anticipates an increasing range of familiar problems, their symptoms and causes, actively looking for early warning signs and implementing contingency plans |
| Technology | Follows routine procedures for using digital technology by reading data from electronic monitoring and control systems |

# Unit Mapping Information

Supersedes and is equivalent to ICPPRN322 Produce complex gravure printed product.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d

Assessment Requirements for ICPPRN3220 Produce complex gravure printed product

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 4.0. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

monitor production output and make necessary adjustments to maintain print quality on a gravure machine while producing a complex print on two occasions for two different jobs.

In the course of the above the candidate must:

operate production process in association with fellow workers and according to enterprise procedures and planned daily schedule

follow work health and safety (WHS) requirements, manufacturer specifications and enterprise procedures

anticipate production difficulties and take preventive action to prevent occurrence.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

enterprise and manufacturer procedures and WHS requirements for:

rewinding and sheeting

handling ink

handling and cleaning blades

cleaning feed, transportation, delivery and in-line sections of the machine and waste disposal

machine shutdown

enterprise escalation procedures and required personnel

required checks of in-line machine components and their function

common issues arising with ink in the machine and correction methods

causes and correction methods of faults and errors such as:

the reel wandering and the web breaking at the unwind unit

sheet cut-off wander

substrate distortion

effects of:

poorly adjusted nip rollers when rewinding and sheeting

incorrect drying temperatures on the finished product or in chillers

excessive pressure applied on the slitters on the job

inadequate work team communication on a gravure printing machine

relative humidity increases in the press room on the substrate

purpose and function of removing solvents from final ink film

storage procedures for printed jobs and printing cylinders

procedures for handling substrates and labelling printed product prior to press room removal

difference between a “flying paster” and “zero speed” type reel-stand

excess ink and additive labelling and storage procedures

waste sorting and disposal procedures, and benefits of reusable waste

link between driers and set-off and marking

methods of controlling ratio of print to in-line speed

outcome if machine guards are removed and micro switches are disconnected on a machine, and the personnel responsible for this action

steps for addressing production problems and eliminating processing of unacceptable product

industry standards of client communication and client approval procedure for printed product

procedures for reporting gravure machine problems and calling service personnel

further operations required for printed reels after removal from the printing machine

use of completed records in the final analysis of the job and their benefits

product quality requirements and machine adjustment procedures to achieve them

manufacturer manuals, enterprise procedures  and work health and safety documentation, including their location and purpose.

# Assessment Conditions

Gather evidence to demonstrate consistent performance in conditions that are safe and replicate the workplace. Noise levels, production flow, interruptions and time variances must be typical of those experienced in the printing field of work and include access to:

special purpose tools, equipment and materials

gravure printing machines and in-line units.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d