# Assessment Requirements for ICPPRN322 Produce complex gravure printed product

Release: 1

Assessment Requirements for ICPPRN322 Produce complex gravure printed product

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 1.0. |

# Performance Evidence

Evidence of the ability to:

operate a gravure press ensuring an efficient non-routine production flow that maintains product quality standards; any production problems are rectified with minimum downtime; and the machine is correctly shut down and cleaned according to work health and safety (WHS) guidelines

monitor production output and make necessary adjustments to maintain print quality on a gravure machine while producing a complex print on TWO occasions (if possible using different substrates and, if possible, including at least two in-line processes) according to job specifications and enterprise procedures.

Note: If a specific volume or frequency is not stated, then evidence must be provided at least once.

# Knowledge Evidence

To complete the unit requirements safely and effectively, the individual must:

### Reel transportation and web control

identify what can cause the reel to wander

describe what can cause the web to break at the unwind unit

outline the difference between a “flying paster” and “zero speed” type reel-stand

discuss what print fault results from the reel being run out of centre

describe types of faults in the unwind section which can cause a web break

### Reel delivery for rewinding and sheeting

discuss WHS risks associated with rewinding and sheeting

describe the safety feature in the delivery system for when the web jams

discuss the cause of sheet cut-off wander

outline effects of poorly adjusted nip rollers when rewinding and sheeting

### Printing and drying unit

discuss how a build-up of ink on the impression cylinder can affect printed product

outline the cause of ink foaming in the ink tray

describe the effect of too much reducer in the ink

identify how to reduce doctor blade wear

describe why it is necessary to remove all solvents from the final ink film

discuss the link between driers and set-off and marking

identify what causes the substrate to distort

discuss what happens in the chillers if the drying temperature is too low

outline the effect of incorrect drying temperature on finished product

### In-line processes

discuss why it is necessary to frequently examine in-line components of the job

outline how consistency of the punching unit is checked

describe the impact of excessive pressure on the slitters

discuss the benefit of identification numbers on jobs with multiple similar images

describe how the ratio of print to in-line speed is controlled

### Maintaining production process

discuss possible effects of inadequate communication within the work team on a gravure printing machine

describe safety features within the organisation which aid in maintaining effective production

identify ramifications if machine guards are removed and/or micro switches are disconnected on a machine

name the person held legally responsible for the removal of machine guards and/or disconnection of micro switches

discuss the most accurate method of checking register during a production run

outline why it is necessary to take immediate action when production problems are anticipated

describe what to do to eliminate further processing of unacceptable printed product

discuss what happens to the substrate if the relative humidity increases in the press room

outline the correct procedure to care for a newly delivered substrate to the press room

discuss why waste should be sorted

describe advantages of keeping reusable waste

outline industry standards to enhance effective communication with clients

discuss procedures a client should follow to OK a printed product

### Gravure machine operating problems

describe when it is necessary to call service personnel to correct a machine problem

identify enterprise procedures to report machine operating problems

### Shutdown procedures

discuss what happens if correct shutdown procedures are not followed

describe why it is necessary that correct shutdown procedures are conducted with fellow workers

outline advantages of proper labelling and storage of excess inks and materials

discuss why printed product should be clearly labelled prior to removal from the press room

outline further operations required for printed reels after removal from the printing machine

describe how printed jobs should be stored after removal from the printing machine

### Cleaning and washing the printing unit

list WHS concerns of handling ink

outline safety precautions for cleaning the printing cylinders

discuss why it is necessary to thoroughly clean and wash the printing unit prior to the next print run

describe why the doctor blades should be thoroughly cleaned

explain why doctor blades should be handled with extreme care

describe how to store printing cylinders to minimise damage

### Cleaning feed, transportation, delivery and in-line sections

list WHS precautions for cleaning these sections of the machine

discuss why it is necessary to maintain a clean substrate handling section of the machine

### Completing production records

outline how completed records are used in the final analysis of the job

describe benefits of comprehensive records when considering production of future jobs

identify what machine manuals, safety and other documentation are relevant to this task, where they are they kept and describe the information contained.

# Assessment Conditions

Gather evidence to demonstrate consistent performance in conditions that are safe and replicate the workplace. Noise levels, production flow, interruptions and time variances must be typical of those experienced in the printing field of work and include access to:

special purpose tools, equipment and materials

gravure printing machines and in-line units.

Assessors must satisfy NVR/AQTF assessor requirements.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d