# Assessment Requirements for ICPPRN3310 Set up for basic lithographic printing

Release: 1

Assessment Requirements for ICPPRN3310 Set up for basic lithographic printing

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 4.0. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

set up for two basic lithographic printing jobs according to manufacturer specifications and enterprise procedures.

In the course of the above the candidate must:

demonstrate use of control, monitoring and data entry systems

demonstrate all safety devices on the machine.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

enterprise procedures, manufacturer specifications, and work health and safety (WHS) requirements for:

setting up and operating the machine, in-feed and transfer systems

webbing up the machine in reel systems

the rewind of the reel delivery system

handling inks and additives

enterprise escalation procedures and required personnel, including those who pass colour prior to running the job and have final say in job approval

job documentation or production control system, and procedures if vital information is missing

checks required prior to set-up, including availability of materials

register checks, and machine adjustments that may cause mis-register

procedures and steps for:

identifying grip edge of the plate and printing side of material

determining sheet position before transportation to printing unit

modifying ink that is light and adjusting ink colour and consistency

determining cylinder (plate, blanket and impression) specifications for the job

proof testing the machine

effect on the job of:

printing plate cylinder being overpacked or not cleaned prior to plate fitting

incorrect web tension on the print and the rewind of the machine

side lay selection on the job

excessive jogging on the stack

an incorrectly set dampening system

the purpose or function of:

nip rollers

using a two-sheet cut or setting sheet up in middle of the machine

slowdown devices in delivery

consistently tensioning plates

plate installation, features assisting installation and actions or tools likely to damage the plate

types of web splices required for the job

machine’s method of detecting missing or late sheets

reasons for applying spray powder and effect of excessive use

criteria to check in ink for job suitability and initial print prior to running

environmental concerns and regulations concerning inks and additives

ink profiles and their variance across the machine

optimum ink duct sweep

precautions for UV drying on the ink film

methods to minimise waste during make ready

adjustments required to position the image laterally and circumferentially

manufacturer manuals, enterprise procedures and work health and safety documentation, including their location and purpose.

# Assessment Conditions

Gather evidence to demonstrate consistent performance in conditions that are safe and replicate the workplace. Noise levels, production flow, interruptions and time variances must be typical of those experienced in the printing field of work and include access to:

special purpose tools, equipment and materials

lithographic machine.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d