# Assessment Requirements for ICPPRN3320 Produce complex lithographic printed product

Release: 1

Assessment Requirements for ICPPRN3320 Produce complex lithographic printed product

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 4.0. |

# Performance Evidence

The candidate must demonstrate the ability to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including evidence of the ability to:

demonstrate use of control, monitoring and data entry systems

monitor production output and make necessary adjustments to maintain print quality on a lithographic machine while producing a complex print on two occasions.

In the course of the above the candidate must:

operate production process in association with fellow workers and according to enterprise procedures and planned daily schedule

follow work health and safety (WHS) requirements, manufacturer specifications and enterprise procedures

anticipate production difficulties and take preventive action to prevent occurrence.

# Knowledge Evidence

The candidate must be able to demonstrate knowledge to complete the tasks outlined in the elements, performance criteria and foundation skills of this unit, including knowledge of:

enterprise and manufacturer procedures, and WHS requirements for:

setting up and operating the sheet and reel delivery and transportation systems

rewinding in the reel delivery system

handling inks and materials

cleaning parts of the machine and waste disposal

enterprise escalation procedures and required personnel

causes of:

the reel wandering and web breaking at the unwind unit

the sheet cut-off wandering

mis-register when the sheet is in the feeder

an uneven feeder stack

the web jamming up in the folder

blanket packing-creep during printing

ink lying back in the duct and ink stripping on the inking rollers

conductivity of the fountain solution changing over an eight hour shift

UV ink drying and substrate blistering

problems that arise from the following and their correction methods:

the reel being run out of centre

poorly adjusted nip rollers in rewinding and sheeting

worn suckers at the feeder suction head

gripper malfunction on sheet control and transfer

the feeder stack not being loaded levelly

excessive pressure on the slitters

dirty formers and adjusting rollers at their base

defective pins in the folder

grippers not being set as required in the machine transfer section

plates developing a crack at grip edge during a print run

blankets not being tensioned as required

excessive use of fountain solution on the plate

excessive conductivity

drying temperature being too low in the chillers or incorrect on finished product

inadequate communication within the work team

features and functions, common issues and faults and methods to resolve them for the following machine sections in all systems:

transportation

delivery

in-line processing units

printing units

the rewind unit

client communication standards and approval process for a finished product

plate storage procedures

machine problems where service personnel must be contacted and reporting procedures

labelling procedures for printed reels and their purpose

effect of machine speed on sheet delivery

difference between a “flying paster” and “zero speed” type reel-stand

visible signs of the sheet being registered in the feeder and by side lay

link between driers, set-off and marking, and use of anti-set-off powder in delivery

the enterprise personnel responsible for removing machine guards and disconnecting micro switches and its effects

disadvantages of using a closed looped system for automatic control of the printed product

available measurements besides optimum solid ink density used to assess print quality

actions required when production problems are anticipated and further processing of unacceptable printed product must be eliminated

use of completed records in final job analysis and their benefits in production of future jobs

manufacturer manuals, enterprise procedures  and work health and safety documentation, including their location and purpose.

# Assessment Conditions

Gather evidence to demonstrate consistent performance in conditions that are safe and replicate the workplace. Noise levels, production flow, interruptions and time variances must be typical of those experienced in the printing field of work and include access to:

special purpose tools, equipment and materials

lithographic printing machine.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guide is found on VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d