# Assessment Requirements for ICPPRN393 Set up for basic relief printing

Release: 1

Assessment Requirements for ICPPRN393 Set up for basic relief printing

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 1.0. |

# Performance Evidence

Evidence of the ability to:

set up reel- or sheet-fed platen, cylinder or rotary printing machines for routine print jobs

conduct proof run and adjust settings to ensure production speeds are attained

use computerised control, monitoring and data entry systems if available and appropriate

find and use information relevant to the task from a variety of information sources

demonstrate all safety devices on the machine

manipulate embellishment tools and operations

set up for TWO basic relief printing jobs (if possible including at least one in-line process) according to manufacturer’s specifications and enterprise procedures.

Note: If a specific volume or frequency is not stated, then evidence must be provided at least once.

# Knowledge Evidence

To complete the unit requirements safely and effectively, the individual must:

### Job specifications

explain what to do if vital information is missing from the job ticket

outline checks to do prior to set-up

### Relief plates

explain effect of plates with poor relief

explain importance of calliper of mounting material

explain how to position plate on the mount ensuring plate edges do not lift

### Reel in-feed

outline major work health and safety (WHS) concerns when setting up the reel in-feed

explain how to determine printing side of the material

outline effects of low web tension on the print

explain other types of web splices that can be used appropriately for the job

### Sheet in-feed

outline major WHS concerns of setting up the sheet in-feed

explain how sheet position is determined for the job

outline effects that side lay selection has on the job

explain how to appropriately select front lays

list procedures for a register check

explain why a two-sheet cut is used on most feeders

explain how the machine knows if the sheet is missing or late

### Reel only delivery system

explain effects of excessive web tension at rewind of the machine

outline major risks associated with rewind of the machine

### Sheet only delivery system

explain why application of spray powder is sometimes advisable

list effects of too much spray powder

explain the use of slowdown devices in the delivery

outline effects of excessive jogging on the stack

### Preparation of inks and additives

outline main environmental and WHS concerns about inks and additives

explain how to match ink to a particular job

outline results of ink being too tacky

explain the process for changing ink that is too light

outline methods available to check ink for correct colour

explain responsibility for passing the colour prior to running the job

### Machine set-up

outline WHS concerns of setting up the machine

explain how to determine specifications relating to the specific job

explain effects incorrectly set inking rollers have on the print

outline ways the ink profile may vary across the machine

explain what ‘optimum ink duct sweep’ refers to

### Basic in-line processes

outline precautions to take if UV drying is to be utilised to dry the ink film

list steps to incorporate in-line processes into the make ready

explain how equipment used in in-line processing can be protected against damage during set-up

### Proofing and adjustment

outline methods to minimise waste during make ready

list procedures to be adopted to have the print approved

list quality control measurements applied to the print to test against known standards

outline checks on the initial print prior to running

explain how settings to be adjusted are determined

summarise the process to plot success of the machine adjustment

summarise how to record final results for future reference

### Source information

outline a range of information sources relevant to basic relief printing.

# Assessment Conditions

Gather evidence to demonstrate consistent performance in conditions that are safe and replicate the workplace. Noise levels, production flow, interruptions and time variances must be typical of those experienced in the printing field of work and include access to:

special purpose tools, equipment and materials

reel- or sheet-fed platen

cylinder or rotary printing machines

machine manuals, safety and other documentation relevant to basic relief printing.

Assessors must satisfy NVR/AQTF assessor requirements.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d