# ICPPRN531 Set up for specialised lithographic printing

Release: 1

ICPPRN531 Set up for specialised lithographic printing

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 1.0. |

# Application

This unit describes the skills and knowledge required to set up for specialised lithographic printing.

It applies to individuals who can problem-solve and experiment with substrate and press settings for either wide or narrow reel- or sheet-fed lithographic printing machines for specialised print jobs.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Unit Sector

Printing

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Confirm specialised job specifications | 1.1 Job requirements are read and interpreted from job documentation or production control system 1.2 Availability of all job related components is checked |
| 2. Plan and carry out specialised set-up | 2.1 Specialised job specifications are identified and analysed 2.2 Special set-up requirements are determined 2.3 Specialised set-up is completed in minimum time with minimum wastage |
| 3. Set up delivery systems | 3.1 Delivery systems are set up and adjusted according to job specifications 3.2 Substrate is added to and removed from process according to job specifications |
| 4. Select and prepare inks and additives | 4.1 Colour sequence for the job is considered and confirmed 4.2 Inks, dyes or additives are evaluated with reference to specialised requirements 4.3 Inks, dyes or additives are selected according to job specifications 4.4 Inks, dyes and additives are prepared according to work health and safety (WHS) requirements, and manufacturer’s/supplier’s instructions with suitable precautions to minimise waste 4.5 Correct colour and weight/volume of ink are mixed and prepared to match requirements of the printing process and job specifications 4.6 Formulation of ink, colour match and approved colour are appropriately recorded 4.7 Inks, dyes and additives are appropriately labelled, handled and stored according to manufacturer’s/supplier’s instructions to prevent damage and hazards to personnel and to prolong shelf life |
| 5. Set up machine for specialised lithographic printing | 5.1 Plate cylinder is set up, evaluated and adjusted according to job specifications 5.2 Lithographic plate is selected and prepared for installation 5.3 Plates are correctly mounted safely and according to job specifications 5.4 Blanket and blanket cylinder are set up, evaluated and adjusted according to job specifications 5.5 Impression cylinder is set up, evaluated and adjusted according to job specifications 5.6 Inking system is set up, evaluated and adjusted according to the lithographic process and job specifications 5.7 Dampening system is set up, evaluated and adjusted according to job specifications 5.8 Drying system is set up, evaluated and adjusted according to job specifications |
| 6. Conduct specialised proof run | 6.1 Material to be used for proof is organised correctly 6.2 Machine is operated according to manufacturer’s specifications and enterprise procedures to produce a specified proof 6.3 Specialised proof is visually inspected and/or tested or laboratory testing is organised according to enterprise procedures 6.4 Production does not commence without client approval or authority where appropriate 6.5 Results are interpreted and evaluated with adjustments carried out according to product and machine specifications |
| 7. Troubleshoot machinery and material problems | 7.1 Corrective or preventive action is recommended and implemented where appropriate 7.2 Changes are communicated to relevant personnel in a logical and easily understood manner 7.3 Changes are monitored to confirm improvement to production efficiency 7.4 Ongoing problems are reported according to enterprise procedures |

# Foundation Skills

This section describes language, literacy, numeracy and employment skills incorporated in the performance criteria that are required for competent performance.

| Skill | Performance Criteria | Description |
| --- | --- | --- |
| Reading | 1.1, 2.1, 4.3-4.5, 4.7, 5.1, 5.3-5.8, 6.5, | Recognises and interprets text to establish job requirements from information contained within relevant procedures and specifications |
| Writing | 4.6, 4.7, 6.4, 7.2, 7.4 | Completes workplace recording accurately Demonstrates sophisticated control of a broad range of text types |
| Oral Communication | 1.1, 2.1, 4.3, 4.5, 5.1, 5.3-5.8, 6.4, 7.2, 7.4 | Participates effectively in spoken interactions by using strategies to confirm, clarify or repair understanding |
| Numeracy | 4.5 | Interprets and analyses a range of mathematical information used in familiar and routine tasks |
| Navigate the world of work | 1.2, 2.1, 2.3, 4.2-4.7, 5.1, 5.3-5.8, 6.2-6.5, 7.3, 7.4 | Recognises and follows explicit and implicit protocols and meets expectations associated with own role Complies with WHS, legislative requirements and follows organisational policies and procedures relevant to own role |
| Interact with others | 1.1, 2.1, 5.3, 5.5, 6.1, 6.3-6.8, 7.4, 8.2, 8.4 | Seeks to cooperate with others to achieve results in immediate work context |
| Get the work done | 1.2, 1.3, 2.1-2.3, 4.1-4.5, 4.7, 5.1, 5.3-5.8, 6.1-6.3, 6.5, 7.3, 7.4 | Plans, evaluates, identifies and assembles resources required to complete tasks Determines priorities and sequences steps involved in complex tasks with a focus on production efficiency May use formal analytical and lateral thinking techniques to identify issues, generate and evaluate possible solutions Is acutely aware of the importance of understanding, monitoring and controlling access to digitally stored and transmitted information Uses familiar digital systems and tools to access, organise, analyse and display information relevant to role Reads data from electronic monitoring and control systems |

# Range of Conditions

This section specifies different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included.

| Delivery systems MUST include EITHER: | reel OR sheet |
| --- | --- |
| Setting up and adjusting MUST include EITHER: | set-up options for unwind and rewind reels webbing procedures and web control system splicing/joining reels printed web viewing devices folder and sheeter set off / marking prevention devices OR feeder and delivery sections sheet pick-up, transportation, control and transfer systems set off / marking prevention devices |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| ICPPRN531 Set up for specialised lithographic printing | ICPPR531C Set up for specialised lithographic printing | Updated to meet Standards for Training Packages | Equivalent unit |

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d

Assessment Requirements for ICPPRN531 Set up for specialised lithographic printing

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICP Printing and Graphic Arts Training Package Version 1.0. |

# Performance Evidence

Evidence of the ability to:

set up either wide or narrow reel or sheet-fed lithographic printing machines for specialised print jobs; conduct a proof run and adjust settings to ensure production speeds are attained in minimum time with minimum wastage

set up a lithographic printing machine for a specialised job on TWO occasions using different substrates and sheet sizes if sheet-fed and including at least two in-line processes according to manufacturer’s and job specifications, enterprise procedures and the performance criteria.

find and use information relevant to the task from a variety of information sources

Note: If a specific volume or frequency is not stated, then evidence must be provided at least once.

# Knowledge Evidence

To complete the unit requirements safely and effectively, the individual must:

explain why it is necessary that job specifications are read and properly understood

discuss production problems that could eventuate if job specifications are not read and properly understood

identify who to discuss any production problems with

outline the problem that can result from the plate cylinder not being cleaned prior to plate fitting

describe what happens if the plate is over tensioned during fitting

list faults that can result from a plate being under tensioned

### For reel systems only

describe the need for accurate plate bending on a web-fed machine

explain how to determine position of the reel

describe the effect if brake tension is not set correctly

discuss the function of the ‘dancer’ roller on a web machine

outline how to determine position of the bustle wheels

explain what happens if the web is not spliced correctly

describe the operation of the particular web viewing device

explain how the web is controlled in the rewind unit

outline the function of a slitter on a web machine

discuss what causes the web to jam up in the folder

explain the need to disengage the folder if sheeting

list problems that attributable to a blunt knife when sheeting

outline the safety feature in the delivery system for when the web jams up

describe which fold is always made with the grain of the web

explain the type of folder that folds the web in half in the direction of the web grain

outline remedial steps to take if there is a possibility of ink marking in the folder

describe the main reason for having a silicone applicator on a web machine

explain the main reason for blistering on a heat set machine

explain what the oven evaporates from the ink

discuss the function of chill rollers on a web machine

### For sheet systems only

describe work health and safety (WHS) factors to consider when setting up the sheet transportation and delivery systems

explain what causes more than one sheet to be picked up in the feeder

discuss the need for accurate feeder set up

describe how to determine position of the sheet prior to transfer to the printing unit

discuss how to determine which front lays to use

explain the type of substrate that requires additional front lays to be engaged

explain the need for additional front lays when printing this type of substrate

list the WHS precaution to observe when setting up the delivery

list WHS precautions for removing sheets from the delivery

explain what causes incorrect sheets delivery

outline necessary adjustments when changing from lightweight to heavyweight stocks

explain what determines sheet release into the delivery

list problems caused by excessive use of anti-set off spray powder

describe what might cause printed sheets to set off in the delivery

discuss how the possibility of set off in the delivery might be reduced

explain what can happen if there is excess vacuum on the slow-down wheels

list WHS precautions that must be observed when preparing inks and additives

describe the procedure used to lengthen print length on this type of press

discuss the procedure used to shorten print length on this type of press

outline considerations of deciding colour sequence

point out the visual aid on the plate that identifies ink colour to be used

discuss all WHS precautions for webbing up the machine

outline details to check to determine an ink’s suitability for the printing process

describe what special end-use requirements might be necessary

explain the reason for mixing additives in the ink

explain how a spectrophotometer can help assess ink colour

describe the formula for calculating correct quantity of lithographic ink

discuss the print fault that occurs if excessive driers are mixed into the ink

outline precautions to minimise waste when preparing ink

explain shelf life of most inks

discuss conditions relevant to storage of inks and additives

explain conventions for labelling mixed inks

list WHS factors for setting up the machine

describe checks made on the plate prior to fitting

explain how much plate packing is required

describe normal printing pressure required between plate and blanket

explain how to determine correct printing pressure between blanket and stock

explain the ideal blanket surface condition

describe how to achieve correct blanket tension when fitting a new blanket

describe print faults likely if the impression cylinder is not maintained

explain the order eccentric or concentric roller adjustments should be made in

explain what the width of the contact stripe between two rollers should be when setting the rollers

describe how to determine the ink duct setting

explain the ideal ink duct sweep setting

discuss recommended degrees shore hardness for bare back and conventional dampeners

describe what conductivity of the fountain solution should be

explain why it is important to constantly check conductivity of the fountain solution

explain how to change the amount of fountain solution across the plate surface

outline the need to adjust the fountain solution laterally

list reasons it might be difficult to engage the perfecting unit

list types of ink drying/curing systems

describe what the true inch function feature fitted to some machines does

list what may cause the machine to keep stopping

outline necessary checks prior to engaging the impression

describe checks performed when running the machine

describe the effect position of certain guards has on operation of the machine

explain how steps involved in operating the machine should be communicated to other team members

describe aids available for the testing of the machine proof

discuss where testing should take place

explain the function of a polarisation filter in a densitometer

describe ideal conditions for inspecting the proof

discuss why it is necessary to use visual aids on the printed sheets

describe the acceptable wet trap value range for lithographic inks

outline an indication of optimum solid ink density in the absence of a proof

explain the result of low solid ink density and excessive dot gain

describe available methods to check and adjust ink colour and consistency

list adjustments that may cause mis-register

list adjustments made to position the image laterally

list adjustments made to position the image circumferentially

list adjustments made to position the image diagonally

discuss the effect of changing colour sequence on the wet trap value

explain the difference between mechanical and optical dot gain

outline the cause of excessive mechanical dot gain

explain who has responsibility for the final say in the OK of the job

identify machine manuals, safety and other documentation relevant to this task, where they are kept, and discuss information included in these documents.

# Assessment Conditions

Gather evidence to demonstrate consistent performance in conditions that are safe and replicate the workplace. Noise levels, production flow, interruptions and time variances must be typical of those experienced in the printing field of work and include access to a lithographic printing machine.

Assessors must satisfy NVR/AQTF assessor requirements.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a74b7a0f-a253-47e3-8be0-5d426e24131d