# ICTCBL209 Joint and terminate coaxial cable

Release: 1

ICTCBL209 Joint and terminate coaxial cable

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICT Information and Communications Technology Training Package Version 2.0. |

# Application

This unit describes the skills and knowledge required to install, joint and terminate coaxial cable in the customer access network (CAN) environment for emerging technologies using high speed broadband and the delivery of cable television services on a client’s premises for domestic, commercial or industrial installations.

It applies to technical staff whose work involves jointing and terminating coaxial cable used in all telecommunications applications, including convergence technologies of telephony, data, video and multimedia as part of next generation networks (NGN).

Work functions in the occupational areas where this unit may be used are subject to regulatory requirements. Refer to the ICT Implementation Guide Companion Volume or the relevant regulator for details of licensing, legislative or certification requirements.

# Unit Sector

Telecommunications – cabling

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare to install and joint coaxial cable | 1.1 Confirm approval for site access with client prior to site entry and comply with site security arrangements and relevant legislation, codes, regulations and standards 1.2 Read and interpret client’s installation specifications and physical conditions at site to determine layout of job 1.3 Locate and identify adjoining other services according to enterprise guidelines and work health and safety (WHS) practices 1.4 Test for and respond to presence of dangerous gases in underground enclosures according to enterprise guidelines 1.5 Undertake approved alterations to design according to enterprise guidelines |
| 2. Verify placement and secure coaxial cable | 2.1 Use safety equipment to protect self and public according to enterprise guidelines and WHS 2.2 Maintain coaxial cable segregation to industry standard requirements 2.3 Protect integrity of coaxial shield cable to ensure no loss of performance 2.4 Place cable in position with sufficient slack to allow termination and maintain minimum bend radius according to manufacturer’s specifications 2.5 Follow installation designs to install cable safely without damage to cable or client’s premises 2.6 Maintain radio frequency (RF) signal strength by installing cable lengths within manufacturer’s or design specifications 2.7 Locate securing hardware to reduce cumulative effect on cable wave shape properties and attach cable ties to minimise cable damage |
| 3. Joint coaxial cable | 3.1 Strip coaxial cable according to specifications to required length using appropriate tools 3.2 Select appropriate kit to match type of coaxial cable in use and jointing method according to manufacturer’s recommendations 3.3 Joint cable and ensure jointing fitting retains segregation of conductor and shield 3.4 Seal all joints according to manufacturer’s specifications |
| 4. Terminate coaxial cable | 4.1 Prepare coaxial cable for termination according to specifications using appropriate tool 4.2 Select connectors to match type of coaxial cable in use and use terminating method recommended by manufacturer 4.3 Verify connector fitting retains segregation of conductor and shield 4.4 Terminate connectors to torque as recommended by manufacturer to prevent RF leakage 4.5 Test connectors to mating specifications using gauge tester 4.6 Maintain continuous ground on terminations and waterproof seal to preserve cable integrity |
| 5. Complete installation operation | 5.1 Place cables in enclosure and lay up according to manufacturer’s instructions and enterprise guidelines, and check that no safety hazards are evident 5.2 Place other services according to enterprise guidelines 5.3 Reinstate site and remove waste and debris for disposal according to environmental requirements and to maintain safe worksite conditions |
| 6. Complete installation administration | 6.1 Complete reports according to enterprise policy 6.2 Note alterations to plans using appropriate symbols |

# Foundation Skills

This section describes language, literacy, numeracy and employment skills incorporated in the performance criteria that are required for competent performance.

| Skill | Performance Criteria | Description |
| --- | --- | --- |
| Reading | 1.2-1.5, 2.5, 2.6, 3.2, 3.4, 4.2, 4.4, 6.1, 6.2 | Reads familiar texts fluently, automatically recognising most everyday words and some specialised vocabulary |
| Writing | 1.5, 6.1, 6.2 | Sequences writing to produce cohesive text and uses layout consistent with text type |
| Oral Communication | 1.1 | Demonstrates awareness of choices for register, especially in situations that are familiar |
| Numeracy | 1.5, 2.4, 2.5, 3.1, 4.4, 4.5 | Interprets and comprehends whole numbers and familiar or routine fractions, decimals and percentages Selects and uses appropriate tools, hand-held devices, computers and technological processes |
| Navigate the world of work | 1.1, 1.3-1.5, 2.1, 2.2, 2.4, 2.7, 4.5, 5.1, 5.3, 6.1 | Takes responsibility for decisions regarding when and how to complete tasks, and coordinate with or delegate to others Takes personal responsibility for adherence to legal and regulatory responsibilities relevant to own work context, and draws attention to any issues that may affect self or others |
| Get the work done | 1.3-1.5, 2.2-2.6, 3.1-3.4, 4.1, 4.2, 4.4, 4.6, 5.2 | Implements actions as per plan, makes slight adjustments if necessary, and addresses unexpected issues Automatically implements standard procedures for routine decisions |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| ICTCBL209 Joint and terminate coaxial cable | ICTCBL2066A Joint and terminate coaxial cable | Updated to meet Standards for Training Packages | Equivalent unit |

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=a53af4e4-b400-484e-b778-71c9e9d6aff2