# ICTCBL214 Fix aerial cable

Release: 1

ICTCBL214 Fix aerial cable

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICT Information and Communications Technology Training Package Version 2.0. |

# Application

This unit describes skills and knowledge required to install aerial cables in client and access networks. It involves installing, terminating and securing cables. Aerial cable installation may be for a new cable, a cable upgrade or a cable in need of repair.

It applies to individuals working as telecommunications linesmen and line installers who use support anchors and catenaries for new installations and upgrades, or to maintain existing networks in domestic, commercial and industrial installations to deliver services in x-digital subscriber line (xDSL), fibre to the home (FTTH) and hybrid fibre coaxial (HFC) networks.

Work functions in the occupational areas where this unit may be used are subject to regulatory requirements. Refer to the ICT Implementation Guide Companion Volume or the relevant regulator for details of licensing, legislative or certification requirements.

# Unit Sector

Telecommunications – cabling

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for aerial cable installation | 1.1 Obtain construction plan from appropriate personnel to scope work and arrange for site access 1.2 Notify appropriate personnel of identified safety hazards at cabling worksite 1.3 Determine cable route and type of cable from construction plan, identifying and avoiding other services 1.4 Obtain plant, tools, safety equipment and material to perform tasks safely and efficiently |
| 2. Install aerial cable | 2.1 Use tools according to enterprise guidelines and work health and safety (WHS) regulations 2.2 Install catenary wire, gantry wire or integral bearer cable (IBC) and tension to required specifications 2.3 Install cable according to current Australian Standards and enterprise guidelines, ensuring that no damage is caused and that physical characteristics of cable are maintained 2.4 Secure cable permanently to support structure using aerial fixing devices according to manufacturer’s and enterprise guidelines 2.5 Number cables on towers according to enterprise guidelines |
| 3. Terminate, seal and secure aerial cable | 3.1 Terminate and seal cables according to manufacturer’s specifications 3.2 Joint cable in suitable closures using enterprise guidelines 3.3 Loop and secure cable on support structure with bending radius tolerance for cable materials to reduce damage to conductors 3.4 Test cable for continuity and rectify fault if required 3.5 Record test results for future reference |
| 4. Complete project | 4.1 Complete reports on installation and design amendments and file according to enterprise requirements 4.2 Recover obsolete materials and equipment and return to appropriate point for disposal 4.3 Restore site according to requirements of enterprise or approving authority and to client satisfaction 4.4 Notify appropriate personnel of job completion and obtain sign off |

# Foundation Skills

This section describes language, literacy, numeracy and employment skills incorporated in the performance criteria that are required for competent performance.

| Skill | Performance Criteria | Description |
| --- | --- | --- |
| Reading | 1.1, 1.3, 2.2, 2.3, 4.1 | Interprets information from technical sources to identify relevant and key information |
| Writing | 1.2, 3.5, 4.1 | Documents outcomes and changes to designs using industry relevant terminology |
| Numeracy | 1.3, 2.2, 2.5, 3.3 | Interprets scales and ratios on plans Performs basic calculations required to take accurate measurements |
| Navigate the world of work | 1.2, 2.1, 2.3-2.5, 3.1, 3.2, 4.3 | Takes personal responsibility for adherence to legal, regulatory and organisational requirements relevant to own work context |
| Interact with others | 1.1, 4.3, 4.4 | Identifies and follows accepted communication practices and protocols when liaising with clients and internal and external personnel on technical and operational matters |
| Get the work done | 1.1-1.4, 2.1, 2.3-2.5, 3.1, 3.2, 4.2, 4.3 | Determines job sequence and works logically and systematically to undertake clearly defined tasks Analyses task requirements to decide on appropriate equipment and practices Implements actions as per plan, making slight adjustments if necessary, and addressing some unexpected issues Automatically implements standard procedures for routine decisions in response to familiar problems Determines and uses appropriate equipment |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| ICTCBL214 Fix aerial cable | ICTCBL2134A Fix aerial cable | Updated to meet Standards for Training Packages Performance Criteria clarified | Equivalent unit |

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=a53af4e4-b400-484e-b778-71c9e9d6aff2