# ICTCBL331 Conduct basic identification and fault-finding within cabling networks and customer equipment

Release: 1

ICTCBL331 Conduct basic identification and fault-finding within cabling networks and customer equipment

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with ICT Information and Communications Technology Training Package Version 5.0. |

# Application

This unit describes the skills and knowledge required for fault-finding and testing for the presence of an operational subscriber line at network termination devices (NTDs). These lines are crucial in connecting customer equipment to the carrier for the purposes of providing telephony and internet protocol (IP) data services. These network providers use convergent technologies to deliver services of next generation networks (NGNs).

It applies to technicians and cable installers who install and maintain IP-based equipment for customer and service providers for services, such as internet protocol TV (IPTV), IP security, digital home networks, IP-based cable access, TV (CATV), IP core and access networks, home automation, interactive TV, smart grids and cloud computing

Australian Communications and Media Authority (ACMA) cabling registration applies to maintaining or installing new cable elements. Refer to the ICT Information and Communications Technology Companion Volume Implementation Guide or the relevant regulator for details of licensing, legislative or certification requirements.

# Unit Sector

Telecommunications – Cabling

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Plan to locate and rectify basic faults | 1.1 Arrange site access according to enterprise procedures and customer requirements 1.2 Obtain information from customer about nature of the fault 1.3 Obtain suitable tools and test equipment 1.4 Obtain appropriate personal protective equipment (PPE) 1.5. Identify requirements for basic identification and fault-finding of cabling from network termination device (NTD) to customer equipment and seek assistance as required |
| 2. Identify and confirm customer and NTD equipment in place | 2.1 Identify type of NTD used and its application to satellite, fibre, fixed radio, hybrid fibre coaxial (HFC) or digital subscriber line (DSL) telecommunications network/s 2.2 Identify and document typical equipment connected to NTD 2.3 Confirm correct equipment is being used for installed NTD |
| 3. Identify and inspect NTD and customer equipment cables and patch cords | 3.1 Check that NTD equipment and customer equipment are interconnected utilising cables and patch cords 3.2 Ensure all cabling sockets are visually intact and undamaged 3.3 Ensure all patch cords are visually intact and undamaged 3.4 Complete wire mapping to ensure new work is completed correctly and is in good working order 3.5 Ensure all cable, patch cords and cabling products are of the same signal bandwidth category 3.6 Obtain NTD and customer equipment user guides 3.7 Utilise NTD and customer equipment user guides to determine correct light-emitting diode (LED) status that indicates the equipment is operating correctly 3.8 Identify and record visual status of all customer equipment and NTD equipment prior to any investigative process |
| 4. Repair faults found in NTD and customer equipment | 4.1 Follow appropriate user guide fault tables to remedy equipment status according to equipment manufacturer guidelines 4.2 Conduct basic fault-finding using methodical and safe practices suitable for system and problem type 4.3 Ensure cabling or patch cord elements are correctly terminated utilising wire mapping testers according to industry standards 4.4 Correct faulty cabling by re-terminating sockets, replacing sockets and re-terminating cable or replacing damaged cable elements 4.5 Correct faulty patch cords by re-terminating plugs or replacing with new patch cords 4.6 Power down equipment for 30 seconds and reboot equipment as a remedy to problem/s 4.7 Report failure of equipment to obtain normal working status to customer/supervisor and/or NTD owner for a determination to replace faulty equipment or to have equipment repaired 4.8 Ensure test equipment power adaptors are providing correct voltage to equipment, using an appropriate test device, and replace power adaptors as required 4.9 Ensure NTD equipment and customer equipment is communicating with internet by exploring internet with a computer browser gaining correct page displays from a website 4.10 Ensure customer device is connected to a wi-fi modem using service set identifier (SSID) and password |
| 5. Complete documentation and clean-up worksite | 5.1 Advise customer of successful fault clearance and obtain sign-off for customer agreement once fault is resolved and for invoicing purposes 5.2 Complete all records as required 5.3 Complete reports to justify fault diagnosis and rectification methodology as required 5.4 Remove all waste and debris from worksite and dispose of according to environmental requirements 5.5 Restore changes made to worksite during fault repair, according to industry standards |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance, but not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Identifies, plans and implements strategies to manage gaps in technical knowledge |
| Writing | Recognises when unknown words are essential to meaning and uses a range of decoding strategies to identify them Interprets textual information from relevant sources to identify relevant and key information |
| Oral Communication | Uses clear language and concepts, and tone and pace appropriate for the audience and purpose |
| Numeracy | Articulates information clearly, using specific and relevant language suitable to audience to convey recommendations and provide verbal reports Uses listening and questioning techniques to confirm understanding |
| Navigate the world of work | Takes readings and measurements and interprets results |
| Interact with others | Takes personal responsibility for adherence to legal and regulatory responsibilities relevant to own work context, and draws attention to any issues that may affect self or others Recognises and follows explicit and implicit protocols and meets expectations associated with own role |
| Get the work done | Identifies and follows accepted communication practices and protocols when liaising with internal and external personnel on technical and operational matters |

# Unit Mapping Information

No equivalent unit. New unit.

# Links

Companion Volume Implementation Guides are available from VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a53af4e4-b400-484e-b778-71c9e9d6aff2

Assessment Requirements for ICTCBL331 Conduct basic identification and fault-finding within cabling networks and customer equipment

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with ICT Information and Communications Technology Training Package Version 5.0. |

# Performance Evidence

The candidate must demonstrate the ability to perform the tasks outlined in the elements, performance criteria, and foundation skills, and to do this on at least one of the following network termination device (NTD)/customer equipment systems:

satellite modem/ethernet/telephony

fixed wireless modem/ethernet/telephony

fibre optic TX-RX/ethernet/telephony

hybrid fibre coaxial (HFC) modem/ethernet/telephony

digital subscriber line (DSL) modem/ethernet/telephony.

Depending on the system/s the candidate works on, the candidate must demonstrate the ability to:

connect NTDs to customer equipment

set-up and operate test instruments

gather appropriate instrument test cables and adaptors

determine status of cables/equipment to be tested such as safety and live or inactive status

identify different cable types and their association with NTD/customer equipment

identify whether or not NTD/customer equipment is operating correctly

search the internet for equipment user guides

replace faulty cable or patch cords after testing or identifying damage

replace or re-terminate faulty sockets or connectors

perform a browser search of a web page to confirm system is working.

# Knowledge Evidence

The candidate must demonstrate the knowledge required to complete the tasks outlined in the elements, performance criteria, and foundation skills, which includes knowledge about:

relevant legislation, regulations, codes, standards and associated industry guidelines, in particular:

Australian Communications and Media Authority (ACMA) regulations

work health and safety (WHS) regulations for electrical, fibre optic and electromagnetic telecommunications systems

hazard identification such as laser light and safety signs used for telecommunications equipment

typical equipment connected to NTD including:

personal computer devices including:

desktop/mobile computing devices

smartphone

printers

other devices

wi-fi modem/switch/router

DSL filters

internet service provider (ISP) gateway switch/router (ISP or retailer)

internet protocol TV (IPTV)

IP security, digital home networks

IP-based cable access TV (CATV)

IP core and access networks

home automation

interactive TV

smart grids

cloud computing

basic interconnection concepts for NTD and customer equipment application including:

fibre optic

fixed radio

satellite

HFC

DSL twisted pair

cable types used for NTD/customer equipment connections including:

Cat 5e unshielded twisted pair (UTP) cable

Cat 6 UTP cable

Cat 6/7 shielded twisted pair (STP) cable

RG6 coaxial cable (quad shielded)

fibre optic and other cable types for recognition/demonstration of the NTD network side

appropriate connectors for testing cables

appropriate adaptors and test leads for test instruments and cables to be tested

common basic tests utilised with each NTD/customer equipment system using:

structured cabling certifiers

structured cabling verification/qualification tester

wire mapper

oscillator and probe

multimeter

RF power measurement (RX)

time-domain reflectometer (TDR), available in SC certifiers

ADSL/VDSL test instruments

other appropriate test equipment

NTD or customer equipment user guides

purpose and use of test results

fault escalation enterprise processes.

# Assessment Conditions

Skills must be assessed in a workplace or simulated environment where conditions are typical of those in a telecommunications work environment or workplace.

Access is required to:

satellite modem/ethernet/telephony

fixed wireless modem/ethernet/telephony

fibre optic TX-RX/ethernet/telephony

HFC modem/ethernet/telephony

DSL modem/ethernet/telephony.

As a minimum, customer equipment may also require a portable computer, wi-fi modems, switch/router or devices that combine these parameters

Cable segments that use passive devices will also require access to RF splitters or DSL filters.

Assessors of this unit must satisfy the assessor requirements in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guides are available from VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a53af4e4-b400-484e-b778-71c9e9d6aff2