# ICTOPN504 Test and commission a dense wavelength division multiplexing transmission system

Release: 1

ICTOPN504 Test and commission a dense wavelength division multiplexing transmission system

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version first released with ICT Information and Communications Technology Training Package Version 2.0. |

# Application

This unit describes the skills and knowledge required to test and commission optical transmission systems using dense wavelength division multiplexing (DWDM) technology.

It applies to individuals who work with telecommunications carriers, service providers, public utilities and broadcasting companies. They combine technical optical communications skills with broader organisational and administrative skills in a range of commercial contexts and environments.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Unit Sector

Telecommunications – Optical Networks

# Elements and Performance Criteria

| Element | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare to commission DWDM transmission system | 1.1 Implement hazard risk control measures 1.2 Review system specifications and technical data by identifying system operating parameters 1.3 Obtain and check tools and test equipment needed for the work 1.4 Verify all necessary equipment is installed, and circuit cards and filler cards are correctly located and seated 1.5 Check interconnecting fibres to optical multiplexers and shelves are intact and correctly routed 1.6 Establish communication between computer and DWDM shelf 1.7 Upload latest software release to each DWDM shelf in the network |
| 2. Commission DWDM transmission system | 2.1 Connect an optical test set and a physical optical loopback to the system, and select protocol compatible with DWDM hardware 2.2 Check connection status and that test set data is passing correctly 2.3 Monitor quality of received signal by setting performance monitoring (PM) features and retrieve PM counts 2.4 Verify no errors are counted by the test set during a long term stability test while customer input signal is set to the worst case (minimum) power level 2.5 Verify no alarms are generated during long term stability test period 2.6 Dispose of antistatic packaging to minimise environmental impact |
| 3. Complete test and commissioning documentation | 3.1 Finalise documentation on commissioning test results 3.2 Present results to appropriate person according to established procedures |

# Foundation Skills

This section describes language, literacy, numeracy and employment skills incorporated in the performance criteria that are required for competent performance.

| Skill | Performance Criteria | Description |
| --- | --- | --- |
| Reading | 1.2, 3.2 | Interprets textual information from relevant sources to identify and plan for all job requirements and adhere to procedures and standards |
| Writing | 3.1, 3.2 | Accurately completes relevant reports and documentation using clear and technically specific language and numerical data |
| Oral Communication | 3.2, 3.2 | Uses listening and questioning skills to confirm understanding for requirements, participates in a verbal exchange of ideas and solutions, and uses appropriate, detailed and clear language to address key personnel and clients |
| Numeracy | 1.2, 2.3 | Uses mathematical equations to evaluate technical data and interpret results |
| Navigate the world of work | 2.6, 3.2 | Takes full responsibility for following policies, procedures and legislative requirements, and identifies organisational implications of new legislation or regulation |
| Get the work done | 1.1, 1.3-1.7, 2.1-2.6 | Accepts responsibility for planning and sequencing complex tasks and workload, negotiating key aspects with others taking into account capabilities, efficiencies and effectiveness Monitors progress of plans and schedules and reviews and changes them to meet new demands and priorities |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| ICTOPN504 Test and commission a dense wavelength division multiplexing transmission system | ICTOPN5121A Test and commission a dense wavelength division multiplexing transmission system | Updated to meet standards for Training Packages | Equivalent unit |

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a53af4e4-b400-484e-b778-71c9e9d6aff2