# LMTCL3002B Prepare and produce sewn garment

Revision Number: 1

LMTCL3002B Prepare and produce sewn garment

# Modification History

Not applicable.

# Unit Descriptor

| Unit descriptor | This unit covers the skills and knowledge to prepare, align and sew garments. |
| --- | --- |

# Application of the Unit

| Application of the Unit | The unit applies to sewing garments and may apply to sample machining involving all operations in the assembly of a garment to specification. Machines used may include plain lockstitch sewing machine, a three, four or five thread overlocker, a zigzag sewing machine, blind hemmers, buttonholers, button sewers, pocket and collaring machines, elasticators and binders. If cutting skills are required, then unit LMTCL3005B Lay up, mark and cut complicated fabrics and lays should be selected. Work may be self-directed and unsupervised. Work may be conducted in small to large scale enterprises and may involve individual and team activities. The application of this unit is according to OHS practices of the enterprise and workplace practices, which may include: requirements prescribed by legislation, awards, agreements and conditions of employment standard operating procedures work instructions oral, written and visual communication quality practices, including responsibility for maintenance of own work quality and contribution to quality improvement of team or section output housekeeping tasks related to environmental protection, waste disposal, pollution control, and recycling This unit requires the application of skills associated with planning and organising to prepare and produce sewn garments. Problem solving skills are required to ensure work conforms to specifications and identify any production problems. Communication skills are used to refer to and complete required documentation. Self management skills are used to ensure safe use of technology and conformance of own work to quality standards. |
| --- | --- |

# Licensing/Regulatory Information

Not applicable.

# Pre-Requisites

| Prerequisite | LMTCL2001B LMTCL2004B LMTCL2003B |
| --- | --- |

# Employability Skills Information

| Employability Skills | This unit contains employability skills. |
| --- | --- |

# Elements and Performance Criteria Pre-Content

Not applicable.

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| 1 Prepare for garment construction | 1.1 Garment requirements are identified from production specifications. 1.2 Machining requirements are determined. 1.3 Plan for garment construction is developed. |
| 2 Prepare work pieces | 2.1 Work bundle is received. 2.2 Work pieces are checked for readiness for assembly. 2.3 Work pieces are laid out in sequence according to construction plan. |
| 3 Prepare workstation | 3.1 Workbench and seating are set up according to OHS practices. 3.2 Machines are cleaned and checked. 3.3 Records are maintained. 3.4 Machines are set up and adjusted according to specifications for work. 3.5 Needles, attachments and parts are checked and worn needles and parts are identified and reported or replaced, according to manufacturer instructions. |
| 4 Identify poor machine performance | 4.1 Performance of machine is regularly checked for signs of faulty operation, including evidence from inspection of finished pieces and required action taken |
| 5 Sew garment | 5.1 Garment is sewn according to requirements for operations. 5.2 Garment is sewn according to product requirements and quality standards. 5.3 Garment is sewn according to requirements for OHS and speed of work. 5.4 Machine speed and work handling are controlled for type of operations, fabrics and product type according to workplace practices. |
| 6 Complete work | 6.1 Garment is inspected and checked against quality standards, any faults are identified and appropriate action taken. 6.2 Results of inspection of finished garment are recorded. 6.3 Action taken to either reject or correct faulty garment is recorded. 6.4 Preventative action taken to avoid any recurrence of defective garment is recorded. 6.5 Production records or packing slips are completed. 6.6 Completed work is directed to next operation or packing section. |

# Required Skills and Knowledge

This describes the essential skills and knowledge and their level, required for this unit.

Demonstrates knowledge of: characteristics of a variety of fabrics, threads and other materials used in garment assembly quality standards and practices sequence of operations OHS practices, including hazard identification and control measures workplace practices recording and reporting practices

Demonstrates skills to: plan product construction use and maintain a variety of production machines such as plain lockstitch sewing machine, a three, four or five thread overlocker, a zigzag sewing machine, blind hemmers, buttonholers, button sewers, pocket and collaring machines, elasticators and binders handle, receive and assemble garments conduct quality checks of own work identify non-compliances with quality standards identify hazards and control measures associated with assembly and sewing of garments consistently achieve quality and production output requirements read, interpret and follow information on work specifications, standard operating procedures and work instructions, and other reference material maintain accurate records communicate within the workplace sequence operations meet specifications clarify and check task-related information carry out work according to OHS practices

# Evidence Guide

| The Evidence Guide provides advice on assessment and must be read in conjunction with the Performance Criteria, Required Skills and Knowledge, the Range Statement and the Assessment Guidelines for the Training Package. |  |
| --- | --- |
| Critical aspects of evidence | Demonstrates skills and knowledge to: prepare for product construction check work pieces against pattern or specifications prepare workstation and work before commencing sewing operations apply range of sewing techniques required to construct whole garment operate machines required for complete operations apply OHS practices in work operations perform quality checks maintain accurate records |
| Consistency in performance | Consistently applies skills and knowledge when: organising work completing tasks according to instructions working systematically with attention to detail identifying improvements and avoiding damage using workplace practices using OHS practices recording and reporting accidents and incidents assessing operational readiness of equipment recognising and adapting to cultural differences in the workplace, including modes of behaviour and interactions |
| Resource implications | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Context for assessment | Assessment may occur on the job or in an appropriately simulated environment. |
| Interdependent assessment | This unit may be assessed independently or in combination with other relevant units. |

# Range Statement

| RANGE STATEMENT The Range Statement relates to the unit of competency as a whole. It allows for different work environments and situations that may affect performance. Bold italicised wording, if used in the Performance Criteria, is detailed below. Add any essential operating conditions that may be present with training and assessment depending on the work situation, needs if the candidate, accessibility of the item, and local industry and regional contexts. |  |
| --- | --- |
| Legislative/regulatory requirements | All work must comply with relevant Federal and State or Territory legislative or regulatory requirements. |
| Garment requirements may include: | work pieces, accessories and trims pattern production or delivery time requirements |
| Garment may include: | any item of clothing that includes multiple pieces such as sleeves, collars and trims, and which is made to production specifications (for example shirts, trousers, dresses, unstructured jackets, skirts) |
| OHS practices | OHS practices include hazard identification and control, risk assessment and implementation of risk reduction measures specific to the tasks described by this unit, and may relate to: manual handling techniques standard operating procedures personal protective equipment safe materials handling taking of rest breaks ergonomic arrangement of workplaces following marked walkways safe storage of equipment housekeeping reporting accidents and incidents environmental practices |
| Machines must include: | a full range of machines required to complete whole garment construction, which may include a combination of the following: plain lockstitch sewing machine, a three, four or five thread overlocker, a zigzag sewing machine blind hemmers, buttonholers, button sewers, pocket and collaring machines, elasticators and binders |
| Set-up may include: | correct thread, needle size and length needle guard, tension setting and attachments |
| Operations may include: | â€¢ sewing techniques where the positioning, feeding and handling of work pieces involves discretionary changes, contouring or critical stopping points or involving the special handling skills required to accommodate fabric variations production of whole garment from specifications or patterns sample machining all operations required to complete whole garment construction |

# Unit Sector(s)

| Sector | Clothing Production |
| --- | --- |