# MARA010 Manage loading, unloading and stowage of cargo

Release: 1

MARA010 Manage loading, unloading and stowage of cargo

# Modification History

Release 1. This is the first release of this unit of competency in the MAR Maritime Training Package Release 3.0.

# Application

This unit involves the skills and knowledge required to manage the loading, stowage, securing and care of cargo during the voyage and the unloading of cargo, according to the cargo plan, organisational procedures and vessel stowage limitations.

This unit applies to the work of a Watchkeeper Deck and Master up to 500 gross tonnage.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Not applicable.

# Competency Field

A – Handling Cargo and Vessel Stability

# Unit Sector

Not applicable.

# Elements and Performance Criteria

| ELEMENTS |  | PERFORMANCE CRITERIA |  |
| --- | --- | --- | --- |
| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| 1 | Review cargo plan | 1.1 | Cargo plan is interpreted to determine required cargo operations |
|  |  | 1.2 | Characteristics of cargo being transported are identified and the type, severity and impact of the hazard posed by cargo on safety, the environment, vessel operations and personnel are established |
|  |  | 1.3 | Cargo plan is evaluated to ensure regulations relating to hazardous materials/dangerous goods are observed |
|  |  | 1.4 | Cargo plan is assessed to ensure incompatible cargo stowage is avoided |
|  |  | 1.5 | Cargo plan is checked to ensure unloading sequence is effective |
| 2 | Prepare for loading | 2.1 | Holds are checked to ensure they are clean, dry and free of smell |
|  |  | 2.2 | Safety arrangements are verified to ensure they are operational |
|  |  | 2.3 | Supplies of cargo protection and securing material are reviewed to ensure there are sufficient available |
|  |  | 2.4 | Bilge suctions are protected before loading |
|  |  | 2.5 | Checks are made to ensure cargo is correctly identified, inspected and confirmed against documentation |
|  |  | 2.6 | Preparations for loading are monitored according to stowage plan and organisational procedures |
|  |  | 2.7 | Ballast discharge plan is identified and confirmed and appropriate actions to support this plan are undertaken |
| 3 | Supervise loading/ unloading of cargo | 3.1 | Instructions are given to crew and stevedores involved in cargo loading/unloading according to cargo stowage plan |
|  |  | 3.2 | Compliance with regulations, procedures and instructions pertaining to type of cargo being handled is managed during loading/unloading operations |
|  |  | 3.3 | Loading/unloading is monitored to ensure loading rate is not exceeded |
|  |  | 3.4 | Vessel stability is observed during loading/unloading operations |
|  |  | 3.5 | Loading/unloading operations are checked against cargo plan |
|  |  | 3.6 | Cargo is secured according to cargo plan |
|  |  | 3.7 | Cargo handling documentation is completed according to organisational procedures and regulatory requirements |
|  |  | 3.8 | Transfer of cargo is monitored to prevent hazards |
|  |  | 3.9 | Gas monitoring equipment is regularly inspected and used according to organisational procedures, as required |
| 4 | Monitor care of cargo during voyage | 4.1 | Plan for care of cargo during voyage is implemented according to organisational and customer requirements, and relevant regulations |
|  |  | 4.2 | Ventilation and humidity control systems are checked |
|  |  | 4.3 | Actions required to maintain the safety and security of cargo during the voyage are initiated according to customer requirements and organisational procedures |
|  |  | 4.4 | Compliance with safety and hazard minimisation procedures and regulations related to cargo care is managed at all times during the voyage to maintain the safety of personnel, cargo and vessel |
|  |  | 4.5 | Safety procedures are followed and appropriate action is taken when defects or damage to cargo are detected |
| 5 | Take precautions to prevent pollution of the environment | 5.1 | Procedures to prevent pollution are identified and observed at all times |
|  |  | 5.2 | Measures to prevent pollution during normal and emergency situations are applied according to regulatory requirements and organisational procedures |
|  |  | 5.3 | All relevant information is immediately reported to appropriate persons when a vapour leak or cloud is detected or a malfunction has occurred that poses a risk of a vapour leak or cloud |
|  |  | 5.4 | Shore-based response personnel are promptly notified when a vapour leak or cloud occurs |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| Range is restricted to essential operating conditions and any other variables essential to the work environment. |  |
| --- | --- |
| Cargo plan includes: | cargo weight correct description and stowage of hazardous and dangerous goods description of cargo to be loaded load/discharge port segregation of non compatible cargo stowage of refrigerated containers |
| Cargo includes one or more of the following: | bulk cargo chemicals and oils containerised cargo deck cargo heavy lift cargo liquefied gas liquid cargo material, equipment or machinery that may be safely handled and stowed on vessel refrigerated cargo |
| Hazardous materials/dangerous goods include: | any cargo described in the International Maritime Dangerous Goods (IMDG) Code as hazardous or dangerous |
| Incompatible cargo stowage includes one or more of the following: | cargo liable to taint dangerous and hazardous goods |
| Cargo handling documentation includes one or more of the following: | cargo gear register cargo receipts Cargo Securing Manual logbook entries Note of Protest Notice of Readiness to load or discharge operation orders and instructions safety data sheets (SDSs)/material safety data sheets (MSDSs) safety management system relating to cargo carriage ship/shore safety checklists |
| Actions required to maintain the wellbeing of cargo include one or more of the following: | maintaining adequate ventilation monitoring cargo spaces temperature control of refrigerated or cooled cargo |
| Defects or damage to cargo include one or more of the following: | damage caused by cargo movement deterioration of perishable cargo water ingress |

# Unit Mapping Information

No equivalent unit.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=772efb7b-4cce-47fe-9bbd-ee3b1d1eb4c2