# MARB058 Refuel a vessel

Release: 1

MARB058 Refuel a vessel

# Modification History

Release 1. This is the first release of this unit of competency in the MAR Maritime Training Package.

# Application

This unit involves the skills and knowledge required to apply safe work practices when refuelling vessels in a marina, including conducting pre-operational checks and completing post-refuelling operations and refuelling documentation. This unit applies to marina operators who refuel vessels in a marina and on water. This unit applies to individuals who provide assistance under supervision in the operational management of the marina. Licensing/Regulatory Information No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Not applicable.

# Competency Field

B – Equipment Checking and Maintenance

# Unit Sector

Not applicable.

# Elements and Performance Criteria

| ELEMENTS |  |  | PERFORMANCE CRITERIA |  |  |
| --- | --- | --- | --- | --- | --- |
| Elements describe the essential outcomes. |  |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |  |
| 1 | Plan vessel refuelling | 1.1 |  | Dangers of hazardous chemicals and vapours are established and recorded according to workplace policies, procedures and HAZCHEM requirements |
|  |  | 1.2 |  | Precautions for minimising risks associated with volatile liquids are identified and recorded according to workplace policies, procedures and HAZCHEM requirements |
|  |  | 1.3 |  | Safety and environmental requirements for refuelling vessel are identified and recorded according to workplace policies and procedures |
|  |  | 1.4 |  | Fuel type is selected according to the vessel operators’ instructions |
|  |  | 1.5 |  | Pump is identified and selected according to the fuel type and vessel operators’ instructions |
|  |  | 1.6 |  | Procedures for refuelling are identified and followed according to workplace policies, procedures, relevant regulations and standards |
|  |  | 1.7 |  | Emergency plan is accessed, reviewed and clarified with appropriate personnel |
| 2 | Prepare for refuelling | 2.1 |  | Personal protective equipment (PPE) is identified, accessed and used according to workplace policies and procedures |
|  |  | 2.2 |  | Firefighting and spill response equipment is located and checked, faulty and/or used equipment is replaced and faults are reported as required according to workplace policies, procedures and supervisor instructions |
|  |  | 2.3 |  | Refuelling equipment is checked and secured according to workplace policies, procedures and supervisor instructions |
|  |  | 2.4 |  | Vessel is positioned and secured for refuelling according to workplace policies, procedures and supervisor instructions |
|  |  | 2.5 |  | Tank valves are opened as required, according to workplace policies, procedures and supervisor instructions |
| 3 | Refuel vessel | 3.1 |  | Safety and environmental requirements for refuelling vessel are applied according to workplace policies, procedures and supervisor instructions |
|  |  | 3.2 |  | Vessel is refuelled according to workplace policies, procedures, supervisor instructions and relevant regulations and standards |
|  |  | 3.3 |  | Workplace policies, procedures and supervisor instructions are followed to contain spill and minimise environment or safety dangers as required |
|  |  | 3.4 |  | Communication is maintained with relevant personnel to ensure the safety and integrity of vessel and/or crew during vessel refuelling |
|  |  | 3.5 |  | Incidents which occur during vessel refuelling are responded to according to workplace policies, procedures and supervisor instructions |
| 4 | Complete refuelling | 4.1 |  | Refuelling equipment is cleaned and stowed according to workplace policies and procedures |
|  |  | 4.2 |  | Spillage response is carried out according to workplace procedures and regulatory requirements |
|  |  | 4.3 |  | Required documentation is completed according to workplace policies and procedures |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| Range is restricted to essential operating conditions and any other variables essential to the work environment. Non-essential conditions can be found in the MAR Maritime Training Package Companion Volume Implementation Guide. |  |  |
| --- | --- | --- |
| Spill containment includes: | absorbing materials containment booms and portable bunding |
| Fuels include one or more of the following: | diesel (distillate) liquefied petroleum gas (LPG) petrol (gasoline) including: premixed two stroke fuel |

# Unit Mapping Information

This unit replaces and is not equivalent to MARB022 Refuel a vessel

# Links

Companion Volume Implementation Guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=772efb7b-4cce-47fe-9bbd-ee3b1d1eb4c2