# MARL040 Apply advanced principles of marine mechanics

Release: 1

MARL040 Apply advanced principles of marine mechanics

# Modification History

Release 1. This is the first release of this unit of competency in the MAR Maritime Training Package.

# Application

This unit involves the skills and knowledge required to apply advanced principles of marine mechanics and to perform associated calculations needed to operate and maintain marine machinery. This unit applies to people working in the maritime industry in the capacity of: Engineer Class 1 (STCW Chief Engineer Unlimited) Engineer Class 2 (STCW Second Engineer Unlimited). Licensing/Regulatory Information Legislative and regulatory requirements are applicable to this unit. This unit is one of the requirements to obtain Australian Maritime Safety Authority (AMSA) certification as an Engineer Class 1 (STCW Chief Engineer Unlimited) or Engineer Class 2 (STCW Second Engineer Unlimited) and to meet regulatory requirements this unit must be delivered consistent with Marine Orders and with the relevant sections of the International Convention on Standards of Training, Certification and Watchkeeping for Seafarers (STCW). Those regulatory requirements include STCW International Maritime Organisation Organization (IMO) model course competencies and areas of knowledge, understanding and proficiency, together with the estimated total hours required for lectures and practical exercises. Teaching staff should note that timings are suggestions only and should be adapted to suit individual groups of trainees depending on their experience, ability, equipment and staff available for training.

# Pre-requisite Unit

Not applicable.

# Competency Field

L – Engineering

# Unit Sector

Not applicable.

# Elements and Performance Criteria

| ELEMENTS |  |  | PERFORMANCE CRITERIA |  |
| --- | --- | --- | --- | --- |
| Elements describe the essential outcomes. |  |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| 1 | Apply principle of statics to determine forces in structures, connections, support systems, and trusses in two and three dimensions | 1.1 |  | Bows notation is applied to solve problems related to trusses |
|  |  | 1.2 |  | Individual loads are computed using method of sections |
|  |  | 1.3 |  | Forces in three-dimensional structures are calculated |
|  |  | 1.4 |  | Principle of moments is applied to solve moments of any quantity |
|  |  | 1.5 |  | Resultant of a system of co-planer forces is calculated |
|  |  | 1.6 |  | Twisting moment due to engine crank mechanisms is calculated |
|  |  | 1.7 |  | Moments of areas and solids are calculated |
|  |  | 1.8 |  | Equilibrium of solids is explained |
| 2 | Calculate friction torque in plate and cone clutches | 2.1 |  | Laws of friction are applied to develop formulae, using uniform wear, to find the torque in a plate, centrifugal and cone clutch |
|  |  | 2.2 |  | Laws of friction are applied to develop formulae, using uniform pressure, to find the torque in plate and cone clutches |
|  |  | 2.3 |  | Power to overcome friction in plate and cone clutches using uniform wear and uniform pressure formulae is computed |
|  |  | 2.4 |  | Laws of friction are applied to solve problems involving friction in inclined planes, including angle of repose |
|  |  | 2.5 |  | Friction theory is applied to solve problems involving screw threads |
| 3 | Calculate displacement, velocity and acceleration in cams, engine mechanisms and gear systems | 3.1 |  | Output of epicyclic gears is calculated by applying relative velocity and acceleration theory |
|  |  | 3.2 |  | Problems of linear and angular motion involving uniform acceleration and deceleration are solved |
|  |  | 3.3 |  | Velocity and acceleration diagrams are applied to illustrate relative velocity and acceleration |
|  |  | 3.4 |  | Inertia loads are calculated using piston velocity and acceleration equations |
|  |  | 3.5 |  | Problems involving free falling bodies are solved |
| 4 | Analyse forces and couples to balance reciprocating machinery | 4.1 |  | How primary force balance is obtained is graphically illustrated |
|  |  | 4.2 |  | Relationship between complete balance and dynamic balance is explained |
|  |  | 4.3 |  | Reciprocating piston acceleration formula is applied to differentiate between primary and secondary forces |
|  |  | 4.4 |  | Complete balance for a multicylinder reciprocating engine or machine is illustrated graphically using vector diagrams and computed analytically |
|  |  | 4.5 |  | Relationship between momentum and impulse is explained |
|  |  | 4.6 |  | Conservation of energy theory is applied to problems involving collision of perfectly elastic bodies |
| 5 | Apply simple harmonic motion (SHM) principles to solve problems in free and forced vibration | 5.1 |  | Differences in the terms amplitude, frequency and period are explained |
|  |  | 5.2 |  | SHM equations are derived from the scotch yoke mechanism |
|  |  | 5.3 |  | Equations for displacement, velocity, acceleration and frequency in SHM are developed |
|  |  | 5.4 |  | Displacement, velocity, acceleration and frequency in SHM in a vibrating spring-mass system are determined |
|  |  | 5.5 |  | Spring constant (k) for springs in series and parallel is calculated |
|  |  | 5.6 |  | Forced vibration caused by an out-of-balance rotating mass is analysed to derive an expression for amplitude of forced vibration |
|  |  | 5.7 |  | Dangers of resonance are explained |
| 6 | Calculate stresses in components | 6.1 |  | How rotational stress is generated by centrifugal force is explained |
|  |  | 6.2 |  | Formula for hoop stress in a rotating ring is applied to calculate hoop stress and/or limiting speed of rotation |
|  |  | 6.3 |  | Stresses in compound bars subject to axial loads and/or temperature change are determined |
|  |  | 6.4 |  | Reduction in area and percentage elongation of tensile test specimens is calculated |
|  |  | 6.5 |  | Stresses in composite bodies of dissimilar dimensions and dissimilar materials are calculated |
|  |  | 6.6 |  | Problems involving thermal stress on components due to temperature change with free and restricted expansion are solved |
| 7 | Apply strain energy and resilience theory to determine stresses caused by impact or suddenly applied loads | 7.1 |  | Equation is derived to calculate strain energy in a deformed material |
|  |  | 7.2 |  | Stress in a material due to impact or dynamic loads is determined using energy equation |
|  |  | 7.3 |  | Equation to calculate stress caused by suddenly applied loads is derived |
| 8 | Apply beam theory to solve problems | 8.1 |  | Reactions of a loaded beam are calculated |
|  |  | 8.2 |  | Shear force and bending moment diagrams are constructed for simply supported and cantilever beams |
|  |  | 8.3 |  | Shear force and bending moment diagrams for beams with concentrated and uniformly distributed loads are calculated |
|  |  | 8.4 |  | Beam equation is applied to derive stresses in beams loaded with concentrated and uniformly distributed loads |
|  |  | 8.5 |  | Beam equation is applied to calculate bending stresses |
|  |  | 8.6 |  | Macaulay’s method is applied to calculate beam deflection |
|  |  | 8.7 |  | Deflection of cantilever and simply supported beams is calculated using standard deflection formulae for different loads |
| 9 | Apply Euler’s formula to find buckling load of a column | 9.1 |  | Effective length of a column with various end restraints is determined |
|  |  | 9.2 |  | Slenderness ratio is applied to determine the strength of columns |
|  |  | 9.3 |  | Relationship between slenderness ratio and buckling is explained |
|  |  | 9.4 |  | How buckling load for a slender column is applied, including a factor of safety |
| 10 | Calculate stresses | 10.1 |  | How to combine stress formula and calculate stress with combined loading is explained |
|  |  | 10.2 |  | Superposition is used to describe stress due to combined axial and bending stress |
|  |  | 10.3 |  | Mohr’s Circle is employed to illustrate normal and shear stress |
|  |  | 10.4 |  | Principal stress formulae are applied to explain how maximum combined normal and shear stress can be obtained |
| 11 | Apply thick and thin shell formulae | 11.1 |  | Tangential stress distribution caused by internal and external pressure is analysed |
|  |  | 11.2 |  | Lame’s theorem is applied to describe stress in thick cylinders due to internal and external pressure |
|  |  | 11.3 |  | Stress on thin-shelled pressure vessels due to internal pressure is calculated |
|  |  | 11.4 |  | Formula for calculating stress on thin-shelled pressure vessels to incorporate special conditions is modified |
| 12 | Apply continuity equation to determine changes in fluid velocity | 12.1 |  | Conservation of energy theory is applied to calculate pressure, head and velocity of fluids flowing through orifices |
|  |  | 12.2 |  | Volumetric and mass flow through a venturi meter is calculated |
|  |  | 12.3 |  | Forces exerted by flowing fluids either free (jet) or contained are determined, including coefficients of velocity, contraction of area and discharge |
| 13 | Determine changes in fluid flows through pipe systems and centrifugal pumps | 13.1 |  | Variation of fluid pressure with depth is calculated |
|  |  | 13.2 |  | Bernoulli’s Theorem is used to solve problems of velocity, pressure and head in pipes and ducted systems |
|  |  | 13.3 |  | Archimedes’ Principle is used to solve problems related to floating vessels using real and apparent weight |
|  |  | 13.4 |  | Difference between steady and unsteady flow is clarified |
|  |  | 13.5 |  | Viscosity of fluids is analysed and difference between dynamic and kinematic viscosity is explained |
|  |  | 13.6 |  | Significance of Reynolds number in fluid mechanics is explained |
|  |  | 13.7 |  | Importance of critical Reynolds number is explained |
|  |  | 13.8 |  | Flow losses in pipes and fittings are calculated |
|  |  | 13.9 |  | Changes of velocity of liquids in a centrifugal pump are analysed and entry and exit vane angles are determined |
| 14 | Apply torsion theory to calculate stress | 14.1 |  | Twisting moment due to engine crank mechanisms is calculated |
|  |  | 14.2 |  | Torsion equation is applied to solve problems involving solid and hollow shafts |
|  |  | 14.3 |  | Power transmitted in shafts and coupling bolts is calculated |
|  |  | 14.4 |  | Torsion equation is applied to calculate stress and deflection in a close-coiled helical spring |
|  |  | 14.5 |  | Power transmitted by shafts and couplings is calculated |
| 15 | Solve problems using principles of dynamics | 15.1 |  | Centripetal force is distinguished from centrifugal force |
|  |  | 15.2 |  | Relationship between centripetal and centrifugal force and mass, angular velocity and radius is clarified |
|  |  | 15.3 |  | Problems are solved involving centripetal and centrifugal forces |
|  |  | 15.4 |  | Centripetal acceleration is distinguished from centrifugal force |
|  |  | 15.5 |  | Out-of-balance forces on co-planer systems are calculated |
|  |  | 15.6 |  | Bearing reactions in rotating shafts are determined |
|  |  | 15.7 |  | Radius of gyration and moment of inertion when applied to rotating bodies is explained |
|  |  | 15.8 |  | Centrifugal forces in governors are calculated |
|  |  | 15.9 |  | Principles of dynamics are applied to solve problems involving rotating bodies, accelerating shafts, motors and flywheels |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

Range is restricted to essential operating conditions and any other variables essential to the work environment.

# Unit Mapping Information

This unit replaces and is not equivalent to MARL016 Apply intermediate principles of marine mechanics. This unit replaces and is equivalent to MARL020 Apply advanced principles of marine mechanics.

# Links

Companion Volume implementation guide can be found in VetNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=772efb7b-4cce-47fe-9bbd-ee3b1d1eb4c2

Assessment Requirements for MARL040 Apply advanced principles of marine mechanics

# Modification History

Release 1. This is the first release of this unit of competency in the MAR Maritime Training Package.

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria on at least one occasion and include: assessing own work outcomes and maintaining knowledge of current codes, standards, regulations and industry practices identifying and applying relevant mathematical formulas and techniques to solve advanced problems related to marine mechanics identifying and interpreting numerical and graphical information, performing complex mathematical calculations, such as determining hoop stresses in rotating rings and stresses in compound bars and solving problems related to fluids identifying, collating and processing information required to perform complex calculations related to marine mechanics imparting knowledge and ideas through verbal, written and visual means reading and interpreting written information needed to perform complex calculations in marine mechanics solving problems using appropriate laws and principles using calculators to perform accurate, reliable and complex mathematical calculations.

# Knowledge Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria and include knowledge of: advanced principles of marine mechanics angular and linear motion beam theory Bows notation centre of gravity (CG) centrifugal governors conservation of energy theorem machine dangers, including: catastrophic failure due to physical limitations of machines being exceeded as determined by their susceptibility and resistance to vibrations violent swaying motions different loads, including: combined concentrated distributed factor of safety fluids force and forces, including: balanced and unbalanced forces conditions for equilibrium definitions of matter, mass, weight, force, density and relative density moments of couples parallelogram and triangle of forces inertia force joint efficiency factor laws of motion mechanics and hydromechanics, including: balancing combined stress fluid mechanics simple harmonic motion stress and strain torsion momentum motion, including: action and reaction force, velocity and acceleration linear and angular motion Newton’s laws of motion nature and laws of friction physical and chemical properties of fuel and lubricants, including: shore side and shipboard sampling and testing interpretation of test results contaminants, including microbiological infection treatment of fuel and lubricants, including storage, centrifuging, blending, pre-treatment and handling polygon of forces pressure vessels principle of moments principles of dynamics reactions relationship between torque and power simple harmonic motion (SHM) stress and strain, including: direct stress and strain Hooke’s Law load extension graphs modulus of elasticity shear stress and strain technology of material, including: destructive and non-destructive testing of material engineering processes used in construction and repair thin cylinder theory turning moment vector diagrams work health and safety (WHS)/occupational health and safety (OHS) requirements and work practices.

# Assessment Conditions

Assessors must hold credentials specified within the Standards for Registered Training Organisations current at the time of assessment. Assessment must satisfy the Principles of Assessment and Rules of Evidence and all regulatory requirements included within the Standards for Registered Training Organisations current at the time of assessment. Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate. Practical assessment must occur in a workplace, or realistic simulated workplace, under the normal range of workplace conditions. Simulations and scenarios may be used where situations cannot be provided in the workplace or may occur only rarely, in particular for situations relating to emergency procedures and adverse weather conditions where assessment would be unsafe, impractical or may lead to environmental damage. Resources for assessment must include access to: relevant documentation, including workplace procedures, regulations, codes of practice and operation manuals tools, equipment, materials, and personal protective equipment (PPE) currently used in industry.

# Links

Companion Volume implementation guide can be found in VetNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=772efb7b-4cce-47fe-9bbd-ee3b1d1eb4c2