# MARM016 Survey hull and superstructure of a commercial vessel

Release: 1

MARM016 Survey hull and superstructure of a commercial vessel

# Modification History

Release 1. New unit of competency.

# Application

This unit involves the skills and knowledge required to undertake a survey of hull and superstructure of a commercial vessel according to relevant regulations. It includes planning survey, carrying out survey and providing survey report.

This unit applies to people working in the maritime industry as a domestic commercial vessel marine surveyor and may form part of accreditation requirements for surveyors under Australian legislation.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Not applicable.

# Competency Field

M – Marine Surveying

# Unit Sector

Not applicable.

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan and prepare for survey task | 1.1 | Relevant standards for vessel hull and superstructure are accurately identified and accessed to support survey task |
|  |  | 1.2 | Vessel survey regime is identified and relevant regulatory requirements, organisational requirements and procedures for survey scope are accessed and reviewed |
|  |  | 1.3 | Vessel survey regime is confirmed against regulatory and organisational requirements |
|  |  | 1.4 | Survey scope and depth is confirmed against relevant regulatory and organisational requirements |
|  |  | 1.5 | Survey purpose, objectives and variations are discussed with relevant personnel |
|  |  | 1.6 | Operational limits, certificate of operations and previous certificates of survey are reviewed to identify and take into consideration any special conditions, equivalent solutions, specific areas of operations and other regulatory limitations, exceptions or conditions that may impact on survey task |
|  |  | 1.7 | Survey equipment and tools to carry out survey are accurately identified, selected and checked prior to survey |
| 2 | Confirm hull type and material construction | 2.1 | Types of hull common to domestic commercial vessels are accurately identified and regulatory or additional standards are accessed and reviewed for use in survey |
|  |  | 2.2 | Common materials used in hull construction and superstructure are identified and regulatory or additional standards are accessed and reviewed for use in survey |
|  |  | 2.3 | Construction, type of hull and materials are confirmed prior to survey by accessing vessel records |
| 3 | Conduct periodic survey of hull and superstructure | 3.1 | Survey of hull and superstructure is carried out according to regulatory requirements |
|  |  | 3.2 | Changes to operational equipment or equivalent solutions are identified and examined for fitness both in or out of water as required by survey schedule |
|  |  | 3.3 | Watertight openings and skin fittings are inspected for compliance |
|  |  | 3.4 | Deformation and integrity of hull and superstructure are inspected for compliance |
|  |  | 3.5 | Paint and coatings are inspected for condition |
|  |  | 3.6 | Cathodic protection is inspected where applicable |
|  |  | 3.7 | Superstructure weather tightness is checked for integrity |
|  |  | 3.8 | Appendages are inspected for integrity |
| 4 | Report and act on non-compliance | 4.1 | Non-compliance is detected, recorded and reported according to regulatory and organisational requirements |
|  |  | 4.2 | Specialist support services are identified and sourced as appropriate |
|  |  | 4.3 | Risks arising from detected non-compliance are reported and communicated to relevant personnel |
|  |  | 4.4 | Relevant provisions of legislation appropriate to level of risk detected are identified and followed |
|  |  | 4.5 | Appropriate reports and documentation relating to survey are developed and managed according to organisational and regulatory requirements |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| Range is restricted to essential operating conditions and any other variables essential to the work environment. |  |
| --- | --- |
| Relevant standards must include: | Australian/New Zealand Standards (AS/NZS) class manufacture guidelines Marine Orders National Standard for the Administration of Marine Safety (NSAMS) section 4 National Standard for Commercial Vessels (NSCV) safety data sheets (SDS)/material safety data sheets (MSDS) safety of life at sea (SOLAS) Uniform Shipping Law (USL) work health and safety/occupational health and safety (WHS/OHS) |
| Survey regime must include: | class of vessel survey depth and level of vessel |
| Relevant regulatory requirements include one or more of the following: | AS/NZS, in particular: AS/NZS ISO 9001: 2008 Quality Management Systems – requirements Marine Safety (Domestic Commercial Vessel) National Law NSAMS Section 4 NSCV Part B – General Requirements Part C – Vessel Construction Part E – Operational Practices |
| Survey scope and depth includes one or more of the following: | condition initial modification/further building periodic survey (in or out of water) repair/damage |
| Survey equipment and tools include one or more of the following: | communication equipment draft survey hydrometer drill entry authority hammer/welder’s hammer meat piercing thermometer mirror personal protective equipment such as respirators, gloves, overalls, boots, hearing protection, goggles, masks photographs plastic sampling bags pocket calculator recording equipment: camera dictaphone lap top computer notebook sampling equipment: silver nitrate test kit for chlorides test kit equipment thermometers water-detecting paste scraper screwdriver small mallet sounding tapes storage equipment/facilities tape measure /measuring wheel |
| Types of hull include one or more of the following: | box catamaran foils non water displacement shallow draft single wave piercing |
| Domestic commercial vessels must include: | vessels defined as commercial vessels in Marine Safety (Domestic Commercial Vessel) National Law |
| Materials include one or more of the following: | aluminium cement composite fibreglass steel timber |
| Survey schedule include one or more of the following: | twelve months twenty-four months thirty-months five year cycle with NSAMS: periodic survey (either annual or bi-annual) change of class survey safety equipment only survey damage/repair or condition surveys equivalent solution or deemed-to-satisfy surveys SMS components in water out of water |
| Watertight openings and skin fittings include one or more of the following: | cooling water systems doppler log drain plugs hatch inlet valve sea chest stabilisers |
| Deformation and integrity include one or more of the following: | blisters chemical attack cracking degradation distortion dry rot isolated damage leaching out of trim structural failure ultra violet (UV) breakdown water ingress weathering |
| Paint and coatings include one or more of the following: | anti fouling copper sheathing gel based |
| Appendages include one or more of the following: | bilge keels keel coolers rudders stabilisers |
| Specialist support services include one or more of the following: | analytical laboratories chemical (sewage systems) electrical naval architects non-destructive evaluation (NDE) services noise pressure vessel testing |
| Appropriate reports and documentation include one or more of the following: | certificate of operation certificate of survey statements of compliance survey report |

# Unit Mapping Information

This unit replaces and is equivalent to MARM5006A Survey hull and superstructure of a commercial vessel

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=772efb7b-4cce-47fe-9bbd-ee3b1d1eb4c2