# MARM018 Undertake a periodic statutory survey

Release: 1

MARM018 Undertake a periodic statutory survey

# Modification History

Release 1. New unit of competency.

# Application

This unit involves the skills and knowledge required to undertake statutory periodic surveys of vessels.

This unit applies to people working in the maritime industry as a domestic commercial vessel marine surveyor and may form part of accreditation requirements for surveyors under Australian legislation.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Not applicable.

# Competency Field

M – Marine Surveying

# Unit Sector

Not applicable.

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Identify survey scope | 1.1 | Vessel survey regime is identified and relevant regulatory requirements, organisational requirements and procedures for survey scope are accessed and reviewed |
|  |  | 1.2 | Vessel survey regime is confirmed against relevant regulatory requirements |
|  |  | 1.3 | Survey scope and depth is confirmed against relevant regulatory and organisational requirements |
|  |  | 1.4 | Survey purpose objectives and variations are clarified with relevant personnel |
|  |  | 1.5 | Relevant regulatory and organisational requirements are reflected in survey plan |
| 2 | Plan and prepare for survey | 2.1 | Clients/representatives are informed of survey schedule according to relevant regulatory and organisational requirements |
|  |  | 2.2 | Survey time is estimated and location confirmed with relevant personnel |
|  |  | 2.3 | Vessel history and supporting documents are located and reviewed in preparation for survey |
|  |  | 2.4 | Operational limits, certificate of operations and previous certificates of survey are reviewed for special conditions or equivalent solutions and included in survey plan where required |
|  |  | 2.5 | Relevant standards for vessel equipment are identified, accessed and included in survey plan as required |
|  |  | 2.6 | Survey tools and equipment requirements for survey and range of variations are accurately identified, selected and tested for serviceability |
|  |  | 2.7 | Likelihood of confrontation or risks to self are identified and managed according to organisational procedures |
|  |  | 2.8 | Risks related to scope of survey are identified and managed according to organisational procedures |
| 3 | Conduct survey | 3.1 | Legal requirements for conducting periodic surveys are reviewed and applied |
|  |  | 3.2 | Purpose and scope of survey is confirmed with clients/representatives |
|  |  | 3.3 | Continued existence, quantity and/or type of components, systems or equipment on board vessel is verified, examined and/or tested according to survey schedule |
|  |  | 3.4 | Degradation or loss of functionality in components, systems or equipment is noted for repair, deficiency or renewal in survey report |
|  |  | 3.5 | Vessel construction, machinery or equipment not under survey but noted as unsafe are reported according to relevant regulatory, legislative and organisational requirements |
|  |  | 3.6 | Survey findings are documented in vessel history log in a systematic order and according to survey schedule and organisational requirements |
|  |  | 3.7 | Survey results are confirmed with clients/representatives, and improvements and actions required to issue certificate of survey for vessel are noted and issued to clients/representatives |
| 4 | Finalise survey | 4.1 | Survey outcomes are logged or reported to relevant personnel according to organisational requirements |
|  |  | 4.2 | Improvement actions required are reviewed for compliance before certificate of survey is issued |
|  |  | 4.3 | Results of final compliance requirements are forwarded to relevant personnel for certification |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| Range is restricted to essential operating conditions and any other variables essential to the work environment. |  |
| --- | --- |
| Survey regime includes one of the following: | class of vessel survey level of vessel |
| Relevant regulatory requirements include one or more of the following: | Australian/New Zealand Standards (AS/NZS), in particular: AS/NZS 3000: 2007 Electrical installations AS/NZS 3017: 2007 Electrical installations – verification guidelines AS/NZS ISO 9001: 2008 Quality Management Systems – requirements National Standard for the Administration of Marine Safety (NSAMS) Section 4 NSCV: Part B – General Requirements Part E – Operational Practices |
| Variations include one or more of the following: | combined Uniform Shipping Laws (USL) Code and NSCV equivalent solutions NSCV pre USL Code USL Code |
| Relevant personnel include one or more of the following: | classification societies colleagues government bodies owners or owner representatives of vessels and/or charters port authorities |
| Clients/representatives include one or more of the following: | classification societies environmental agencies/authorities government bodies lawyers owners of vessels and/or charterers port authorities |
| Survey schedule includes one or more of the following: | twelve months twenty-four months thirty-months five year cycle with NSAMS: periodic survey (either annual or bi-annual) change of class survey safety equipment only survey damage/repair or condition surveys equivalent solution or deemed-to-satisfy surveys safety management system (SMS) components in water out of water |
| Vessel history and supporting documents include one or more of the following: | case files/incident reports commercial documentation current survey practice, both formal and informal current vessel designs, practices and materials deck and engine logs forms (such as application forms, notification forms) insurance certificates notices (such as seizure notice, infringement notice) previous surveys and certificates of survey relevant national and international standards ship log books and other recordkeeping instruments vessel Stability Book |
| Operational limits include one or more of the following: | certificate of operations certificate of survey restrictions and endorsements SMS |
| Survey tools and equipment include one or more of the following: | communication equipment draft survey hydrometer drill entry authority hammer/welder’s hammer mallet mirror personal protective equipment such as respirators, gloves, overalls, boots, hearing protection, goggles, masks photographs plastic sampling bags pocket calculator recording equipment: camera dictaphone laptop computer notebook sampling equipment: silver nitrate test kit for chlorides test kit equipment thermometers water-detecting paste satellite imagery scraper screwdriver sounding tapes storage equipment/facilities tape measure/measuring wheel |
| Risks include one or more of the following: | age of vessel area of operation, nature of operation incident history of particular class of vessels operational and maintenance performance of operator personal attributes of operator/client physical attributes of vessel |
| Legal requirements include one or more of the following: | common law conflict of interest consequences and penalties for noncompliance duty of care ethical behaviour requirements under Marine Safety (Domestic Commercial Vessel) National Law |

# Unit Mapping Information

This unit replaces and is equivalent to MARM5008A Undertake a periodic statutory survey.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=772efb7b-4cce-47fe-9bbd-ee3b1d1eb4c2