# Assessment Requirements for MEA355 Maintain light aircraft air cycle air conditioning systems

Release: 1

Assessment Requirements for MEA355 Maintain light aircraft air cycle air conditioning systems

# Modification History

Release 1 - New unit of competency

# Performance Evidence

Evidence required to demonstrate competency in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria under the specified conditions of assessment, and include:

using hand skills, tools and test equipment in the testing, adjustment and troubleshooting of light aircraft air cycle air conditioning systems and components, including system component removal and installation

recognising air cycle air conditioning system and component defects/external damage, correct installation and security for the types of system components listed in the Range of Conditions

performing system functional tests and checks to isolate system faults and assess post-maintenance serviceability

effectively using maintenance documentation and relevant fault diagnosis guides in the troubleshooting process and for component removal and installation

applying standard procedures

observing all relevant WHS procedures including the use of PPE and MSDS.

The underlying skills inherent in this unit should be transferable across a range of inspection, testing and troubleshooting applications (including the timely involvement of supervisors or other trades) associated with light aircraft air cycle air conditioning systems and components. It is essential that relevant procedures, cleanliness requirements and safety precautions are fully observed, understood and complied with. Ability to interpret inspection procedures and specifications (allowable limits) and apply them in practice is critical.

This shall be demonstrated through application across air cycle air conditioning systems and components as listed in the Assessment Conditions.

# Knowledge Evidence

Evidence required to demonstrate competency in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria and include knowledge of:

WHS precautions relevant to light aircraft air cycle air conditioning system maintenance and how to obtain PPE and MSDS

standard trade practices relating to tool and test equipment usage and installation/securing of system components

air cycle air conditioning system:

terminology

layout

operation, including component operation and construction

electrical temperature control system components

how to configure the aircraft for inspection, testing and troubleshooting of air cycle air conditioning systems and components

air cycle air conditioning maintenance requirements and troubleshooting

component attachment methods

connection hardware and couplings

relevant maintenance manuals

relevant regulatory requirements and standard procedures

maintenance requirements and troubleshooting procedures.

# Assessment Conditions

Competency should be assessed in the work environment or simulated work environment using tools and equipment specified in maintenance documentation. It is also expected that applicable general-purpose tools and test equipment found in most routine situations would be used where appropriate.

The application of testing procedures should clearly indicate knowledge of system operation, the relationship of individual components and the links with other systems (if applicable) within the limits of the aircraft/system fault-finding guide before undertaking any action. The work plan should take account of applicable safety and quality requirements in accordance with the industry and regulatory standards.

The following conditions of assessment represent the requirements of the Regulators (ADF and CASA) and maintenance stakeholders and must be rigorously observed.

A person cannot be assessed as competent until it can be demonstrated to the satisfaction of the workplace assessor that the relevant elements and performance criteria of this unit of competency are being achieved under routine supervision on an air cycle air conditioning system and on a representative range of the following components:

valves and regulators

heat exchangers, water separators and humidifiers

expansion turbines

rigid and flexible pipelines, hoses and fittings

ducting

temperature sensors, temperature controllers, and electrical control circuit wiring/components.

This shall be established via the records in the Log of Industrial Experience and Achievement or, where appropriate, an equivalent Industry Evidence Guide (for details refer to the Companion Volume Assessment Guidelines).

Assessors must satisfy the requirements of the National Vocational Education and Training Regulator (Australian Skills Quality Authority, or its successors).

Where the unit is to be used for CASA licensing purposes the Assessor must also meet the criteria specified in the CASR Part 147 Manual of Standards.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=ce216c9c-04d5-4b3b-9bcf-4e81d0950371