# Assessment Requirements for MEA420 Fabricate basic structural components for aircraft

Release: 3

Assessment Requirements for MEA420 Fabricate basic structural components for aircraft

# Modification History

Release 3. Equivalent to MEA420 Fabricate basic structural components for aircraft with amended prerequisite codes.

# Performance Evidence

Evidence required to demonstrate competency in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria under the specified conditions of assessment, and must include:

applying relevant WHS procedures, including the use of MSDS and relevant items of PPE

using approved maintenance documentation and aircraft publications relating to aircraft structure

identifying various aircraft metals and their basic metallurgy properties by interpretation of markings, numbering systems or visual, chemical or mechanical means

handling and storing aircraft metals to industry standards

identifying aircraft structural assembly fasteners by interpretation of markings, numbering systems, size, shape and colour

fabricating aircraft structural components and parts by:

correctly interpreting drawings, including third angle projection, isometric, sectional formats and hand sketches

developing component flat pattern using basic drawing tools, geometric drawing processes, calculating and applying bend allowance/deduction/setback

using appropriate hand tools and machines for cutting and drilling

assembling component parts using appropriate hand and machine tools and standard aircraft fasteners to industry standards

applying appropriate metal heat treatment processes

applying appropriate metal surface treatments.

The underlying skills inherent in this unit should be transferable across a range of aircraft applications. It is essential that procedures take into account all safety precautions and quality requirements. Skill and knowledge application must include set back, bend allowance and other standard practices associated with forming of material.

Evidence of knowledge and skills associated with basic structural fabrication techniques and their application to different materials used in aircraft manufacture will be required to supplement evidence of ability to fabricate basic aircraft structural components.

# Knowledge Evidence

Evidence required to demonstrate competency in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria and include knowledge of:

aircraft construction principles

causes of structural fatigue and preventative measures

structural corrosion and preventative measures

aircraft structural drawings and repair scheme drawings

material specifications for aluminium alloys and steel alloys used in aircraft structure

structural material identification by markings and numbering systems

material identification by chemical, electrical and mechanical methods

material storage requirements

hardware types and specifications

identification of hardware

sealants used in aircraft structure

chemical surface treatments

electroplating

paints and finishes

flat pattern development and associated terminology

basic metal heat treatment metallurgy

WHS precautions associated with fabrication of aircraft structural components

MSDS

PPE.

# Assessment Conditions

Competency should be assessed in the work environment, or simulated work environment, using tools and equipment specified by aircraft maintenance manuals. It is expected that general-purpose tools and test equipment found in most routine situations would be used where appropriate.

The following conditions of assessment represent the requirements of the Regulators (ADF and CASA) and maintenance stakeholders and must be rigorously observed.

A person cannot be assessed as competent until it can be demonstrated to the satisfaction of the workplace assessor that the relevant elements and performance criteria of the unit of competency are being achieved under routine supervision on at least one structural fabrication task.

This shall be established via the records in the Log of Industrial Experience and Achievement or, where appropriate, an equivalent Industry Evidence Guide (for details refer to the Companion Volume Assessment Guidelines).

Assessors must satisfy the requirements of the National Vocational Education and Training Regulator (Australian Skills Quality Authority, or its successors).

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=ce216c9c-04d5-4b3b-9bcf-4e81d0950371