# MEA420 Fabricate basic structural components for aircraft

Release: 3

MEA420 Fabricate basic structural components for aircraft

# Modification History

Release 3. Equivalent to MEA420 Fabricate basic structural components for aircraft with amended prerequisite codes.

# Application

This unit of competency requires application of hand skills and the use of drawings, specifications and maintenance publications to fabricate basic structural components (that are flat or have only a single curve) requiring hand forming only from aluminium alloys and steel alloys at various states of temper during scheduled or unscheduled maintenance. Work may be performed individually or as part of a team.

Applications include components for fixed and rotary wing aircraft.

The unit is part of the Aeroskills Structures Maintenance Certificate IV training pathway.

The unit is used in workplaces that operate under the airworthiness regulatory systems of the Australian Defence Force (ADF) and the Civil Aviation Safety Authority (CASA).

# Pre-requisite Unit

| MEA107 | Interpret and use aviation maintenance industry manuals and specifications |
| --- | --- |
| MEA154 | Apply work health and safety practices in aviation maintenance |
| MEA155 | Plan and organise aviation maintenance work activities |
| MEA156 | Apply quality standards during aviation maintenance activities |
| MEA157 | Complete aviation maintenance industry documentation |
| MEA158 | Perform basic hand skills, standard trade practices and fundamentals in aviation maintenance |

# Competency Field

Aviation maintenance

# Unit Sector

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1. | Interpret specifications and organise materials | 1.1 | Specifications and drawings are used to determine material requirements |
|  |  | 1.2 | Equipment use is planned by determining the procedure for fabricating component |
|  |  | 1.3 | Material is correctly identified in accordance with specifications |
|  |  | 1.4 | All materials and equipment are organised |
| 2. | Prepare material and tooling | 2.1 | Dimensions to material are translated in accordance with specifications |
|  |  | 2.2 | Cutting and forming equipment are prepared and adjusted to ensure accuracy of fabrication |
|  |  | 2.3 | Material is cut according to specifications ensuring minimisation of wastage and maintenance of surplus material identification while observing all relevant work health and safety (WHS) requirements, including the use of material safety data sheets (MSDS) and items of personal protective equipment (PPE) |
|  |  | 2.4 | Material requiring special treatment is prepared for the appropriate processes |
|  |  | 2.5 | Solution treatment of materials is carried out in accordance with approved procedures and specifications |
| 3. | Hand form material | 3.1 | Hand forming is accurately carried out ensuring that specifications are met while observing all relevant WHS requirements, including the use of MSDS and items of PPE |
|  |  | 3.2 | Formed components are checked for irregularities and correction requirements determined |
|  |  | 3.3 | Irregularities are removed to meet required dimensions and specifications |
| 4. | Inspect components | 4.1 | Fabricated components are inspected to confirm dimensional accuracy and specifications are met |
|  |  | 4.2 | Checking fixtures are used, where appropriate, to ensure requirements are met |
|  |  | 4.3 | Components requiring special or further treatment are prepared for the appropriate processes |
|  |  | 4.4 | Completed components are tagged or identified, as required |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Components are to be fabricated: | Using materials comprising various types of sheet metal used in aircraft manufacture, including aluminium alloys and structural steel alloys across a range of temper |
| Machinery processes include: | Cutting, bending and drilling |
| Procedures and requirements include: | Industry standard procedures specified by manufacturers, regulatory authorities or the enterprise |

# Unit Mapping Information

Release 3. Equivalent to MEA420 Fabricate basic structural components for aircraft

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=ce216c9c-04d5-4b3b-9bcf-4e81d0950371

Assessment Requirements for MEA420 Fabricate basic structural components for aircraft

# Modification History

Release 3. Equivalent to MEA420 Fabricate basic structural components for aircraft with amended prerequisite codes.

# Performance Evidence

Evidence required to demonstrate competency in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria under the specified conditions of assessment, and must include:

applying relevant WHS procedures, including the use of MSDS and relevant items of PPE

using approved maintenance documentation and aircraft publications relating to aircraft structure

identifying various aircraft metals and their basic metallurgy properties by interpretation of markings, numbering systems or visual, chemical or mechanical means

handling and storing aircraft metals to industry standards

identifying aircraft structural assembly fasteners by interpretation of markings, numbering systems, size, shape and colour

fabricating aircraft structural components and parts by:

correctly interpreting drawings, including third angle projection, isometric, sectional formats and hand sketches

developing component flat pattern using basic drawing tools, geometric drawing processes, calculating and applying bend allowance/deduction/setback

using appropriate hand tools and machines for cutting and drilling

assembling component parts using appropriate hand and machine tools and standard aircraft fasteners to industry standards

applying appropriate metal heat treatment processes

applying appropriate metal surface treatments.

The underlying skills inherent in this unit should be transferable across a range of aircraft applications. It is essential that procedures take into account all safety precautions and quality requirements. Skill and knowledge application must include set back, bend allowance and other standard practices associated with forming of material.

Evidence of knowledge and skills associated with basic structural fabrication techniques and their application to different materials used in aircraft manufacture will be required to supplement evidence of ability to fabricate basic aircraft structural components.

# Knowledge Evidence

Evidence required to demonstrate competency in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria and include knowledge of:

aircraft construction principles

causes of structural fatigue and preventative measures

structural corrosion and preventative measures

aircraft structural drawings and repair scheme drawings

material specifications for aluminium alloys and steel alloys used in aircraft structure

structural material identification by markings and numbering systems

material identification by chemical, electrical and mechanical methods

material storage requirements

hardware types and specifications

identification of hardware

sealants used in aircraft structure

chemical surface treatments

electroplating

paints and finishes

flat pattern development and associated terminology

basic metal heat treatment metallurgy

WHS precautions associated with fabrication of aircraft structural components

MSDS

PPE.

# Assessment Conditions

Competency should be assessed in the work environment, or simulated work environment, using tools and equipment specified by aircraft maintenance manuals. It is expected that general-purpose tools and test equipment found in most routine situations would be used where appropriate.

The following conditions of assessment represent the requirements of the Regulators (ADF and CASA) and maintenance stakeholders and must be rigorously observed.

A person cannot be assessed as competent until it can be demonstrated to the satisfaction of the workplace assessor that the relevant elements and performance criteria of the unit of competency are being achieved under routine supervision on at least one structural fabrication task.

This shall be established via the records in the Log of Industrial Experience and Achievement or, where appropriate, an equivalent Industry Evidence Guide (for details refer to the Companion Volume Assessment Guidelines).

Assessors must satisfy the requirements of the National Vocational Education and Training Regulator (Australian Skills Quality Authority, or its successors).

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=ce216c9c-04d5-4b3b-9bcf-4e81d0950371