# Assessment Requirements for MEAARM0004 Inspect, test and troubleshoot aircraft stores management systems and components

Release: 1

Assessment Requirements for MEAARM0004 Inspect, test and troubleshoot aircraft stores management systems and components

# Modification History

Release 1. Application changed. Elements and Performance Criteria changed. Foundation Skills made explicit. Range of Conditions removed, and relevant information moved to Assessment Requirements. Assessment Requirements clarified. Supersedes and is equivalent to MEA604 Inspect, test and troubleshoot management systems and components.

# Performance Evidence

There must be evidence the candidate has completed all the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

inspect, test and troubleshoot aircraft stores management systems (SMS) and components under routine supervision on multiple different tasks involving the following SMS components:

mission computer

armament computer

encoder/decoder

control unit

head-up display (HUD)

multi-function display (MFD)

perform functional testing by applying logic processes, taking and interpreting measurements, using test equipment and appropriate documentation and manuals to isolate SMS malfunctions and assess post-maintenance serviceability

follow safety precautions relating to explosive ordnance handling and manual handling of SMS components while performing the above maintenance

apply safety requirements and appropriate legislative and regulatory requirements while performing the above maintenance

for the components being worked on:

verify the task to be completed by identifying each of:

maintenance restrictions

aircraft tail number

aircraft entered as unserviceable

nil stores fitted

check at least one of the following safety device types:

earth lead

aircraft safety pins

aircraft switches

check at least one of the following firing device types:

stores release cartridges

chaff and flares

gun system

missiles

loaded stores

prepare aircraft for testing and troubleshooting by:

open panels

connect ground support equipment and test equipment

obtain specialist advice from at least one of the following:

subject matter expert

manufacturer

systems program office.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

the aircraft operating and maintenance environment

aircraft SMS and interfaces with other aircraft systems

work health and safety (WHS) procedures and policies for aircraft SMS components

Aircraft SMS and component test procedures

explosive ordnance safety and handling procedures

technical publications and manuals, including:

aircraft publications

maintenance instruction manuals and process specifications

servicing or service bulletins or structural repair manuals

illustrated parts catalogues, aircraft wiring manuals or drawings

tooling, equipment or manufacturer’s manuals

standard practices

enterprise aviation regulations and publications

hardware and component attachment methods

technical documentation procedures, including:

EE500 maintenance form

Computer-Aided Maintenance Management (CAMM2) system documents

SMS documentation.

# Assessment Conditions

The following conditions of assessment represent the requirements of the regulator Defence Aviation Safety Authority (DASA) and maintenance stakeholders and must be rigorously observed.

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies encountered in inspecting, testing and troubleshooting aircraft SMS components. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

workplace procedures, manufacturing specifications, codes, standards, manuals, and reference materials relevant to inspecting, testing and troubleshooting aircraft SMS components

materials, tools and equipment specified in the maintenance manuals and applicable procedures

general and special-purpose tools and ground support equipment.

Evidence of tasks demonstrating competency must be recorded in a log of industrial experience and achievement. In addition, assessment methods may include questioning and observations in operational or simulated environments.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume Implementation Guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=ce216c9c-04d5-4b3b-9bcf-4e81d0950371