# Assessment Requirements for MEAARM0005 Inspect, test and troubleshoot aircraft stores suspension systems and components

Release: 1

Assessment Requirements for MEAARM0005 Inspect, test and troubleshoot aircraft stores suspension systems and components

# Modification History

Release 1. Application changed. Elements and Performance Criteria changed. Foundation Skills made explicit. Range of Conditions removed, and relevant information moved to Assessment Requirements. Assessment Requirements clarified. Supersedes and is equivalent to MEA605 Inspect, test and troubleshoot aircraft stores suspension systems and components.

# Performance Evidence

There must be evidence the candidate has completed all the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

inspect, test and troubleshoot aircraft stores suspension systems (SSS) on multiple different tasks involving the following SSS components:

bomb release unit

pylon

missile launcher

perform functional testing by applying system and component knowledge and logical fault-finding procedures and applicable maintenance data to troubleshoot SSS and component faults and assess post-maintenance serviceability

follow safety precautions relating to explosive ordnance handling and manual handling of SSS components while performing the above maintenance

apply safety requirements and appropriate legislative and regulatory requirements while performing the above maintenance

undertake the following for each component being worked on:

verify the reconfiguration task to be completed by identifying each of:

maintenance restrictions

aircraft tail number

aircraft entered as unserviceable

nil stores fitted

check at least one of the following safety device types:

earth lead

aircraft safety pins

aircraft switches

check at least one of the following firing device types:

stores release cartridges

chaff and flares

gun system

missiles

loaded stores

prepare aircraft for testing and troubleshooting by:

opening panels

connecting ground support equipment and test equipment

obtain specialist advice from at least one of the following:

subject matter expert

manufacturer

systems program office

commence task prior to initiating documentation.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

the aircraft operating and maintenance environment

aircraft SSS and component operation and interfaces with other aircraft systems

hardware and component attachment methods

work health and safety (WHS) procedures and policies

explosive ordnance safety and handling procedures

technical publications and manuals relevant to inspecting, testing and troubleshooting aircraft SSS and components, including:

aircraft publications

maintenance instruction manuals and process specifications

servicing or service bulletins or structural repair manuals

illustrated parts catalogues, aircraft wiring manuals or drawings

tooling, equipment or manufacturer’s manuals

standard practices

enterprise aviation regulations and publications

technical documentation procedures, including:

EE500 maintenance form

Computer-Aided Maintenance Management (CAMM2) system documents

SSS documentation.

# Assessment Conditions

The following conditions of assessment represent the requirements of the regulator Defence Aviation Safety Authority (DASA) and maintenance stakeholders and must be rigorously observed.

Skills must have been demonstrated in the workplace under routine supervision or in a simulated environment that reflects workplace conditions and contingencies encountered in inspecting, testing and troubleshooting aircraft SSS components.

The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

workplace procedures, manufacturing specifications, codes, standards, manuals, and reference materials relevant to inspecting, testing and troubleshooting aircraft SSS and components

materials, tools and equipment specified in the maintenance manuals and applicable procedures

general and special-purpose tools and ground support.

Evidence of tasks demonstrating competency must be recorded in a log of industrial experience and achievement. In addition, assessment methods may include questioning and observations in operational or simulated environments.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume Implementation Guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=ce216c9c-04d5-4b3b-9bcf-4e81d0950371