# MEAAVI0012 Inspect, test and troubleshoot advanced aircraft communications systems and components

Release: 1

MEAAVI0012 Inspect, test and troubleshoot advanced aircraft communications systems and components

# Modification History

Release 1. Application changed. Elements and Performance Criteria changed. Foundation Skills made explicit. Range of Conditions removed, and relevant information moved to Assessment Requirements. Assessment Requirements clarified. Supersedes and is equivalent to MEA215 Inspect, test and troubleshoot advanced aircraft communications systems and components.

# Application

This unit describes the skills and knowledge required to apply hand skills and use system and component knowledge, applicable maintenance publications, and test equipment to inspect, test and troubleshoot advanced communication systems and components of fixed and rotary wing aircraft during scheduled or unscheduled maintenance. Work may be performed individually or as part of a team.

The unit is used in workplaces that operate under the airworthiness regulatory systems of the Civil Aviation Safety Authority (CASA) and the Defence Aviation Safety Authority (DASA).

Any other relevant legislation, industry standards and codes of practice within Australia must be applied.

# Pre-requisite Unit

MEA206 Remove and install aircraft basic radio communication and navigation system components

MEA246 Manufacture and/or repair aircraft electrical hardware or parts

# Competency Field

Aviation maintenance

# Elements and Performance Criteria

| Elements Elements describe the essential outcomes. | Performance Criteria Performance criteria describe the performance needed to demonstrate achievement of the element. |
| --- | --- |
| 1. Inspect advanced communications systems and components | 1.1 Identify specific inspection requirements using maintenance documentation and modification status, including relevant system defect reports 1.2 Check isolation tags and configure aircraft for safe system inspection and operation in accordance with maintenance manual 1.3 Visually or physically check communications system components for external signs of defects in accordance with maintenance manual while observing all relevant work health and safety (WHS) requirements 1.4 Identify and report defects in accordance with standard enterprise procedures |
| 2. Test or adjust advanced communications systems and components | 2.1 Prepare aircraft and system for application of power or system or operation in accordance with maintenance manual 2.2 Perform functional testing of communications system for evidence of serviceability or malfunction in accordance with maintenance manual 2.3 Perform required calibration or adjustments to system in accordance with maintenance manual |
| 3. Troubleshoot advanced communications systems | 3.1 Use available information from maintenance documentation, inspection and test results to assist in fault determination of identified issues 3.2 Troubleshoot issues to line replacement level using maintenance manual fault diagnosis guides and logic processes 3.3 Obtain required specialist or supervisory advice to assist with the troubleshooting process 3.4 Locate communications system faults and identify and record causes of faults in required maintenance documentation in accordance with standard enterprise procedures 3.5 Determine requirements for rectification of faults |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

Reading skills to interpret maintenance manuals, inspection procedures and specifications (allowable limits).

Writing skills to record and report on defects and faults and their causes.

Numeracy skills to interpret and communicate technical information and take and interpret system measurements.

Other foundation skills essential to performance are explicit in the performance criteria of this unit.

# Unit Mapping Information

Release 1. Supersedes and is equivalent to MEA215 Inspect, test and troubleshoot advanced aircraft communications systems and components.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=ce216c9c-04d5-4b3b-9bcf-4e81d0950371

Assessment Requirements for MEAAVI0012 Inspect, test and troubleshoot advanced aircraft communications systems and components

# Modification History

Release 1. Application changed. Elements and Performance Criteria changed. Foundation Skills made explicit. Range of Conditions removed, and relevant information moved to Assessment Requirements. Assessment Requirements clarified. Supersedes and is equivalent to MEA215 Inspect, test and troubleshoot advanced aircraft communications systems and components.

# Performance Evidence

There must be evidence the candidate has completed all the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

perform inspection, testing and troubleshooting on advanced aircraft communications systems and components during scheduled or unscheduled maintenance on a system for at least one major system component or line replaceable unit (LRU) from each of the following:

ultra-high frequency (UHF) and satellite communications (SATCOM)

intercommunication and cockpit voice recorder (CVR) systems.

recognise system and component defects, external damage, correct/incorrect installation, attachment hardware (including cabling/harnesses/transmission lines) and security in each of the following:

external communications systems (UHF and SATCOM)

internal communications systems (intercommunication and CVR)

perform functional testing by applying logic processes, taking and interpreting system measurements to accurately and effectively isolate malfunctions and faults within the listed systems and assess post-maintenance serviceability

apply testing procedures, cleanliness requirements and safety precautions at all times, and as relevant to the systems being maintained.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

component attachment methods

the basic layout (block diagram level), function and operation of:

external communications systems (UHF and SATCOM)

internal communications systems (intercommunication and cockpit voice recorders)

maintenance requirements for the above systems and troubleshooting procedures

work health and safety (WHS) practices for advanced aircraft communications systems and components

basic principles/functions, relating to the above-listed systems and associated with:

electromagnetic radiation and propagation

basic alternating current (AC) and direct current (DC) circuit theory

printed circuit boards

digital fundamentals

analogue fundamentals

transmitter and receiver principles

antenna characteristics

transmission line characteristics

maintenance manuals for the inspection, testing and troubleshooting of advanced aircraft communications systems and components

relevant regulatory requirements and standard procedures.

# Assessment Conditions

The following conditions of assessment represent the requirements of the regulators (DASA and CASA) and maintenance stakeholders and must be rigorously observed.

Skills must have been demonstrated under routine supervision in the workplace or in a simulated environment that reflects workplace conditions and contingencies encountered in inspecting, testing and troubleshooting advanced aircraft communications systems and components. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

workplace procedures, manufacturing specifications, codes, standards, manuals, and reference materials relevant to inspecting, testing and troubleshooting advanced aircraft communications systems and components

tools and equipment specified in the maintenance manuals

general and special-purpose tools and items of ground support and test equipment.

Evidence of tasks demonstrating competency must be recorded in a log of industrial experience and achievement.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=ce216c9c-04d5-4b3b-9bcf-4e81d0950371