# Assessment Requirements for MEAAVI0044 Maintain basic light aircraft electrical systems and components

Release: 1

Assessment Requirements for MEAAVI0044 Maintain basic light aircraft electrical systems and components

# Modification History

Release 1. Application changed. Elements and Performance Criteria changed. Foundation Skills made explicit. Range of Conditions removed, and relevant information moved to Assessment Requirements. Assessment Requirements clarified. Supersedes and is equivalent to MEA274 Maintain basic light aircraft electrical systems and components.

# Performance Evidence

There must be evidence the candidate has completed all the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

maintain basic light aircraft electrical systems and components during scheduled or unscheduled maintenance activity to meet requirements on electrical looms, cables and connection hardware, and on each of the following systems and on at least one major component/line replaceable unit (LRU) in each case:

direct current (DC) power generation, regulation and distribution systems

piston engine and gas turbine engine ignition and starting systems (gas turbine may be omitted where not applicable to the enterprise)

DC electrical systems, such as flaps, including related motors and actuators

aircraft lighting

aircraft main battery (competency may be demonstrated through the performance of a battery check).

perform component removal and installation on at least one component from each of the following:

DC generators, and alternator or rectifier generators, and components of related single generator regulation and distribution systems

motors

actuators

piston engine and gas turbine engine ignition and starting system components (gas turbine may be omitted where not applicable to the enterprise)

aircraft batteries

aircraft lighting components such as bulbs, lenses, switches and rheostats

correctly connect DC generators, alternator or rectifier generators and starter/generators during the above maintenance activities

recognise system and component defects or external damage, correct installation, connection of plugs, terminations, attaching hardware (including cabling or harnesses) and security in each of:

DC power generation systems including regulation, distribution and control

battery installations

piston engine ignition and starting systems

gas turbine engine ignition and starting systems (where applicable to the enterprise)

internal/external lighting systems, including controls

motors and actuators in basic DC electrical systems

perform functional testing by applying logic processes, taking and interpreting electrical measurements, using test equipment and appropriate wiring diagrams and manuals to isolate malfunctions faults in the listed systems and assess post-maintenance serviceability

apply testing procedures, cleanliness requirements and safety precautions at all times, and as relevant to the system/s being maintained.

# Knowledge Evidence

component attachment methods

connection of hardware and plugs

DC electrical principles, including:

properties of permanent magnets

precautions for the care and storage of permanent magnets

properties of electromagnets

primary and secondary cells

aircraft battery types, construction, care and safety

resistor characteristics

fuses and circuit breakers

fundamental DC circuits

inductive circuits

capacitive circuits

basic fault-finding principles

general construction, operating characteristics and applications for aircraft, including:

DC generators

alternator/rectifier generators

DC motors including starter motors

starter/generators

DC actuators (linear and rotary)

gas turbine high energy ignition system components and related safety precautions

lighting systems

how to locate and identify components of:

DC power generation, regulation, distribution and control systems and components, including:

generators and starter/generators

regulators

bus bars

circuit breakers and fuses

wiring

piston engine ignition and starting systems and components, including

magnetos or coils

starter motors

ignition switches/start switches

ignition harnesses

low tension wiring

spark plugs

auxiliary starting devices

gas turbine engine ignition and starting systems (where applicable to the enterprise), including:

starter motors and starter/generators

high energy ignition units

control units

switches

batteries and associated mounting equipment, including related anti-vibration aids

motors and actuators in basic DC electrical systems

the basic layout (block diagram level), function and operation of the systems listed above

electrical system maintenance requirements and troubleshooting procedures

work health and safety (WHS) practices for basic light aircraft electrical systems and components, including those relating to gas turbine engine high energy ignition units

maintenance manuals for basic light aircraft electrical systems and components

relevant regulatory requirements and standard procedures.

# Assessment Conditions

The following conditions of assessment represent the requirements of the regulators (DASA and CASA) and maintenance stakeholders and must be rigorously observed.

Skills must have been demonstrated under routine supervision in the workplace or in a simulated environment that reflects workplace conditions and contingencies encountered in maintaining basic light aircraft electrical system components. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

workplace procedures, manufacturing specifications, codes, standards, manuals, and reference materials relevant to maintaining basic light aircraft electrical system components

tools and equipment specified in maintenance documentation

general-purpose tools and test equipment found in most routine situations and required for maintaining basic light aircraft systems and components.

Evidence of tasks demonstrating competency must be recorded in a log of industrial experience and achievement.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=ce216c9c-04d5-4b3b-9bcf-4e81d0950371