# Assessment Requirements for MEAAVI0045 Maintain basic light aircraft instrument systems and components

Release: 1

Assessment Requirements for MEAAVI0045 Maintain basic light aircraft instrument systems and components

# Modification History

Release 1. Application changed. Elements and Performance Criteria changed. Foundation Skills made explicit. Range of Conditions removed, and relevant information moved to Assessment Requirements. Assessment Requirements clarified. Supersedes and is equivalent to MEA275 Maintain basic light aircraft instrument systems and components.

# Performance Evidence

There must be evidence the candidate has completed all the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

maintain basic light aircraft instrument systems and components on a system and at least one major system component/line replaceable unit (LRU) from each of the following groups:

pitot/static systems and components, air speed indicator (ASI), vertical speed indicator (VSI) and counter-pointer altimeters

directional gyros (DGs) and artificial horizons (AHs) (air and electrically driven)

turn and bank and slip/turn coordinators

direct reading compasses

piston engine and gas turbine engine indication system components (direct reading measuring instruments and temperature indication)

electronic flight and engine instrument system components (may be omitted if not relevant to the enterprise)

electrical systems indication (voltage and current)

basic fuel quantity indication systems and components

vacuum indication components

locate and identify flight instrument system components during maintenance and comprising:

engine system temperature and speed, including mechanical and electrical tachometers

auxiliary direct reading systems, including vacuum, fuel storage quantities

flight systems, including attitude, altitude, air speed and outside air temperature (OAT)

locate and identified direct reading compasses during above maintenance

correctly handle and use observed maintenance precautions relating to gyroscopes, gimbals and pitot or static systems (connections, heating and protrusions) during the above maintenance

recognise system and component defects or external damage, correct installation, connection of plugs, terminations, attaching hardware, including cabling/harnesses, and security in each of the following:

flight instruments

pitot/static systems

direct reading compasses

piston engine and gas turbine engine indication systems

electrical systems indication

basic fuel quantity indication systems

vacuum indication systems

perform functional testing by applying logic processes, taking and interpreting system measurements and using test equipment and appropriate wiring diagrams and manuals to isolate malfunctions in the above systems and assess post-maintenance serviceability

apply testing procedures, cleanliness requirements and safety precautions at all times, and as relevant to the system/s being maintained

locate and identify at least one of the following:

electronic flight instrument system (EFIS)

engine indicating and crew alerting system (EICAS)

electronic central aircraft monitoring (ECAM)

data linkage and transmission systems

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

component attachment methods

connection of hardware and plugs

handling precautions for electrostatic sensitive devices

work health and safety (WHS) practices for basic light aircraft instrument systems and components

the basic layout (block diagram level), function and operation of:

pitot/static systems

piston engine and gas turbine engine direct reading indication systems

basic fuel quantity measurement systems

the operating principles of the above systems and associated with:

atmospheric conditions; properties and effects on aircraft instruments and systems

pressure and temperature sensing elements and their use in aircraft instruments

gyroscopes and their use in aircraft instrument systems

electrical fundamentals

instrument construction and operation, including:

instrument groupings, panel layout and construction

pitot static instruments (ASI, VSI and counter-pointer altimeters), their operation, calibration, safe handling and related terminology

pitot pressure

static pressure

altimeter Q code settings:

QNH

QNE

QFE

indicated airspeed (IAS)

true airspeed (TAS)

vacuum system indication component construction and operation

air and electrically powered artificial horizon construction and operation

DG construction and operation

construction and operation of direct reading engine instruments

turn and bank and slip/turn coordinator construction and operation

direct reading compass construction and compass calibration

piston and gas turbine engine direct reading measuring instruments and temperature indication instruments construction and operation

voltage and current measuring instrument construction and operation

volumetric fluid quantity system components, construction and operation

instrument system maintenance requirements and troubleshooting procedures, including pitot or static system leak testing

maintenance documentation and maintenance publications for basic light aircraft instrument systems and components

relevant regulatory requirements and standard procedures

for electronic flight and engine instrument systems, including:

the layout and operation (to block diagram level) of EFIS, EICAS and ECAM systems and related data linkage and transmission systems

interpretation of display data

component removal and installation procedures

testing of system operation using on-board testing procedures and/or simple external test equipment with a go or no-go outcome

software management control requirements.

# Assessment Conditions

The following conditions of assessment represent the requirements of the regulators (DASA and CASA) and maintenance stakeholders and must be rigorously observed.

Skills must have been demonstrated under routine supervision in the workplace or in a simulated environment that reflects workplace conditions and contingencies encountered in maintaining basic light aircraft instrument systems and components. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

workplace procedures, manufacturing specifications, codes, standards, manuals, and reference materials relevant to maintaining basic light aircraft instrument systems and components

tools and equipment specified in maintenance documentation

general-purpose tools and test equipment found in most routine situations and required for maintaining basic light aircraft instrument systems and components.

Evidence of tasks demonstrating competency must be recorded in a log of industrial experience and achievement.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=ce216c9c-04d5-4b3b-9bcf-4e81d0950371