# Assessment Requirements for MEAAVI0051 Maintain light aircraft AC-powered instrument systems and components

Release: 1

Assessment Requirements for MEAAVI0051 Maintain light aircraft AC-powered instrument systems and components

# Modification History

Release 1. Application changed. Elements and Performance Criteria changed. Foundation Skills made explicit. Range of Conditions removed, and relevant information moved to Assessment Requirements. Assessment Requirements clarified. Supersedes and is equivalent to MEA281 Maintain light aircraft AC powered instrument systems and components.

# Performance Evidence

There must be evidence the candidate has completed all the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

maintain light aircraft alternating current (AC) powered instrument systems and components during scheduled or unscheduled maintenance on a system and at least one major system component or line replaceable unit (LRU) from each of the following groups:

directional gyro (DG), artificial horizon (AH), altitude heading and reference system (AHRS) and components, servo and encoding altimeters, remote reading gyro compasses and components

piston engine and gas turbine engine indication system components

fuel quantity indication and flow indication systems and components

transmitter/indicator measuring instrument systems (pressure, temperature, vacuum) and components

inverters and transformer/rectifier units.

locate and identify AC-powered instrument system components for the systems being maintained including:

AH, DG, AHRS, servo and encoding altimeters

engine system temperature, speed, oil pressure, torque and manifold pressure

auxiliary systems, including vacuum, fuel storage quantities and fuel flow

remote reading gyro compasses

locate and identify AC instrument power supply system components for the systems being maintained, including:

inverters

transformer/rectifier units

voltage and frequency indication

correctly handle and observe maintenance precautions for gyroscopes and gimbals

recognise system and component defects, external damage, correct/incorrect installation, connection of plugs, terminations, attachment hardware (including cabling/harnesses) and security in AC-powered instruments and related power supply, including:

flight instruments

remote reading gyro compasses

piston engine and gas turbine engine indication systems (indicators and transmitters or sensors)

fuel quantity and flow indication systems (indicators, probes and transmitters)

transmitter/indicator measuring instrument systems (pressure, temperature and vacuum)

AC power supply, including inverters and transformer or rectifier units

perform functional testing by applying logic processes, taking and interpreting system measurements, using test equipment and appropriate wiring diagrams and manuals to isolate malfunctions in the above systems and assess post-maintenance serviceability

apply testing procedures, cleanliness requirements and safety precautions at all times, and as relevant to the system/s being maintained, including correct handling and observing maintenance precautions for gyroscopes and gimbals.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

component attachment methods

connection of hardware and plugs

handling precautions for electrostatic sensitive devices

work health and safety (WHS) practices for light aircraft AC powered instrument systems and components

AC theory, including:

calculations and measurement in AC circuits

inductors and inductive reactance

transformer operation

capacitors and capacitive reactance

LCR circuits and resonance

passive filters

power relationship in reactive circuits

the basic layout (block diagram level), function and operation of AC-powered:

flight instruments

remote reading gyro compasses

piston engine and gas turbine engine indication systems (indicators and transmitters or sensors)

fuel quantity and flow indication systems (indicators, probes and transmitters)

transmitter/indicator measuring instrument systems (pressure, temperature and vacuum)

AC power supply including AC generators, inverters and transformer/rectifier units

the operating principles of the above systems and associated with:

atmospheric conditions – properties and effects on aircraft instruments and systems

pressure and temperature sensing elements and their use in aircraft instrument systems

gyroscopes and their use in aircraft instrument systems

electrical fundamentals

relevant maintenance documentation and maintenance publications

maintenance requirements and troubleshooting procedures.

# Assessment Conditions

The following conditions of assessment represent the requirements of the regulators (DASA and CASA) and maintenance stakeholders and must be rigorously observed.

Skills must have been demonstrated under routine supervision in the workplace or in a simulated environment that reflects workplace conditions and contingencies encountered in maintaining light aircraft AC-powered instrument systems and components. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

workplace procedures, manufacturing specifications, codes, standards, manuals, and reference materials relevant to maintaining light aircraft AC-powered instrument systems and components.

tools and equipment specified in maintenance documentation

general-purpose tools and test equipment found in most routine situations.

Evidence of tasks demonstrating competency must be recorded in a log of industrial experience and achievement.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=ce216c9c-04d5-4b3b-9bcf-4e81d0950371