# Assessment Requirements for MEAMEC0058 Perform major modifications and repairs to small aircraft metal structure

Release: 1

Assessment Requirements for MEAMEC0058 Perform major modifications and repairs to small aircraft metal structure

# Modification History

Release 1. Application changed. Performance Criteria changed. Foundation Skills made explicit. Range of Conditions removed, and relevant information moved to Assessment Requirements. Assessment Requirements clarified. Supersedes and is equivalent to MEA371 Perform major repairs and modifications to small aircraft metal structure.

# Performance Evidence

There must be evidence the candidate has completed all the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

perform major modifications and repairs to small aircraft metal structure on at least one of the following tasks:

removing and installing structural hardware and fastening devices

repairing by replacing major load-carrying structural members (any of skins, longerons, spars, frames and bulkheads)

identify assembly stress defects during alignment or testing, including at least one of:

oil canning

buckling

contour misalignment

identify aircraft structural assembly fasteners by interpretation of markings, numbering systems, size, shape and colour to inform selection of modification or repair scheme

interpret and/or produce repair scheme or modification drawings, including third angle projection, isometric, sectional formats and hand sketches

use appropriate hand tools and machines to remove and assemble aircraft structural components, parts, sections and skin, including riveting equipment, drilling equipment, aligning tools and material fasteners (grip pins)

apply correct removal, installation and repair techniques for:

at least five rivets blind and solid, standard and oversize, including three different diameters

at least five close tolerance fasteners (standard and oversize – hillocks or taper locks), including hole preparation

at least two threaded devices, including internal and external thread cutting, and including Helicoil inserts and damaged stud replacement

hardware assembled by close tolerance fits using heat, cooling and force methods, including bearings, bushes and inserts on at least two occasions.

support the aircraft structure by jacking, trestling and jigging methods

perform mensuration checks after structure reassembly.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

aircraft construction principles and structural component attachment methods for primary structure:

fuselage

wings and attachment fittings

horizontal and vertical stabilisers and attachment fittings

pylons and attachment fittings

flight control surfaces

engine mounts and nacelles and attachment fittings

undercarriage attachments

procedures for replacement of primary structure components, including:

fuselage frames and longerons

wing spars or spar caps

wing-to-fuselage attachment fittings

tailplane attachment fittings

undercarriage attachment fittings

aircraft mensuration checks and methodology – when required

repair of integral fuel tanks and sealing of faying surfaces, including specific work health and safety (WHS) and personal protective equipment (PPE) requirements

surface finishes and methods of restoration

how to obtain material safety data sheets (MSDSs)

structural flight loads and aerodynamic requirements

relevant maintenance and structural repair manuals and modification data

relevant regulatory requirements and standard procedures.

# Assessment Conditions

The following conditions of assessment represent the requirements of the regulators Defence Aviation Safety Authority (DASA) and Civil Aviation Safety Authority (CASA) and maintenance stakeholders and must be rigorously observed.

Skills must have been demonstrated under routine supervision in the workplace or in a simulated environment that reflects workplace conditions and contingencies encountered in performing major modifications and repairs to small aircraft metal structure. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

maintenance documentation relevant to performing major modifications and repairs to small aircraft metal structure

tools and equipment specified in maintenance documentation

general-purpose tools and test equipment found in most routine situations.

Evidence of tasks demonstrating competency must be recorded in a log of industrial experience and achievement.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=ce216c9c-04d5-4b3b-9bcf-4e81d0950371