# Assessment Requirements for MEASTR0017 Perform major disassembly and reassembly on aircraft structure

Release: 1

Assessment Requirements for MEASTR0017 Perform major disassembly and reassembly on aircraft structure

# Modification History

Release 1. Application changed. Elements and Performance Criteria changed. Foundation Skills made explicit. Range of Conditions removed, and relevant information moved to Assessment Requirements. Assessment Requirements clarified. Supersedes and is equivalent to MEA423 Aircraft structure major disassembly and reassembly.

# Performance Evidence

There must be evidence the candidate has completed all the tasks outlined in the elements and performance criteria of this unit and demonstrated the ability to:

perform major disassembly and reassembly on aircraft structures during scheduled or unscheduled maintenance on elements from each of the following groups:

assembly of aircraft sub-assemblies or end items from detailed parts using jigs and fixtures

disassembly and reassembly of aircraft structure, such as wings, tailplanes or fuselage sections, using trestling, jigs and fixtures

replacement of major structural load carrying members, for example, skins, longerons, spars, frames and bulkheads.

safely handle heavy components during removal and assembly of structures being worked on

identify various aircraft metals/composite materials and their basic metallurgy properties by interpretation of markings, numbering systems or visual, chemical or mechanical means to inform requirements

handle and store aircraft metal and composite components to industry standards during and after assembly work

identify aircraft structural assembly fasteners (metal and composite) by interpretation of markings, numbering systems, size, shape and colour to inform suitable selections for structures being worked on

use appropriate hand tools and machines to remove and assemble aircraft structural components, parts, sections and skin, including riveting equipment, drilling equipment, aligning tools, reamers and material fasteners (grip pins)

apply correct removal and installation techniques for general and close tolerance fasteners (rivets, standard and oversize hilocks), including hole preparation and location techniques.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

aircraft construction principles, including the causes of structural fatigue and corrosion

describing the basic construction methods used to assemble:

fuselage (pressure and non-pressurised)

wings, vertical and horizontal stabilisers, rotary wing tail cones and pylons

engine nacelles/pylons

doors and windows, including seals, sealants and locking mechanisms

assembly procedures, including:

drilling, reaming and countersinking

fitting fasteners and rivets

applying adhesives and sealants

fitting hardware

applying corrosion inhibitors and protective coatings

assembly stress defects, including:

oil canning

buckling

contour misalignment

stress raisers

material specifications for aluminium alloys and steel alloys used in aircraft structure

composite cloths, matrix materials and adhesives

structural material identification by markings and numbering systems

material identification by chemical, electrical and mechanical methods

material storage requirements

hardware types and specifications

composite bonding methods

identification of hardware

sealants used in aircraft structure and their application and handling

chemical surface treatments

electroplating

paints and finishes

work health and safety (WHS) precautions for disassembly and reassembly of aircraft structures, including safe handling of heavy components

how to access and apply material safety data sheets (MSDSs)

personal protective equipment (PPE) for disassembly and reassembly of aircraft structures.

# Assessment Conditions

The following conditions of assessment represent the requirements of the regulators Civil Aviation Safety Authority (CASA) and the Defence Aviation Safety Authority (DASA) and maintenance stakeholders and must be rigorously observed.

Skills must have been demonstrated under routine supervision in the workplace or in a simulated environment that reflects workplace conditions and contingencies encountered in disassembling and reassembling aircraft structures. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

workplace procedures, manufacturing specifications, codes, standards, manuals, and reference materials relevant to disassembling and reassembling aircraft structures

tools and equipment specified in the aircraft maintenance manuals

general-purpose tools and ground support equipment found in most routine situations.

Evidence of tasks demonstrating competency must be recorded in a log of industrial experience and achievement.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=ce216c9c-04d5-4b3b-9bcf-4e81d0950371