# MEA217C Inspect, test and troubleshoot fixed wing autopilot systems and components

Release: 2

MEA217C Inspect, test and troubleshoot fixed wing autopilot systems and components

# Modification History

Minor formatting and editorial changes made.

# Unit Descriptor

This unit of competency is part of the Avionic Certificate IV (Aircraft Maintenance Stream) training pathway. It covers the competencies required to inspect, test and troubleshoot multi-axis autopilot systems and components of fixed wing aircraft. The unit is used in workplaces that operate under the airworthiness regulatory systems of the ADF and CASA. Where a CASA licensing outcome is sought this unit forms part of the CASA requirement for the granting of the chosen Aircraft Maintenance Engineer Licence under CASR Part 66, in accordance with the licensing provisions in Section 3, Assessment Guidelines.

# Application of the Unit

This unit requires application of hand skills and the use of system/component knowledge and applicable maintenance publications and test equipment to inspect, test and troubleshoot fixed wing multi-axis autopilot systems and components. Applications include fixed wing aircraft that have multi-axis autopilot systems, including those incorporating a radio-coupled flight director.

# Licensing/Regulatory Information

Not applicable.

# Pre-Requisites

| MEA207C | Remove and install aircraft electronic system components |
| --- | --- |
| MEA246C | Fabricate and/or repair aircraft electrical components or parts |

# Employability Skills Information

This unit contains employability skills.

# Elements and Performance Criteria Pre-Content

| Elements describe the essential outcomes of a unit of competency. | Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold italicised text is used, further information is detailed in the required skills and knowledge section and the range statement. Assessment of performance is to be consistent with the evidence guide. |
| --- | --- |

# Elements and Performance Criteria

| 1. Inspect fixed wing autopilot systems and components | 1.1. Relevant maintenance documentation and modification status, including system defect reports, where relevant, are used to identify specific inspection requirements 1.2. Isolation tags are checked and aircraft configured for safe system inspection and operation in accordance with the applicable maintenance manual 1.3. Autopilot systems and components are visually or physically checked for external signs of defects in accordance with applicable maintenance manual 1.4. Defects are correctly identified and reported |
| --- | --- |
| 2. Test/adjust fixed wing autopilot systems and components | 2.1. Aircraft and system are prepared in accordance with applicable maintenance manual for the application of power/system operation 2.2. Autopilot systems are functionally tested, in accordance with maintenance manual, for evidence of serviceability or malfunction 2.3. System calibration or adjustments are performed in accordance with maintenance manual, as appropriate |
| 3. Troubleshoot fixed wing autopilot systems | 3.1. Available information from maintenance documentation, inspection and test results is used, where necessary, to assist in fault determination 3.2. Maintenance manual fault diagnosis guides and logic processes are used to ensure efficient and accurate troubleshooting 3.3. Specialist advice is obtained, where required, to assist with the troubleshooting process 3.4. Autopilot system faults are located and the causes of the faults are clearly identified and correctly recorded in maintenance documentation, where required, in accordance with standard enterprise procedures 3.5. Rectification requirements are determined |

# Required Skills and Knowledge

Required skills

Look for evidence that confirms skills in: applying relevant OHS practices using approved maintenance documentation and aircraft publications relating to the autopilot and flight director system being maintained recognition of system and component defects/external damage, correct installation, connection of plugs, terminations, attaching hardware (including cabling/harnesses) and security in: flight director components and interface multi-axis autopilot system components applying logic processes, taking and interpreting system measurements, use test equipment and appropriate wiring diagrams and manuals, to accurately and effectively isolate malfunctions in the above systems testing listed systems to isolate system malfunctions and assess post-maintenance serviceability

Required knowledge

Look for evidence that confirms knowledge of: component attachment methods the basic layout (block diagram level), function and operation of: flight director components and interface multi-axis autopilot system components integration with other avionic systems, e.g. flight management systems and navigation systems flight director and autopilot system maintenance requirements and troubleshooting procedures relevant OHS practices basic principles/functions, relating to the listed systems and associated with: basic AC and DC circuit theory digital fundamentals analogue fundamentals fixed wing flight theory inner and outer loop control flight control modes/channels relevant maintenance manuals relevant regulatory requirements and standard procedures

# Evidence Guide

| The Evidence Guide provides advice on assessment and must be read in conjunction with the performance criteria, required skills and knowledge, range statement and the Assessment Guidelines for the Training Package. |  |
| --- | --- |
| Overview of assessment | A person who demonstrates competency in this unit must be able to inspect, test and troubleshoot fixed wing multi-axis autopilot systems and components while observing all relevant safety precautions. |
| Critical aspects for assessment and evidence required to demonstrate competency in this unit | The underlying skills inherent in this unit should be transferable across a range of inspection, testing and troubleshooting applications (including the timely involvement of supervisors or other trades) associated with fixed wing aircraft autopilot systems and components. It is essential that system testing procedures, cleanliness requirements and safety precautions applicable to the system being maintained are fully observed, understood and complied with. Ability to interpret inspection procedures and specifications (allowable limits) and apply them in practice is critical. Evidence of transferability of skills and knowledge related to inspection, testing and troubleshooting is essential. This is to be demonstrated through application across an autopilot system and its components. The application of testing procedures should clearly indicate knowledge of system operation, the relationship of individual components and the links with other systems (if applicable) within the limits of the aircraft/system fault finding guide before undertaking any action. The work plan should take account of applicable safety and quality requirements in accordance with the industry and regulatory standards. A person cannot be assessed as competent until it can be demonstrated to the satisfaction of the workplace assessor that the relevant elements of the unit of competency are being achieved under routine supervision on an autopilot system and its components. This shall be established via the records in the Log of Industrial Experience and Achievement or, where appropriate, an equivalent Industry Evidence Guide. |
| Context of and specific resources for assessment | Competency should be assessed in the workplace or simulated workplace using tools and equipment specified in the maintenance manuals. It is also expected that general and special purpose tools, test and ground support equipment would be used where appropriate. |
| Method of assessment |  |
| Guidance information for assessment |  |

# Range Statement

| The range statement relates to the unit of competency as a whole. It allows for different work environments and situations that may affect performance. Bold italicised wording, if used in the performance criteria, is detailed below. Essential operating conditions that may be present with training and assessment (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) may also be included. |  |
| --- | --- |
| Autopilot systems and components | Autopilot systems and components may include: three axis autopilot systems and components, including those incorporating a radio-coupled flight director |
| Troubleshooting | Troubleshooting involves the use of fault finding charts or similar, to line replacement level |
| Application | Application of this unit may relate to: scheduled or unscheduled maintenance activities individual or team-related activities |
| Procedures and requirements | Refer to industry standard procedures specified by manufacturers, regulatory authorities or the enterprise |

# Unit Sector(s)

Aviation maintenance

# Competency field

# Co-requisite units

Not applicable