# MEM04007 Pour molten metal

Release: 1

MEM04007 Pour molten metal

# Modification History

Release 1. Supersedes and is equivalent to MEM04007B Pour molten metal

# Application

This unit of competency defines the skills and knowledge required to manually pour molten metal as part of metal casting and moulding processes.

Where the selection and use of engineering measuring equipment is required unit MEM12023 Perform engineering measurements should also be selected.

Where the selection and use of tools is required unit MEM18001 Use hand tools and unit MEM18002 Use power tools/hand held operations, should also be selected as appropriate.

Where lifting and moving of ladles requires the use of overhead cranes, applicable units should also be selected.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

Band:	A

Unit Weight:	4

# Pre-requisite Unit

| MEM11011 | Undertake manual handling |
| --- | --- |
| MEM13004 | Work safely with molten metals/glass |
| MEM13015 | Work safely and effectively in manufacturing and engineering |
| MEM16006 | Organise and communicate information |

# Competency Field

Casting and moulding

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Determine job requirements | 1.1 | Follow standard operating procedures (SOPs) |
|  |  | 1.2 | Comply with work health and safety (WHS) requirements at all times |
|  |  | 1.3 | Use appropriate personal protective equipment (PPE) in accordance with SOPs |
|  |  | 1.4 | Identify job requirements from specifications, job sheets or work instructions |
| 2 | Prepare for pouring molten metal | 2.1 | Select equipment as required |
|  |  | 2.2 | Check the condition of the mould and ladle according to procedures |
|  |  | 2.3 | Check the temperature of molten metal for conformance to specification |
|  |  | 2.4 | Sequence pouring method to procedures |
|  |  | 2.5 | Identify the capacity of the required pour against specification |
|  |  | 2.6 | Preheat and prepare the ladle to receive molten metal |
| 3 | Transfer ladle to furnace | 3.1 | Check safety clips in accordance with safety procedures |
|  |  | 3.2 | Fill and transfer the ladle to the pouring area |
|  |  | 3.3 | Determine additives from specification and add to molten metal as required |
| 4 | Maintain quality of metal as required | 4.1 | Remove slag and/or dross where necessary |
|  |  | 4.2 | Monitor the temperature as required |
|  |  | 4.3 | Undertake chemical analysis and apply remedial action as required |
| 5 | Pour molten metal | 5.1 | Inform personnel in the immediate area of the metal pour that pour is to take place |
|  |  | 5.2 | Pour metal safely to specification |
|  |  | 5.3 | Pour metal at an appropriate and continuous rate |
|  |  | 5.4 | Pour a test bar as required |
| 6 | Empty excess metal from ladle and return ladle | 6.1 | Pour and tag pigs |
|  |  | 6.2 | Empty, clean and maintain the ladle according to SOPs |

# Foundation Skills

This section describes those required skills (reading, writing, oral communication and numeracy) that are essential to workplace performance in this unit of competency.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Ladle includes one (1) or more of the following: | lip pour tea pot bottom pour barrel bull ladles of varying capacity |
| Additives include one (1) or more of the following: | alloys inoculants spheroidisers coagulants |

# Unit Mapping Information

Release 1. Supersedes and is equivalent to MEM04007B Pour molten metal

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2

Assessment Requirements for MEM04007 Pour molten metal

# Modification History

Release 1. Supersedes and is equivalent to MEM04007B Pour molten metal

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy the requirements of the elements and performance criteria on at least two (2) occasions and include:

following work instructions, standard operating procedures (SOPs) and safe work practices

identifying and interpreting specifications, SOPs and standard test data sheets

preparing manual pour of molten metal, including selecting equipment as required, and checking condition of mould and appropriate ladle

checking temperature of molten metal for conformance against specification, sequencing pouring method and identifying the capacity of the required pour

transferring ladle to furnace after preheating/preparing to receive molten metal

filling and transferring the ladle to the pouring area in a safe and effective manner and adding additives as required to molten metal

maintaining quality of metal by removing slag/dross, monitoring temperature and undertaking chemical analysis and applying any remedial action

pouring molten metal safely at an appropriate and continuous rate after informing personnel in immediate area that a pour is to take place to required specifications, pouring a test bar as required

emptying excess metal and cleaning and maintaining ladle.

# Knowledge Evidence

Evidence required to demonstrate the required knowledge for this unit must be relevant to and satisfy the requirements of the elements and performance criteria and include knowledge of:

safe work practices and procedures and use of personal protective equipment (PPE)

types and pouring characteristics of metals

types and characteristics of ladles

procedures for maintaining condition and integrity of ladle

procedures for safe handling and transference of molten metal

metal treatments, applications and procedures for making additions to molten metal

slag and dross removing procedures

techniques for sampling and testing molten metal

pouring procedures

metal identification and tagging procedures.

# Assessment Conditions

Assessors must:

have vocational competency in pouring molten metal at least to the level being assessed with relevant industry knowledge and experience

satisfy the assessor requirements in the Standards for Registered Training Organisations 2015 or its replacement and comply with the National Vocational Education and Training Regulator Act 2011, its replacement or equivalent legislation covering VET regulation in a non-referring state/territory as the case requires

Where possible assessment must occur in operational workplace situations. Where this is not possible or where personal safety or environmental damage are limiting factors, assessment must occur in a sufficiently rigorous simulated environment that reflects realistic operational workplace conditions. This must cover all aspects of workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills

Conditions for assessment must include access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2