# MEM04011 Produce polymer patterns

Release: 2

MEM04011 Produce polymer patterns

# Modification History

Release 2. Quantum of hours of workplace practice removed. Supersedes and is equivalent to MEM04011 Produce polymer patterns (Release 1).

Release 1. Supersedes and is equivalent to MEM04011B Produce polymer patterns.

# Application

This unit of competency defines the skills and knowledge required to design, manufacture and finish polymer patterns.

It applies to the manufacture of investment casting dies and polymer patterns for medium production castings, core boxes, tooling aids and jigs and checking of fixtures.

Construction includes solid polymer, filled, laminated fibreglass or metal inserts.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

Band:	A

Unit Weight:	8

# Pre-requisite Unit

| MEM07005 | Perform general machining |
| --- | --- |
| MEM09002 | Interpret technical drawing |
| MEM11011 | Undertake manual handling |
| MEM12023 | Perform engineering measurements |
| MEM12024 | Perform computations |
| MEM13003 | Work safely with industrial chemicals and materials |
| MEM13004 | Work safely with molten metals/glass |
| MEM13015 | Work safely and effectively in manufacturing and engineering |
| MEM14006 | Plan work activities |
| MEM16006 | Organise and communicate information |
| MEM18001 | Use hand tools |
| MEM18002 | Use power tools/hand held operations |

# Competency Field

Casting and moulding

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Determine job requirements | 1.1 | Follow standard operating procedures (SOPs) |
|  |  | 1.2 | Comply with work health and safety (WHS) requirements at all times |
|  |  | 1.3 | Use appropriate personal protective equipment (PPE) in accordance with SOPs |
|  |  | 1.4 | Identify job requirements from specifications, drawings, job sheets or work instructions |
| 2 | Inspect and prepare patterns and cores | 2.1 | Inspect patterns or cores for appropriate surface finish and set up on jointline or flat board to specification |
|  |  | 2.2 | Select appropriate parting agent and apply to polymer specifications |
| 3 | Manufacture moulds, patterns and tooling aids | 3.1 | Select appropriate polymer materials to specification |
|  |  | 3.2 | Mix polymer and hardener to correct ratios and specifications using standard safety and operating procedures |
|  |  | 3.3 | Apply polymer to specification and use predetermined methods, ensuring that air is not entrapped in application, excessive heat is not generated and delamination does not occur in final use |
|  |  | 3.4 | Strip, inspect, clean and repair pattern/core box as required |
|  |  | 3.5 | Machine and finish faces of polymer pattern to specifications |
|  |  | 3.6 | Apply appropriate method of location to patterns and core boxes |
|  |  | 3.7 | Check polymer tooling for conformance to specifications as required |

# Foundation Skills

This section describes those required skills (reading, writing, oral communication and numeracy) that are essential to workplace performance in this unit of competency.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Surface finish includes one (1) or more of the following: | texture gel coat aluminium fill |
| Parting agent includes one (1) or more of the following: | wax PVA |
| Polymer materials include one (1) or more of the following: | epoxy resins urethane |
| Method of location includes one (1) or more of the following: | male and female joint pin and bush |

# Unit Mapping Information

Release 2. Supersedes and is equivalent to MEM04011 Produce polymer patterns (Release 1).

Release 1. Supersedes and is equivalent to MEM04011B Produce polymer patterns.

# Links

The MEM Metal and Engineering Training Package Companion Volume Implementation Guide is available from VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2