# MEM04012 Assemble plated patterns

Release: 2

MEM04012 Assemble plated patterns

# Modification History

Release 2. Quantum of hours of workplace practice removed. Supersedes and is equivalent to MEM04012 Assemble plated patterns (Release 1).

Release 1. Supersedes and is equivalent to MEM04012B Assemble plated patterns.

# Application

This unit of competency defines the skills and knowledge required to assemble all types of plated patterns using a range of materials. It covers the manufacture (when appropriate) and mounting of runner systems.

Where the manufacture of polymer-plated patterns is required unit MEM04011 Produce polymer patterns should also be selected.

Where the selection and use of cast aluminium plates is required unit MEM04014 Develop and manufacture production patterns should also be selected.

Where the selection and use of precision measurement is required unit MEM12003 Perform precision mechanical measurement should also be selected.

Where the selection and use of machining competencies is required applicable units should also be selected.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

Band:	A

Unit Weight:	8

# Pre-requisite Unit

| MEM04010 | Develop and manufacture wood patterns |
| --- | --- |
| MEM04018 | Perform general woodworking machine operations |
| MEM09002 | Interpret technical drawing |
| MEM11011 | Undertake manual handling |
| MEM12006 | Mark off/out (general engineering) |
| MEM12023 | Perform engineering measurements |
| MEM12024 | Perform computations |
| MEM12026 | Perform advanced trade calculations in a manufacturing, engineering or related environment |
| MEM13015 | Work safely and effectively in manufacturing and engineering |
| MEM14006 | Plan work activities |
| MEM16006 | Organise and communicate information |
| MEM18001 | Use hand tools |
| MEM18002 | Use power tools/hand held operations |

# Competency Field

Casting and moulding

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Determine job requirements | 1.1 | Follow standard operating procedures (SOPs) |
|  |  | 1.2 | Comply with work health and safety (WHS) requirements at all times |
|  |  | 1.3 | Use appropriate personal protective equipment (PPE) in accordance with SOPs |
|  |  | 1.4 | Identify job requirements from specifications, drawings, job sheets or work instructions |
| 2 | Inspect and layout patterns | 2.1 | Inspect pattern to ensure dimensions and surface finish conforms to specifications |
|  |  | 2.2 | Lay out pattern and runner system to specifications from drawings, sketches or verbal instructions |
|  |  | 2.3 | Align pattern in an appropriate manner |
| 3 | Mount pattern on plates | 3.1 | Attach cope and drag patterns/double sided match plate patterns to pattern plate/s according to specification |
|  |  | 3.2 | Inspect cope and drag patterns/double sided match plate patterns for security and alignment |
| 4 | Mount runner system | 4.1 | Adjust volume of runner system to conform to specifications |
|  |  | 4.2 | Attach runner components pattern plates to specification using appropriate fixing and joining techniques |
| 5 | Inspect plated pattern assembly | 5.1 | Inspect surface and mouldability of plated pattern assembly for compliance with specifications |

# Foundation Skills

This section describes those required skills (reading, writing, oral communication and numeracy) that are essential to workplace performance in this unit of competency.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Materials include one (1) or more of the following: | timber timber composite metal |
| Aligned includes one (1) or more of the following: | measurement dowels |

# Unit Mapping Information

Release 2. Supersedes and is equivalent to MEM04012 Assemble plated patterns (Release 1).

Release 1. Supersedes and is equivalent to MEM04012B Assemble plated patterns.

# Links

The MEM Metal and Engineering Training Package Companion Volume Implementation Guide is available from VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2

Assessment Requirements for MEM04012 Assemble plated patterns

# Modification History

Release 2. Quantum of hours of workplace practice removed. Supersedes and is equivalent to MEM04012 Assemble plated patterns (Release 1).

Release 1. Supersedes and is equivalent to MEM04012B Assemble plated patterns.

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy the requirements of the elements and performance criteria on at least two (2) occasions and include:

following work instructions, standard operating procedures (SOPs) and safe work practices

identifying and interpreting specifications, drawings, sketches and other applicable reference documents to assemble plated patterns

undertaking relevant engineering calculations, including the determination of contraction rates

inspecting patterns for conformance, laying out pattern and runner system and aligning pattern to specifications

securely attaching cope and drag patterns/double sided match plate patterns to pattern plates, inspecting for alignment according to specifications

attaching runner components pattern plates using appropriate fixing and joining techniques and adjusting volume of runner system to conform to specifications

inspecting plated pattern assembly including surface and mouldability for compliance with specifications.

# Knowledge Evidence

Evidence required to demonstrate the required knowledge for this unit must be relevant to and satisfy the requirements of the elements and performance criteria and include knowledge of:

safe work practices and procedures and use of personal protective equipment (PPE)

units of measurement, numerical operations and calculations associated with assembling plated patterns

fixing and drilling techniques

reason for using particular layout and runner system

all types of plated patterns using a range of materials

techniques for avoiding cross jointing or mismatch of the pattern and their relationship to the pin-centre

methoding systems

mouldability i.e. surface finish, face taper, convex or concave perspectives and undercuts

moulding and casting techniques.

# Assessment Conditions

Assessors must:

have vocational competency in assembling plated patterns at least to the level being assessed with relevant industry knowledge and experience

satisfy the assessor requirements in the Standards for Registered Training Organisations 2015 or its replacement and comply with the National Vocational Education and Training Regulator Act 2011, its replacement or equivalent legislation covering VET regulation in a non-referring state/territory as the case requires.

Assessment must occur in a functioning workplace. Where assessment in the workplace would be unsafe, impractical or threatens the environment, assessment must occur in a sufficiently rigorous simulated environment that reflects the circumstances that would be experienced in a functioning workplace. Assessment must cover all aspects of workplace performance, including task skills, task management skills, contingency management skills and job role environment skills.

Conditions for assessment must include access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications.

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

# Links

The MEM Metal and Engineering Training Package Companion Volume Implementation Guide is available from VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2