# MEM04016 Develop and manufacture precision models

Release: 2

MEM04016 Develop and manufacture precision models

# Modification History

Release 2. Quantum of hours of workplace practice removed. Supersedes and is equivalent to MEM04016 Develop and manufacture precision models (Release 1).

Release 1. Supersedes and is equivalent to MEM04016C Develop and manufacture precision models.

# Application

This unit of competency defines the skills and knowledge required to develop and manufacture precision models where accurate and complicated models are required to assist in the development and manufacture of production of tooling for a wide variety of processes.

This unit can also apply to the inspection/completion of items made in the stereo lithography process.

When the programming of coordinate measurement machines and/or NC/CNC machines is required appropriate machine programming units should also be selected.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

Band:	A

Unit Weight:	6

# Pre-requisite Unit

| MEM04010 | Develop and manufacture wood patterns |
| --- | --- |
| MEM04011 | Produce polymer patterns |
| MEM04012 | Assemble plated patterns |
| MEM04013 | Develop and manufacture polystyrene patterns |
| MEM04014 | Develop and manufacture production patterns |
| MEM04015 | Develop and manufacture vacuum forming moulds and associated equipment |
| MEM04018 | Perform general woodworking machine operations |
| MEM07005 | Perform general machining |
| MEM09002 | Interpret technical drawing |
| MEM11011 | Undertake manual handling |
| MEM12003 | Perform precision mechanical measurement |
| MEM12006 | Mark off/out (general engineering) |
| MEM12023 | Perform engineering measurements |
| MEM12024 | Perform computations |
| MEM12026 | Perform advanced trade calculations in a manufacturing, engineering or related environment |
| MEM13003 | Work safely with industrial chemicals and materials |
| MEM13015 | Work safely and effectively in manufacturing and engineering |
| MEM14006 | Plan work activities |
| MEM16006 | Organise and communicate information |
| MEM18001 | Use hand tools |
| MEM18002 | Use power tools/hand held operations |

# Competency Field

Casting and moulding

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Determine job requirements | 1.1 | Follow standard operating procedures (SOPs) |
|  |  | 1.2 | Comply with work health and safety (WHS) requirements at all times |
|  |  | 1.3 | Use appropriate personal protective equipment (PPE) in accordance with SOPs |
|  |  | 1.4 | Identify job requirements from specifications, drawings, job sheets or work instructions |
| 2 | Lay out model | 2.1 | Select appropriate material to meet specifications |
|  |  | 2.2 | Develop and plan finished model design with reference to customer’s specifications (written or verbal) for finish, quality and form, using applicable processes |
|  |  | 2.3 | Calculate contractions allowances, clearances and tapers to establish model parameters |
|  |  | 2.4 | Design and manufacture datum boards, jigs and fixtures as required |
| 3 | Manufacture model | 3.1 | Determine sequence of manufacture, including build-up on datum board, and establishing datum’s mark out of model and areas to be NC/CNC machined |
|  |  | 3.2 | Select and use appropriate machines and machining processes to shape/produce model to specifications |
|  |  | 3.3 | Select and use a range of hand and hand held power tools to fashion and manufacture model to fine tolerances according to specifications, ensuring that surface finish is appropriate to the type of model |
|  |  | 3.4 | Undertake appropriate measurement/calculations to check specifications, including coordinate measuring and machine checking as required |
|  |  | 3.5 | Record and report all deviations or modifications to original tooling design, prints or plans, consistent with procedures |

# Foundation Skills

This section describes those required skills (reading, writing, oral communication and numeracy) that are essential to workplace performance in this unit of competency.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Models include one (1) or more of the following: | flow viewing prototype development |  |
| Material includes one (1) or more of the following: | timber metal plastic fibreglass composites |  |
| Processes include one (1) or more of the following: | high volume foundry tooling blow moulding metal forming vacuum forming rotational moulding gravity die casting pressure die casting low pressure die casting complex ferrous and non-ferrous castings plastic injection moulding |  |
| Specifications include one (1) or more of the following: | complex drawings Mylar drawings technical sketches customer requirements |  |

# Unit Mapping Information

Release 2. Supersedes and is equivalent to MEM04016 Develop and manufacture precision models (Release 1).

Release 1. Supersedes and is equivalent to MEM04016C Develop and manufacture precision models.

# Links

The MEM Metal and Engineering Training Package Companion Volume Implementation Guide is available from VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2

Assessment Requirements for MEM04016 Develop and manufacture precision models

# Modification History

Release 2. Quantum of hours of workplace practice removed. Supersedes and is equivalent to MEM04016 Develop and manufacture precision models (Release 1).

Release 1. Supersedes and is equivalent to MEM04016C Develop and manufacture precision models.

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy the requirements of the elements and performance criteria on at least two (2) occasions and include:

following work instructions, standard operating procedures (SOPs) and safe work practices

planning finished model design by reading, interpreting customer’s specifications (verbal or written) for finish, quality and form, and selecting appropriate material

undertaking calculations for contraction allowances, clearances and tapers to establish model parameters

designing and manufacturing datum boards, jigs and fixtures as required

determining sequence of manufacture, including build-up on datum board, marking out and areas to be NC/CNC machined

selecting and using appropriate machines and machining process

selecting and using hand and power tools as needed to shape, produce, fashion and manufacture model to fine tolerances according to specification ensuring that surface finish is appropriate to the type of model

checking specifications by using appropriate measuring techniques, including coordinate measuring and machine checking

recording all deviations or modifications to original tool design, prints or plans according to SOPs.

# Knowledge Evidence

Evidence required to demonstrate the required knowledge for this unit must be relevant to and satisfy the requirements of the elements and performance criteria and include knowledge of:

safe work practices and procedures and use of personal protective equipment (PPE)

consequences of selecting inappropriate materials

various processes requiring models

calculus, engineering calculations and formulae relating to developing and manufacturing precision models

properties and uses of datum boards, datum holes or datum faces

reasons for developing the sequence of manufacture

range of machines and machining processes and their operations

various checking procedures and devices, such as coordinate measuring and machine checking

procedures for recording deviation or modification to original drawings or specifications.

# Assessment Conditions

Assessors must:

have vocational competency in the development and manufacture of precision models at least to the level being assessed with relevant industry knowledge and experience

satisfy the assessor requirements in the Standards for Registered Training Organisations 2015 or its replacement and comply with the National Vocational Education and Training Regulator Act 2011, its replacement or equivalent legislation covering VET regulation in a non-referring state/territory as the case requires.

Assessment must occur in a functioning workplace. Where assessment in the workplace would be unsafe, impractical or threatens the environment, assessment must occur in a sufficiently rigorous simulated environment that reflects the circumstances that would be experienced in a functioning workplace. Assessment must cover all aspects of workplace performance, including task skills, task management skills, contingency management skills and job role environment skills.

Conditions for assessment must include access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications.

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

# Links

The MEM Metal and Engineering Training Package Companion Volume Implementation Guide is available from VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2