# MEM04033 Develop and manufacture turned wood patterns

Release: 1

MEM04033 Develop and manufacture turned wood patterns

# Modification History

Release 1. Supersedes but is not equivalent to MEM04010 Develop and manufacture wood patterns.

# Application

This unit of competency defines the skills and knowledge required to develop and manufacture turned wood patterns and pattern component parts, including general engineering and master patterns.

It applies to patterns manufactured on wood turning equipment based on an understanding of casting and moulding principles. These patterns may be constructed using  Timber or timber laminate, and may be further broken don for other pattern types.

Where patterns are required to be set up on pattern plates, unit MEM04039 Assemble plated patterns should also be selected.

Where the development and manufacture of cast gear patterns is required, unit MEM04026 Develop and manufacture spur gear patterns should also be selected.

Where the development and manufacture of cast bevel gears is required, unit MEM04027 Develop and manufacture bevel gear patterns should also be selected.

Where the development and manufacture of cast chain gear and sprocket patterns is required, unit MEM04028 Develop and manufacture chain gear and sprocket patterns should also be selected.

Where the development and manufacture of cast conveyor screw patterns is required, unit MEM04029 Develop and manufacture conveyor screw patterns should also be selected.

Where the development and manufacture of cast marine propellers is required, unit MEM04030 Develop and manufacture propeller patterns should also be selected.

Where the selection and use of precision measurement is required, unit MEM12003 Perform precision mechanical measurement should also be selected.

This unit should not be selected when unit MEM18014 Manufacture press tools and gauges has already been selected.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

### Band: A

### Unit Weight: 3

# Pre-requisite Unit

MEM04018 Perform general woodworking machine operations

MEM09002 Interpret technical drawing

MEM11011 Undertake manual handling

MEM12006 Mark off/out (general engineering)

MEM12023 Perform engineering measurements

MEM12024 Perform computations

MEM12026 Perform advanced trade calculations in a manufacturing, engineering or related environment

MEM13015 Work safely and effectively in manufacturing and engineering

MEM14006 Plan work activities

MEM16006 Organise and communicate information

MEM18001 Use hand tools

MEM18002 Use power tools/hand held operations

# Competency Field

Casting and moulding

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Determine job requirements | 1.1 Follow standard operating procedures (SOPs) 1.2 Comply with work health and safety (WHS) requirements at all times 1.3 Use appropriate personal protective equipment (PPE) in accordance with SOPs 1.4 Identify job requirements from specifications |
| 2. Develop and lay out turned wood patterns | 2.1 Determine type of turned wood pattern required through application of moulding and/or casting techniques and foundry processes 2.2 Select timber or timber composites to meet specifications 2.3 Calculate pattern parameters based on specifications 2.4 Lay out pattern to specifications, showing tapers, machining allowances, core prints and method of construction 2.5 Develop and manufacture jigs or fixtures that aid turned wood pattern manufacture |
| 3. Manufacture turned wood patterns | 3.1 Mark out materials and develop construction to specifications 3.2 Produce pattern or pattern component parts to size and shape using wood patternmaking techniques, tools, equipment and procedures 3.3 Check pattern or pattern component parts for compliance with specifications 3.4 Join and fix pattern component parts to specifications 3.5 Colour code, mark or label pattern in accordance with SOPs |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Specifications include one or more of the following: | drawings job sheets work instructions. |
| Master patterns include one or more of the following: | multiple contraction skeleton frame and strickle wheels pulleys chain sheaves impellors. |
| Timber or timber composites include one or more of the following: | hardwood softwood laminates plywood veneers bonded fibreboard. |
| Joining and fixing includes one or more of the following: | glued screwed nailed stapled. |

# Unit Mapping Information

No equivalent unit.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2

Assessment Requirements for MEM04033 Develop and manufacture turned wood patterns

# Modification History

Release 1. Supersedes but is not equivalent to MEM04010 Develop and manufacture wood patterns.

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy the requirements of the elements and performance criteria and include:

following work instructions, standard operating procedures (SOPs) and safe work practices

identifying and interpreting specifications to develop turned wood patterns

determining type of turned wood pattern required through application of moulding and/or casting techniques and selecting the appropriate timber or timber composites to meet requirements

preparing for manufacture of patterns using the appropriate mathematical calculations for angles, tapers, clearances and determination of contraction rates

laying out pattern to specifications on at least two occasions, showing tapers, machining allowances, core prints and method of construction

designing and manufacturing required jigs or fixtures

marking out and manufacturing turned wood patterns and wood pattern component parts on at least two occasions, using appropriate hand and power tools

joining and fixing pattern components on at least two occasions and checking compliance with specifications

identifying pattern in accordance with SOPs.

Note: Where a volume and/or frequency is not specified, demonstration must be provided at least once.

# Knowledge Evidence

Evidence required to demonstrate the required knowledge for this unit must be relevant to and satisfy the requirements of the elements and performance criteria and include knowledge of:

safe work practices and procedures and use of personal protective equipment (PPE)

timber products, including features, characteristics and applications

moulding and casting techniques

tooling required for casting and moulding

methoding techniques

use and application of jigs and fixtures

methods of constructing turned wood patterns

formulae and mathematical techniques required for manufacturing of patterns or core boxes including those related to contraction, taper, clearances and machining allowances

identification coding and numbering

pattern checking techniques

mouldability including surface finish, face taper, convex or concave perspectives and undercuts.

# Assessment Conditions

Assessors must:

have vocational competency in the development and manufacture of turned wood patterns at least to the level being assessed with relevant industry knowledge and experience

satisfy the assessor requirements in the Standards for Registered Training Organisations 2015 or its replacement and comply with the National Vocational Education and Training Regulator Act 2011, its replacement or equivalent legislation covering VET regulation in a non-referring state/territory as the case requires.

Where possible, assessment must occur in operational workplace situations. Where this is not possible or where personal safety or environmental damage are limiting factors, assessment must occur in a sufficiently rigorous simulated environment that reflects realistic operational workplace conditions that cover all aspects of workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

Conditions for assessment must include access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications.

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2