# MEM07033 Operate and monitor basic boiler

Release: 1

MEM07033 Operate and monitor basic boiler

# Modification History

Release 1. Supersedes and is equivalent to MEM07033B Operate and monitor basic boiler

# Application

This unit of competency defines the skills and knowledge required to perform start-up, take-over/handover, monitor, shutdown and storage of a basic boiler used for all purposes, including the generation of steam.

All work is carried out to applicable state/territory and national work health and safety (WHS) legislation, standards and codes of practice.

Where the selection and use of engineering measurement is required unit MEM12023 Perform engineering measurements should also be selected.

Where the selection and use of tools is required unit MEM18001 Use hand tools and unit MEM18002 Use power tools/hand held operations, as appropriate should also be selected.

Some jurisdictions may require the holder of this unit to be licensed or certified and users should check with the relevant authorities.

Band:	A

Unit Weight:	6

# Pre-requisite Unit

| MEM11011 | Undertake manual handling |
| --- | --- |
| MEM13015 | Work safely and effectively in manufacturing and engineering |
| MEM16006 | Organise and communicate information |

# Competency Field

Machine and process operations

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Determine job requirements | 1.1 | Follow standard operating procedures (SOPs) |
|  |  | 1.2 | Comply with work health and safety (WHS) requirements at all times |
|  |  | 1.3 | Use appropriate personal protective equipment (PPE) in accordance with SOPs |
|  |  | 1.4 | Identify job requirements from specifications, job sheets or work instructions |
| 2 | Carry out pre-operational checks | 2.1 | Undertake pre-operational checks of boiler to plant operating procedures |
|  |  | 2.2 | Identify maintenance requirements and report in accordance with procedures |
| 3 | Maintain health and safety standards in work area | 3.1 | Identify hazards and potential hazards and report according to SOPs |
|  |  | 3.2 | Select hazard prevention/control measures, as required |
| 4 | Start boiler and conduct hand-over/take-over procedures | 4.1 | Start boiler and bring online safely, consistent with workplace procedures and production requirements |
|  |  | 4.2 | Confirm operating status of boiler |
|  |  | 4.3 | Maintain operating log and communicate boiler status/operation according to workplace procedures |
| 5 | Operate, monitor and shut down boiler | 5.1 | Operate boiler and monitor consistent with production and safety requirements |
|  |  | 5.2 | Conduct boiler water quality tests and adjust boiler water quality to manufacturers’ recommendations and workplace procedures, as required |
|  |  | 5.3 | Act upon boiler failures/emergencies according to workplace procedures and notify downstream users, if necessary |
|  |  | 5.4 | Shut down boiler consistent with workplace procedures, production and safety requirements |
| 6 | Carry out boiler shutdown for an internal inspection | 6.1 | Shut down boiler for internal inspection |
|  |  | 6.2 | Clean boiler internally and externally to manufacturers’ recommendations and workplace procedures |
| 7 | Store boiler in shutdown mode | 7.1 | Identify manufacturers’ recommendations and workplace procedures required for shutdown storage mode |
|  |  | 7.2 | Store boiler in identified shutdown mode |

# Foundation Skills

This section describes those required skills (reading, writing, oral communication and numeracy) that are essential to workplace performance in this unit of competency.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Pre-operational checks include one or more of the following: | feed water supply boiler water level fuel supply/heat source boiler valves - their operation and position combustion air supply combustion equipment |
| Boilers include the following: | single fixed combustion air supply, non-modulating single heat source and fixed firing rate |
| Hazards include one (1) or more of the following: | chemical thermal manual handling guarding of machinery illumination of work area rubbish and combustibles leakage of steam and fuel |
| Monitor includes one (1) or more of the following: | steam pressure flame and combustion conditions feed system and condensate returns fuel system water level combustion management system water management system boiler and steam manifold fittings soot blowers |
| Handover includes the following: | previous load requirements maintenance issue, including equipment isolated for maintenance operational incidences completing/reading operating log general inspection of boiler to detect any defects accept responsibility of boiler noted equipment malfunctions required equipment tests |
| Storage mode includes one (1) or more of the following: | wet dry open condition closed condition |

# Unit Mapping Information

Release 1. Supersedes and is equivalent to MEM07033B Operate and monitor basic boiler

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2

Assessment Requirements for MEM07033 Operate and monitor basic boiler

# Modification History

Release 1. Supersedes and is equivalent to MEM07033B Operate and monitor basic boiler

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy the requirements of the elements and performance criteria on at least two (2) occasions and include:

following work instructions, standard operating procedures (SOPs) and safe work practices

safely conducting pre-operational checks of the boiler and reporting any faults or hazards

starting and bringing the boiler online and confirming its operating status

monitoring the boiler operation and conducting water quality tests, as required

reacting effectively to any boiler emergency situations

shutting down the boiler in the required condition, cleaning and putting into storage mode.

# Knowledge Evidence

Evidence required to demonstrate the required knowledge for this unit must be relevant to and satisfy the requirements of the elements and performance criteria and include knowledge of:

safe work practices and procedures and use of personal protective equipment (PPE)

pre-operational checks, including:

feed water supply

boiler water level

fuel supply/heat source

boiler valves their operation and position

combustion air supply

combustion equipment

procedures for identifying and reporting maintenance

statutory requirements and workplace procedures for identifying and reporting hazards in the work area

processes for starting a boiler, including:

heat input

warm up of the reticulation system

steam traps and steam line purge

systems operation

reticulation line pressure

steam usage

supply

processes for confirming operational status of boiler

procedures for maintaining boiler operating log

procedures for communicating boiler status and operation

principles of boiler operation

boiler fittings

preparing boiler for inspection

procedures for monitoring a boiler, including:

steam reticulation line pressure

usage, supply and quality of steam

combustion/heat source system

feed water system

fuel system combustion air supply

water level

boiler steam pressures

operation of boiler control/safety devices

location of inspection and explosion doors

procedures for conducting boiler water quality tests

feed water systems and treatment

emergency procedures, including:

identification of emergency

isolation of heat source

selection and application of appropriate fire-fighting equipment

notification of downstream users

processes and procedures, including:

confirming water level

cooling down

boiler pressure/vacuum

fuel/heat source isolation

removal of combustion equipment and water from boiler

isolation from any common connection

the opening of all access points required for inspection

procedures for cleaning the boiler internally and externally

various modes of boiler storage

procedures for storing the boiler in open or closed condition.

# Assessment Conditions

Assessors must:

have vocational competency in operating and monitoring a basic boiler at least to the level being assessed with relevant industry knowledge and experience

satisfy the assessor requirements in the Standards for Registered Training Organisations 2015 or its replacement and comply with the National Vocational Education and Training Regulator Act 2011, its replacement or equivalent legislation covering VET regulation in a non-referring state/territory as the case requires

Where possible assessment must occur in operational workplace situations. Where this is not possible or where personal safety or environmental damage are limiting factors, assessment must occur in a sufficiently rigorous simulated environment that reflects realistic operational workplace conditions. This must cover all aspects of workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills

Conditions for assessment must include access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2