# MEM13013 Work safely with ionizing radiation

Release: 1

MEM13013 Work safely with ionizing radiation

# Modification History

Release 1. Supersedes and is equivalent to MEM13013B Work safely with ionizing radiation

# Application

This unit of competency defines the skills and knowledge required to work safely with ionizing radiation when performing radiographic testing in a range of open or closed industrial applications on fabrications, structures and components across a wide range of industries.

The work can relate to scheduled and unscheduled maintenance activities, using general tools, specific radiographic testing equipment as specified in maintenance documentation, testing procedures or operator instructions.

All testing must be completed with particular attention to personal and work health and safety (WHS) regulations. Certification against Australian Standards can be achieved where assessment in this unit of competency is carried out in conjunction with an examining authority as described in ISO 9712 Non-destructive testing – Qualification and certification of non-destructive testing (NDT) personnel.

Materials and chemicals which are subject to codes and regulations – for example, chemicals, explosives, solvents, dangerous materials, acids, or noxious waste products – must be subject to safe work habits and must be stored and used in accordance with safe work practices.

This unit is a prerequisite to undertaking any other radiographic units of competency.

Where interpretation of technical drawings is required unit MEM09002 Interpret technical drawing should also be selected.

Where the selection and use of engineering measurement is required unit MEM12023 Perform engineering measurements should also be selected.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

Band:	A

Unit Weight:	4

# Pre-requisite Unit

| MEM11011 | Undertake manual handling |
| --- | --- |
| MEM13015 | Work safely and effectively in manufacturing and engineering |
| MEM16006 | Organise and communicate information |

# Competency Field

Work health and safety

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Determine job requirements | 1.1 | Follow standard operating procedures (SOPs) |
|  |  | 1.2 | Comply with work health and safety (WHS) requirements at all times |
|  |  | 1.3 | Use appropriate personal protective equipment (PPE) in accordance with SOPs |
|  |  | 1.4 | Identify job requirements from specifications, job sheets or work instructions |
| 2 | Identify the hazards and effects of ionizing radiation in the workplace | 2.1 | Identify the source of ionizing radiation in accordance with relevant organisational policy and procedures |
|  |  | 2.2 | Outline attenuation factors of ionizing radiation and the biological effects on living tissue |
|  |  | 2.3 | Identify the biological effects of radiation |
| 3 | Apply radiation safety procedures/plans | 3.1 | Employ appropriate ionizing radiation protective measures in accordance with relevant organisational policy and procedures |
|  |  | 3.2 | State and adhere to exposure limits for personnel as laid down by the radiation authorities in Australia |
|  |  | 3.3 | Determine minimum exposure rates/distances from calculations and charts |
|  |  | 3.4 | Operate ionizing radiation sources in accordance with legislation, standards and/or organisational policy, procedures or guidelines |
| 4 | Select and use radiation monitoring equipment | 4.1 | Select the tools and equipment necessary to monitor radiation and use, as required |
|  |  | 4.2 | Select and apply techniques and system verification checks necessary to monitor radiation |
|  |  | 4.3 | Document safety breaches and/or report in accordance with organisational policy and procedures |
| 5 | Respond to emergency situations | 5.1 | Demonstrate procedures for dealing with both X-ray and gamma ray emergency situations |

# Foundation Skills

This section describes those required skills (reading, writing, oral communication and numeracy) that are essential to workplace performance in this unit of competency.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Radiation safe work practices and controls include the following: | ensuring security of radiation sources during storage, transport and use using signs, barriers, shielding to minimise radiation exposure of public reducing personal exposure time maintaining greatest feasible distance between radiation source and equipment operator using maximum feasible shielding between radiation source and equipment operator using a dosimeter to monitor personal radiation exposure maintaining statutory records of the use of radiation sources and/or instruments that emit ionising radiation |
| Standards and codes include the following: | the latest version of all relevant Australian and international standards and codes applicable to radiographic testing AS 2177 Non-destructive testing—Radiography of welded butt joints in metal AS 3507.1 Non-destructive testing - Guide to radiography for ferrous castings ISO 9712 Non-destructive testing—Qualification and certification of NDT personnel codes of practice from Australian and overseas engineering and technical associations and societies |
| WHS, regulatory requirements and workplace procedures include: | WHS Acts and regulations industry codes of practice risk assessments safe work practices workplace documents, such as: SOPs quality procedures equipment manuals calibration and maintenance schedules safety data sheets (SDS) safety procedures work schedules workplace recording and reporting procedures waste minimisation containment, processing and safe disposal procedures codes, regulations and safe work practices covering the use, handling, storage and transport of: radiation sources instruments that emit ionising radiation dangerous materials such as chemicals, solvents, acids, or noxious waste products |

# Unit Mapping Information

Release 1. Supersedes and is equivalent to MEM13013B Work safely with ionizing radiation

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2

Assessment Requirements for MEM13013 Work safely with ionizing radiation

# Modification History

Release 1. Supersedes and is equivalent to MEM13013B Work safely with ionizing radiation

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy the requirements of the elements and performance criteria on at least two (2) occasions and include:

following work instructions, standard operating procedures (SOPs) and safe work practices

identifying and interpreting charts, specifications, relevant organisational policy and procedures and other applicable reference documents in working safely with ionizing radiation

employing appropriate ionizing radiation protective measures and personal protective equipment (PPE) relevant to the industrial application

determining minimum exposure rates/distances from calculations and charts

undertaking numerical operations and calculations associated with determining minimum exposure rates/distances

selecting and using appropriate tools and equipment necessary to monitor radiation

documenting all safety breaches and recording/reporting in accordance with SOPs

demonstrating the ability to assess risks and handle emergencies.

# Knowledge Evidence

Evidence required to demonstrate the required knowledge for this unit must be relevant to and satisfy the requirements of the elements and performance criteria and include knowledge of:

safe work practices and procedures and use of PPE

International System of Units (SI) of radiation in accordance with the National Health and Medical Research Council (NHMRC)/statutory requirements

production of X-rays and gamma rays in relation to radiographic testing activities

principal radioactive sources used in industrial radiography

attenuation factors

known biological effects of radiation

general principles of gas ionisation, photographic effect and luminescence

use of film, film badges, ionisation chamber devices, quartz fibre, fluorescent, and electronic devices accuracy limits (energy/range)

different SI units of radiation, including Becquerel, Sievert and Gray

exposure limits for personnel as laid down by the radiation authorities in Australia

exposure reduction factors, including:

time

distance

shielding

procedures for establishing safe working barriers

relevant techniques and checks

emergency procedures

safety procedures for:

types of X-ray equipment

types of isotope cameras

shielding materials

design and requirements for exposure areas

requirements for storage of radioisotopes

emergency situations, causes and appropriate responses

storage requirements of equipment and materials

legal requirements including:

Australian, state/territory regulations, code of practice (detail)

International Commission on Radiological Protection (ICRP) recommended limits for various persons and various parts of the body for short-term, long-term and accumulated exposure

background radiation

duties of Radiation Safety Officer (RSO)

requirements for transport

International Air Transport Association (IATA) regulations

obligations of the licensee.

# Assessment Conditions

Assessors must:

have vocational competency in working safely with ionizing radiation at least to the level being assessed with relevant industry knowledge and experience

satisfy the assessor requirements in the Standards for Registered Training Organisations 2015 or its replacement and comply with the National Vocational Education and Training Regulator Act 2011, its replacement or equivalent legislation covering VET regulation in a non-referring state/territory as the case requires

Where possible assessment must occur in operational workplace situations. Where this is not possible or where personal safety or environmental damage are limiting factors, assessment must occur in a sufficiently rigorous simulated environment that reflects realistic operational workplace conditions. This must cover all aspects of workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills

Conditions for assessment must include access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2